3GPP TSG-RAN WG4 Meeting # 99-e                                                        R4-2108306

Electronic Meeting, May 19-27

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | 1 | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [draftCR] CR for PRS configurations for NR Pos RRM tests |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Pos-Perf |  |  |  |  |  |  | Date: |  | 2021-5-12 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Correct some PRS configuation pattern parameters which are algined with the latest performance accuracy requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated PRS configurations on the repetition factore and PRS BW |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PRS parameters in RRM tests may not cover some typical using cases. The RRM requirements cannot be verified in realistic environment. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.2.x, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 38.533 |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

--- start of change 1 ---

## A.3.x	PRS Configurations

#### A.3.x.1.	PRS Configurations for FR1

#### A.3.x.1.1.	PRS pattern 1 in FR1: SCS=15 KHz in 10 MHz

Table A.3.x.1.1 -1: PRS.1 FR1: PRS Pattern 1 for SSB SCS=15 KHz in 10 MHz channel

| PRS Parameters | Values |  |
| --- | --- | --- |
| Reference channel | PRS.1.1 FR1 | PRS.1.2 FR1 |
| Channel bandwidth | 10 MHz |  |
| PRS periodicity | 160 ms |  |
| PRS Resourceset slot offset | 10 ms Note 1 |  |
| PRS Resource slot offset (slot) | 0 Note 1 | 4 Note 1 |
| PRS RE offset | 0 |  |
| SCS | 15 KHz |  |
| PRS comb size | [2 ] | [4 ] |
| Number of PRS symbol | [4 ] | [4 ] |
| Repetion factor | [2 ] | [1 ] |
| PRS resource time gap (slot) | [1] | [1] |
| RB numbers containing PRS within channel BW | 0-23 | 0~103 |
| PRS Start PRB | 0 |  |
| Note 1: Unless otherwise specified in the test case |  |  |

#### A.3.x.1.2.	PRS pattern 2 in FR1: SCS=30 KHz in 40 MHz

Table A.3.x.1.2 -1: PRS.2 FR1: PRS Pattern 2 for SCS=30 KHz in 50 MHz channel

| PRS Parameters | Values |  |  |
| --- | --- | --- | --- |
| Reference channel | PRS.2.1 FR1 |  | PRS.2.2 FR1 |
| Channel bandwidth | 50 MHz |  |  |
| PRS periodicity | 160 ms |  |  |
| PRS Resourceset slot offset | 10 ms |  |  |
| PRS Resource slot offset (slot) | 0 |  | 4 |
| PRS RE offset | 0 |  |  |
| SCS | 30 KHz |  |  |
| PRS comb size | [2 ] | [4 ] |  |
| Number of PRS symbol | [4] | [4 ] |  |
| Repetion factor | [2] | [1 ] |  |
| PRS resource time gap (slot) | [1] | [1] |  |
| RB numbers containing PRS within channel BW | 0-23 | 0~131 |  |
| PRS Start PRB | 0 |  |  |
| Note 1: Unless otherwise specified in the test case |  |  |  |

#### A.3.x.2.	PRS Configurations for FR2

#### A.3.x.2.1.	PRS pattern 1 in FR2: SCS=120 KHz in 100 MHz

Table A.3.x.2.1 -1: PRS.1 FR2: PRS Pattern 1 for SCS=120 KHz in 100 MHz channel

| PRS Parameters | Values |  |
| --- | --- | --- |
| Reference channel | PRS.1.1 FR2 | PRS.1.2 FR2 |
| Channel bandwidth | 100 MHz |  |
| PRS periodicity | 160 ms |  |
| PRS Resourceset slot offset | 10 ms Note 1 |  |
| PRS Resource slot offset (slot) | 0 Note 1 | 4 Note 1 |
| PRS RE offset | 0 |  |
| SCS | 120 KHz |  |
| PRS comb size | [2 ] | [4 ] |
| Number of PRS symbol | [4 ] | [4 ] |
| Repetion factor | [1 ] | [1 ] |
| PRS resource time gap (slot) | [1] | [1] |
| RB numbers containing PRS within channel BW | 0-23 | 0-127 |
| PRS Start PRB | 0 |  |
| Note 1: Unless otherwise specified in the test case |  |  |

--- end of change 1 ---