3GPP TSG-RAN WG4 Meeting # 99-e 												R4-2108389

Electronic Meeting, May 19th – 27th, 2021

Agenda item:			6.6.1, 6.6.2.1, 6.6.2.2

Source:	Moderator (CATT)

Title:	Email discussion summary for [99-e][217] NR_CSIRS_L3meas_1

Document for:	Information

Introduction

The documents in agenda items 6.6.1, 6.6.2.1 and 6.6.2.2 contain the following 2 main topics:

- Topic #1: CSI-RS RRM core requirements maintenance

- Topic #2: CSI-RS RRM performance requirements.

Topic #1: CSI-RS RRM core requirements maintenance

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2109077 | CATT | Proposal 1: For the time domain restriction of CSI-RS resources configuration(5ms time window), keep the current specification unchanged. Proposal 2: For UE behavior when the timing offset exceeds the threshold, no spec updates are needed. Proposal 3: The associated SSB is detected if it has been meeting the relevant cell identification requirement during the last 5 seconds. |
| R4-2109078 | CATT | CR |
| R4-2109079 | CATT | CR |
| R4-2109080 | CATT | Cat A CR |
| R4-2109081 | CATT | Cat A CR |
| R4-2109177 | MediaTek inc. | CR |
| R4-2109178 | MediaTek inc. | Cat A CR |
| R4-2109239 | Intel Corporation | Proposal 1: Prefer option 1. For option 3, NW needs to make sure that no measurement gaps collide with all the CSI-RS resources. Otherwise, no requirement will apply. Proposal 2: When the timing offset exceeds the threshold in R16 with single FFT assumption, support option 1. |
| R4-2109376 | Apple | Proposal 1: - All L3 CSI-RS resources in the same MO have identical periodicity. - For inter-frequency measurements, Rel-16 L3 CSI-RS requirements are defined under assumption that all CSI-RS resources in the same MO are configured in the same 5ms window - For intra-frequency measurements: Rel-16 L3 CSI-RS requirements are defined under assumption that CSI-RS resources in the same MO can be configured in up to two separated 5ms windows during one CSI-RS resource period. - When intra-frequency L3 CSI-RS are configured in two separated windows, the minimum periodicity of the configured CSI-RS resources is 20ms. - Measurement requirements are not impacted by separated 5ms windows. Proposal 2: - For intra-frequency measurement, the UE is not required to measure the configured CSI-RS resources of a neighbour cell if the symbol level misalignment between serving and the corresponding neighbour cell exceeds the threshold. - For inter-frequency measurement, UE can pick up any cell as the reference cell per frequency layer. UE is not required to measure the configured CSI-RS resources of a neighbour cell if the symbol level misalignment between the reference cell and the corresponding neighbour cell belonging to the same frequency layer exceeds the threshold. Proposal 3: Regarding the time validity of the detected associatedSSB, the timing information of CSI-RS resources for L3 measurement can be assumed as known if the associatedSSB has been detectable during the last 5 seconds. |
| R4-2109552 | Nokia, Nokia Shanghai Bell | Proposal1: The CSI-RS resources in the same MO may fall in different 5ms windows up to network configuration. Proposal2: In Rel16, the UE is not required to measure or report the CSI-RS resource if the timing difference exceeds a threshold, hence Option 1 is preferred. |
| R4-2109553 | Nokia, Nokia Shanghai Bell | CR |
| R4-2109554 | Nokia, Nokia Shanghai Bell | Cat A CR |
| R4-2109927 | vivo | CR |
| R4-2109928 | vivo | Cat A CR |
| R4-2109933 | vivo | - Proposal 1: If there is no agreement on time domain restriction for CSI-RS configuration, then keep the current specification unchanged. - Proposal 2: No UE behaviour is specified when timing offset exceeds the threshold. - Proposal 3: The associatedSSB is detected if it has been meeting the relevant cell identification requirement during the last 5 seconds |
| R4-2110056 | OPPO | Proposal 1: Prefer the clarification in the specification that requirements are defined for the cases where all CSI-RS resources in the same MO are configured in the same 5ms window. Proposal 2: Can compromise to option 2 by keeping the current specification unchanged. Proposal 3: No spec updates are needed for UE behavior when the timing offset exceeds the threshold. Proposal 4: Prefer option 1 that the associatedSSB is detected if it has been meeting the relevant cell identification requirement during the last 5 seconds. |
| R4-2110364 | Huawei, HiSilicon | Proposal 1: Adding the time validity of detected associatedSSB and SFN information in 9.10.2.5 section: If the associatedSSB which has been detectable at least for the time period Tidentify_intra_with_index defined in clause 9.2.5.1 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again with the same spatial reception parameter provided the timing to that cell has not changed more than ± 3200 Tc, PSS/SSS detection time of associatedSSB and time period used to acquire the SFN information are equal to 0. Observation 1: It may be difficult for NW to accommodate all CSI-RS resources in the same 5ms window. Observation 2: UE supporting Case 1 should also be able to support Case 2, since UE can determine the presence of each CSI-RS resource in each window occasion. Proposal 2: Adopt the following applicability conditions for CSI-RS measurement requirements: - For inter-frequency measurements, Rel-16 L3 CSI-RS requirements are defined under assumption that all CSI-RS resources in the same MO are configured in the same 5ms window - For intra-frequency measurements: Rel-16 L3 CSI-RS requirements are defined under assumption that CSI-RS resources in the same MO can be configured in up to two separated 5ms windows during one CSI-RS resource period, and the overlapping status with MG is same for the two windows. |
| R4-2110365 | Huawei, HiSilicon | CR |
| R4-2110366 | Huawei, HiSilicon | Cat A CR |
| R4-2110386 | Huawei, HiSilicon | CR |
| R4-2110387 | Huawei, HiSilicon | Cat A CR |
| R4-2110903 | Huawei, HiSilicon | CR |
| R4-2110904 | Huawei, HiSilicon | Cat A CR |
| R4-2111412 | Apple | CR |
| R4-2111413 | Apple | Cat A CR |

Open issues summary

Sub-topic 1-1 Time domain restriction for CSI-RS configuration

Proposals:

- Option 1: (Intel, OPPO)

  - Rel-16 L3 CSI-RS requirements are defined under assumption that all CSI-RS resources in the same MO are configured in the same 5ms window

    - Note: It is up to the network whether to configure all CSI-RS in the 5ms window and if CSI-RS resources are configured outside then UE may not measure it and the requirements do not apply.

- Option 2: (CATT, vivo, OPPO)

  - Keep the current specification unchanged if no agreements reached.

- Option 3: (Apple)

  - For inter-frequency measurements, Rel-16 L3 CSI-RS requirements are defined under assumption that all CSI-RS resources in the same MO are configured in the same 5ms window

  - For intra-frequency measurements: Rel-16 L3 CSI-RS requirements are defined under assumption that CSI-RS resources in the same MO can be configured in up to two separated 5ms windows during one CSI-RS resource period.

    - When intra-frequency L3 CSI-RS are configured in two separated windows, the minimum periodicity of the configured CSI-RS resources is 20ms.

    - Measurement requirements are not impacted by separated 5ms windows.

- Option 4: (Nokia)

  - The CSI-RS resources in the same MO may fall in different 5ms windows up to network configuration.

- Option 5: (Huawei)

  - For inter-frequency measurements, Rel-16 L3 CSI-RS requirements are defined under assumption that all CSI-RS resources in the same MO are configured in the same 5ms window

  - For intra-frequency measurements: Rel-16 L3 CSI-RS requirements are defined under assumption that CSI-RS resources in the same MO can be configured in up to two separated 5ms windows during one CSI-RS resource period, and the overlapping status with MG is same for the two windows.

- Recommended WF

  - Need more discussion

| Sub-topic 1-1 Time domain restriction for CSI-RS configuration |  |
| --- | --- |
| Company | Comments |
| Huawei | We support option 3, 4 and 5. Our first preference is option 4, but as a compromise we can also support option 3 and option 5. In our understanding, option 3 and option 5 are rather similar, and we are also fine to combine all the conditions from these two options for intra-freq case. |
| CATT | Support option 2 and option 5. We think the option 1 and option 2 are aligned. The current specification means the CSI-RS resources in the same MO are configured in the same 5ms window in our understanding. And the option 3 and option 5 are also aligned. We think they can be merged into one. |
| OPPO | Prefer option 1 and 2. From network configuration perspective, it can configure CSI-RS resources in the same MO in different 5ms windows. From requirement definition perspective, it performs well if all CSI-RS resources in the same MO are configured in the same 5ms window, which does not conflict with the configuration of network. |
| Nokia | We support Option 4 but can compromise to Option 5 and Option 3 to proceed. In any case, we are expecting certain flexibility on network configuration, and we agree the measurement requirements should not be impacted by the multiple 5ms windows. |
| Xiaomi | Prefer option 1 or option 2 |
| Qualcomm | Prefer Option2 but could consider Option3. |
| MTK | Support Option 1 or Option 2. The current measurement delay requirement allows 5 samples: No DRX; max(200ms, ceil( [5] x Kp_CSI-RS) x CSI-RS period) x CSSFintra. If UE is configured with an A3 event which needs to compare the RSRP from serving cell and neighboring cell. But the CSI-RS of serving cell and neighboring cell are now allocated in 2 different window, then we expect UE will need one more sample to trigger the event. This means we also need to revise the already agreed delay requirement. |
| Intel | Prefer option 1. Considering that the CSI-RS resources may not be enough in one window, we can compromise to option 5. NW need to ensure that MG overlapping status with the two windows are the same, otherwise, no requirement will apply. |
| Apple | We propose option 3 as a compromised solution. Option 1 and 2 are fine too. |
| vivo | In our understanding, option 1 and option 2 are almost the same. We are not expecting there would be standardization work if option 1 is agreed. We are also fine with compromised solution option3/5. If there is no agreement, then option 2 is supported. |

Sub-topic 1-2 UE behavior when the timing offset exceeds the threshold with single FFT assumption

- Proposals

  - Option 1: (Intel, Apple, Nokia)

    - For intra-frequency measurement, the UE is not required to measure the configured CSI-RS resources of a neighbour cell if the symbol level misalignment between serving and the corresponding neighbour cell exceeds the threshold.

    - For inter-frequency measurement, UE can pick up any cell as the reference cell per frequency layer. UE is not required to measure the configured CSI-RS resources of a neighbour cell if the symbol level misalignment between the reference cell and the corresponding neighbour cell belonging to the same frequency layer exceeds the threshold.

  - Option 2: (CATT, vivo, OPPO)

    - No spec updates are needed.

- Recommended WF

  - Need more discussion

| Sub-topic 1-2 UE behavior when the timing offset exceeds the threshold with single FFT assumption |  |
| --- | --- |
| Company | Comments |
| Huawei | We slightly prefer option 2 since we understand the current requirements do not require UE to measure resources with timing offset larger than the threshold. |
| CATT | Option 2 is preferred. For option 1, if defined, we think a unified description should be used for intra and inter-frequency measurement i.e. UE is not required to measure and report the configured CSI-RS when the timing offset between reference timing and target CSI-RS is larger than the threshold. There is no need to further clarify the reference timing since this is a separated issue which was agreed in previous meeting. |
| OPPO | Prefer Option 2. The solution in option 1 is more related to UE implementation. The current requirements can be maintained without additional clarification. |
| Nokia | Support Option 1. As the timing offset is unknown by network, there is no way to determine if the received measurement report is fulfilling the performance requirements hence may impact the mobility decision. The restriction of UE behavior is intended to avoid misleading measurement reporting and can also save UE measurement efforts. To Huawei: In current spec, it only tells the requirements apply if the timing offset exceeds the threshold but there is no any description on the UE behavior. This is exactly out intention to not measure and report the results to the network. |
| Xiaomi | Option 2, there is no related requirements if the timing offset exceeds the threshold, whether UE perform the measurements up to UE implementation. |
| Qualcomm | Option2 is supported. Option1 has implications on required UE implementation although it’s phrased to be “not required”. There may be alternative ways to enhance before concluding enhancing UE is the only option. |
| MTK | Option 2 is slightly preferred. The timing offset is only a side condition which should be considered jointly with the Es/Iot upper bound. In other words, if the Es/Iot is not that high, UE may still make accurate RSRP/SINR estimation even if the timing offset is slightly larger than the threshold. |
| Intel | Prefer option 1 since the measurement accuracy may not be guaranteed. We can compromise to option 2 since it’s related to UE implementation. |
| Apple | Prefer to option 1. It is better to make the spec as clear as possible. It should be explicit to define UE’s behavior. |
| vivo | Support option 2. Our analysis is provided in the contribution. We think it would be better to leave this to UE implementation. |

Sub-topic 1-3 Time validity of the detected associatedSSB

- Proposals

  - Option 1: (CATT, vivo, OPPO)

    - The associatedSSB is detected if it has been meeting the relevant cell identification requirement during the last 5 seconds.

  - Option 2: (Apple)

    - The timing information of CSI-RS resources for L3 measurement can be assumed as known if the associatedSSB has been detectable during the last 5 seconds.

  - Option 3: (Huawei)

    - Adding the time validity of detected associatedSSB and SFN information in 9.10.2.5 section:

      - If the associatedSSB which has been detectable at least for the time period Tidentify_intra_with_index defined in clause 9.2.5.1 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again with the same spatial reception parameter provided the timing to that cell has not changed more than ± 3200 Tc, PSS/SSS detection time of associatedSSB and time period used to acquire the SFN information are equal to 0.

- Recommended WF

  - Need more discussion

| Sub-topic 1-3 Time validity of the detected associatedSSB |  |
| --- | --- |
| Company | Comments |
| Huawei | Option 1, 2 and 3 are fine. We proposed option1 at last meeting, however certain company has concern. To my knowledge, the main concern from opponent is “associated SSB of the cell being identified or measured is not detectable, UE is not required to satisfy the measurement requirements”. That is also why we change to another description like option3. Anyway we hope the opponent could elaborate the objection reasons. With fully understanding, we can find proper way out. |
| CATT | Support option 1. We think option 1 and option 2 is aligned except that the associated SSB should be detected rather than ‘detectable’ in option 2. And we think it is enough to clarify that the detection time of associated SSB has passed no longer than 5s. More detailed case listed in option 3 is not needed. |
| OPPO | Option 1 is fine. Agree that ‘detected’ is clear. “Detectable” seems another item to be further clarified. |
| Nokia | We prefer Option 3 in principle, and with some small edition as below: If the associatedSSB which has been detectable at least for the time period Tidentify_intra_with_index defined in clause 9.2.5.1 becomes undetectable for a period ≤ 5 seconds and then the associatedSSB becomes detectable again with the same spatial reception parameter provided the timing to that cell has not changed more than ± 3200 Tc, PSS/SSS detection time and time period used to acquire the SFN information are equal to 0. |
| Qualcomm | Option1 and option2 are supported and slightly prefer Option2. For Option2, it seems a stricter condition and consistent with other places in the RAN4 spec. |
| MTK | Support Option 3. Option 3 is more comprehensive and addresses other aspects like timing offset and Rx beam consistency. This aligned with the conclusion in Rel-15 for SSB. |
| Apple | Option 2. We think option 2 can be considered as a compromised one between option 1 and 3. |
| vivo | Support option 1 and option 2. Option 2 can be used as baseline for agreement since it is straightforward. But it seems option 1 is better to be captured in the spec as it is the existing wording in the spec. Option 3 is not complete set. The associatedSSB doen’t have to become undetectable and then become detectable again for a period less than 5 seconds. It can always be detectable during the 5 seconds. It seems such detailed clarification is not necessary. |

Companies views’ collection for 1st round

Open issues

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| Resubmitted CR of RAN4#98bis-e |  |
| R4-2109078 (CATT) | Qualcomm: adding CSI-RS “ - The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on the configured CSI-RS symbol within the configured slot as indicated in slotConfig of the corresponding CSI-RS resource to be measured for mobility. ” |
|  | Company B |
| R4-2109927 (vivo) | Qualcomm: May need to hold till conclusions made in Sub-topic 1-1. If two windows were adopted for intra-frequency, what is the rule to locate the second window? |
|  | vivo: This is formal CR for the endorsed CR R4-2105773 in the last meeting. We can also capture further agreements on two windows in this meeting, if there would be agreements. |
| R4-2110386 (Huawei) | Qualcomm: Are the inter-frequency SSB-based measurements supported outside the gap via reporting no gap in NeedForGaps? Thanks |
| New CR for RAN4#99e |  |
| R4-2109079 (CATT) | Nokia: The updated section number seems to be wrong. Otherwise it looks good. |
| R4-2109177 (MTK) | Nokia: Could you explain why to limit the SSB configuration to PSCell? We understood SSB frequency layers can be configured by both LTE and NR? |
|  | MTK: We are focusing on the additional SSB frequency layer configured through associated CSI-RS. CSI-RS MO can only be configured by NR, not LTE. By saying PSCell, it is definitely an NR cell in TS 38.133. |
| R4-2109553 (Nokia) | CATT: CR number is missing and WI code is ‘NR_CSIRS_L3meas-Core’. The updats on UE behavior depend on the conclusion of sub-topic 1-2. |
| R4-2110365 (Huawei) | CATT: ‘or inter-frequency TSSB_time_index_inter in clause 9.3.4’ should not be deleted since the intra/inter definition for associated SSB and CSI-RS is separated and can be different. The updates on time validity depend on the conclusion of sub-topic 1-3. |
|  | Nokia: Ok with some small changes on wording (as in the comments to subtopic 1-3) |
| R4-2110903 (Huawei) | CATT: depends on the conclusion of sub-topic 1-1. |
|  | Nokia: This depends on the conclusion of Subtopic 1-1. There are some mistakes in the wording in the CR. We can adopt the formulation in listed options of sub-topic 1-1 once concluded. |
| R4-2111412 (Apple) | CATT: change 2 was endorsed in last meeting and is resubmitted in R4-2109927. |
|  | Nokia: We understood the wording in current spec already covers the cases in the proposed CR. Probably there is no need to list every scenario. |
|  | Vivo: Change 2 on starting point is captured in vivo’s CR R4-2109927, which is formal CR for the endorsed CR in the last meeting. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1-1 | Tentative agreements: None Candidate options: - Option 1: (CATT, OPPO, Xiaomi, Qualcomm, MTK, Intel, Apple, vivo) - Rel-16 L3 CSI-RS requirements are defined under assumption that all CSI-RS resources in the same MO are configured in the same 5ms window - Note: It is up to the network whether to configure all CSI-RS in the 5ms window and if CSI-RS resources are configured outside then UE may not measure it and the requirements do not apply. - Option 2: (Huawei, CATT, Nokia, Qualcomm, Intel, Apple, vivo) - For inter-frequency measurements, Rel-16 L3 CSI-RS requirements are defined under assumption that all CSI-RS resources in the same MO are configured in the same 5ms window - For intra-frequency measurements: Rel-16 L3 CSI-RS requirements are defined under assumption that CSI-RS resources in the same MO can be configured in up to two separated 5ms windows during one CSI-RS resource period - The overlapping status with MG is same for the two windows. - The minimum periodicity of the configured CSI-RS resources is 20ms. - Measurement requirements are not impacted by separated 5ms windows. Recommendations for 2nd round: Continue discussion in 2nd round. |
| Sub-topic #1-2 | Tentative agreements: None Candidate options: - Option 1: (Nokia, Intel, Apple) - For intra-frequency measurement, the UE is not required to measure the configured CSI-RS resources of a neighbour cell if the symbol level misalignment between serving and the corresponding neighbour cell exceeds the threshold. - For inter-frequency measurement, UE can pick up any cell as the reference cell per frequency layer. UE is not required to measure the configured CSI-RS resources of a neighbour cell if the symbol level misalignment between the reference cell and the corresponding neighbour cell belonging to the same frequency layer exceeds the threshold. - Option 2: (Huawei, CATT, OPPO, Xiaomi, Qualcomm, MTK, Intel, vivo) - No spec updates are needed. Recommendations for 2nd round: Continue discussion in 2nd round. |
| Sub-topic #1-3 | Tentative agreements: None Candidate options: - Option 1: (Huawei, CATT, OPPO, Qualcomm, vivo) - The associatedSSB is detected if it has been meeting the relevant cell identification requirement during the last 5 seconds. - Option 2: (Huawei, Qualcomm, Apple, vivo) - The timing information of CSI-RS resources for L3 measurement can be assumed as known if the associatedSSB has been detectable during the last 5 seconds. - Option 3: (Huawei, Nokia, MTK) - Adding the time validity of detected associatedSSB and SFN information in 9.10.2.5 section: - If the associatedSSB which has been detectable at least for the time period Tidentify_intra_with_index defined in clause 9.2.5.1 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again with the same spatial reception parameter provided the timing to that cell has not changed more than ± 3200 Tc, PSS/SSS detection time of associatedSSB and time period used to acquire the SFN information are equal to 0. Recommendations for 2nd round: Continue discussion in 2nd round. |

CRs/TPs

Discussion on 2nd round (if applicable)

Sub-topic 1-1 Time domain restriction for CSI-RS configuration

- Option 1: (CATT, OPPO, Xiaomi, Qualcomm, MTK, Intel, Apple, vivo)

  - Rel-16 L3 CSI-RS requirements are defined under assumption that all CSI-RS resources in the same MO are configured in the same 5ms window

    - Note: It is up to the network whether to configure all CSI-RS in the 5ms window and if CSI-RS resources are configured outside then UE may not measure it and the requirements do not apply.

- Option 2: (Huawei, CATT, Nokia, Qualcomm, Intel, Apple, vivo)

  - For inter-frequency measurements, Rel-16 L3 CSI-RS requirements are defined under assumption that all CSI-RS resources in the same MO are configured in the same 5ms window

  - For intra-frequency measurements: Rel-16 L3 CSI-RS requirements are defined under assumption that CSI-RS resources in the same MO can be configured in up to two separated 5ms windows during one CSI-RS resource period

    - The overlapping status with MG is same for the two windows.

    - The minimum periodicity of the configured CSI-RS resources is 20ms.

    - Measurement requirements are not impacted by separated 5ms windows.

| Sub-topic 1-1 Time domain restriction for CSI-RS configuration |  |
| --- | --- |
| Company | Comments |
| Xiaomi | Support option 1. |
| Qualcomm | Can compromise to Option2. Q1: the intra-frequency layer for one cell can be an inter frequency layer for another cell. How does infra plan to deploy this? Q2: Share the same view as Vivo. As UE is not required to measure the intra-frequency resources for R16 within the gap, is it necessary to have the bullet for “The overlapping status with MG is same for the two windows.”? |

Sub-topic 1-1 Time domain restriction for CSI-RS configuration

  - Agreement in GTW

    - FR1

      - For inter-frequency measurements, Rel-16 L3 CSI-RS requirements are defined under assumption that all CSI-RS resources in the same MO are configured in the same 5ms window

      - For intra-frequency measurements: Rel-16 L3 CSI-RS requirements are defined under assumption that CSI-RS resources in the same MO can be configured in up to two separated 5ms windows during one CSI-RS resource period

        - 1/ The overlapping status with MG is same for the two windows.

          - FFS how to capture this in the specification

        - 2/ The periodicity of the configured CSI-RS resources is 20ms or 40ms.

        - 3/ The gap between two 5ms windows shall be half of the CSI-RS periodicity

        - 4/ Measurement requirements are not impacted by separated 5ms windows.

        - FFS whether the conditions 1, 2, 3, 4 apply for the case of two separated 5ms windows during one CSI-RS period only or apply for all cases

    - FFS for FR2

| Issue 1-1-1 whether the conditions 1, 2, 3, 4 apply for the case of two separated 5ms windows during one CSI-RS period only or apply for all cases? |  |
| --- | --- |
| CATT | Yes, condition 1,2,3,4 are applied for the case of two separated 5ms windows. When single window is configured, the CSI-RS periodicity can be 10ms. |
| MTK | Technically, we think the conditions can be applied to 2 window case only. For single window case, we do not have concern for smaller periodicity because RAN4 already introduced a lower bound in the measurement delay max(200ms, ceil( [5] x Kp_CSI-RS) x CSI-RS period) x CSSFintra |
| Nokia | Yes, same understanding with CATT and MTK. |
| Huawei | Yes, condition 1,2,3,4 are applied for the case of two separated 5ms windows. |
| OPPO | Yes for 2 window case only |
| CMCC | Yes, the conditions only apply for the case of two separated 5ms windows. |

| Issue 1-1-2 whether the agreements are applied for FR2? |  |
| --- | --- |
| CATT | Yes, the agreements should also be applied for FR2. |
| MTK | No. As we explained during GTW, the additional UE complexity in FR2 is not trivial. Interested companies can check R4-1914900 to understand UE’s complexity. Note that R4-1914900 was written based on SSB only, and does not yet consider the 2 window structure for L3 CSI-RS measurement.. |
| Huawei | We suggest to keep FR2 FFS until next meeting. We may need to check issues from both NW and UE side. |
| OPPO | Prefer not, but open to further discuss. |

Sub-topic 1-2 UE behavior when the timing offset exceeds the threshold with single FFT assumption

- Option 1: (Nokia, Intel, Apple)

  - For intra-frequency measurement, the UE is not required to measure the configured CSI-RS resources of a neighbour cell if the symbol level misalignment between serving and the corresponding neighbour cell exceeds the threshold.

  - For inter-frequency measurement, UE can pick up any cell as the reference cell per frequency layer. UE is not required to measure the configured CSI-RS resources of a neighbour cell if the symbol level misalignment between the reference cell and the corresponding neighbour cell belonging to the same frequency layer exceeds the threshold.

- Option 2: (Huawei, CATT, OPPO, Xiaomi, Qualcomm, MTK, Intel, vivo)

  - No spec updates are needed.

| Sub-topic 1-2 UE behavior when the timing offset exceeds the threshold with single FFT assumption |  |
| --- | --- |
| Company | Comments |
| Xiaomi | Option 2, we still think it is up to UE implementation, no need to have such clarification. |
| Qualcomm | Option2 is supported. As we commented in GTW, UE will take into account the cell time offset for measuring, but the limitation of R16 shall not be deemed to be resolved by UE alone. Infra is expected to improve the reliability by correlating the improvement as seen in the reported RSRP v.s. SINR for certain validation as well. More importantly, without clear spec implications, it is not agreeable to introduce above requirements. |
| CATT | Option 2 is slightly preferred. Even if option 1 is introduced, the measurement is still up to UE implementation. |
| MTK | Support Option 2. This should be left to UE implementation. BTW, we do not have the requirement on how UE can accurately estimate the timing difference. |
| Nokia | We can understand the concern that measurement is up to UE implementation. But without any restriction on UE behavior, network cannot use the received CSI-RS measurement report due to the uncertainty of the performance. To make the whole feature workable, what do you see the compromised solution as below? This may leave measurement to UE implementation, but network does not expect any report if the timing offset exceeds the threshold. Would this be acceptable? - For intra-frequency measurement, the UE is not required to measure report the configured CSI-RS resources of a neighbour cell if the symbol level misalignment between serving and the corresponding neighbour cell exceeds the threshold. - For inter-frequency measurement, UE can pick up any cell as the reference cell per frequency layer. UE is not required to measure report the configured CSI-RS resources of a neighbour cell if the symbol level misalignment between the reference cell and the corresponding neighbour cell belonging to the same frequency layer exceeds the threshold. |
| Huawei | Slightly prefer option 2. In our view this should be up to UE implementation, so the suggested change may be a bit redundant and anyway we cannot capture everything UE is not required to do. |
| OPPO | Support option 2. It is up to UE implementation, and no need to have such clarification. |

Sub-topic 1-3 Time validity of the detected associatedSSB

- Option 1: (Huawei, CATT, OPPO, Qualcomm, vivo)

  - The associatedSSB is detected if it has been meeting the relevant cell identification requirement during the last 5 seconds.

- Option 2: (Huawei, Qualcomm, Apple, vivo)

  - The timing information of CSI-RS resources for L3 measurement can be assumed as known if the associatedSSB has been detectable during the last 5 seconds.

- Option 3: (Huawei, Nokia, MTK)

  - Adding the time validity of detected associatedSSB and SFN information in 9.10.2.5 section:

    - If the associatedSSB which has been detectable at least for the time period Tidentify_intra_with_index defined in clause 9.2.5.1 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again with the same spatial reception parameter provided the timing to that cell has not changed more than ± 3200 Tc, PSS/SSS detection time of associatedSSB and time period used to acquire the SFN information are equal to 0.

| Sub-topic 1-3 Time validity of the detected associatedSSB |  |
| --- | --- |
| Company | Comments |
| Qualcomm | Option1 or Option2. Option3 appears to be an optimization for skipping PSS/SSS detection in the special case when the associated SSB is re-acquired. We think a minimum requirement as entailed in the option1 or 2 suffices. |

Sub-topic 1-3 Time validity of the detected associatedSSB

  - Agreements in GTW:

    - Adding the time validity of detected associatedSSB and SFN information in 9.10.2.5 section:

      - If the associatedSSB which has been detectable at least for the time period Tidentify_intra_with_index defined in clause 9.2.5.1 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again with the same spatial reception parameter provided the timing to that cell has not changed more than ± 3200 Tc, PSS/SSS detection time of associatedSSB and time period used to acquire the SFN information are equal to 0.

    - The same requirements apply for the case when the associatedSSB remains detectable. FFS how to capture this in the specification.

Moderator: Collect views on how to capture the agreements in specification. If souring company of the CR has suggestion, it can directly be discussed in the CR review.

| The same requirements apply for the case when the associatedSSB remains detectable. FFS how to capture this in the specification. |  |
| --- | --- |
| Company | Comments |
| MTK | We believe this issues has been resolved in offline discussion. Perhaps the proponent will update the CR soon. |
| Huawei | After offline discussion, consensus are reached. The above description can cover the case “when the associatedSSB remains detectable”. As the case “becomes undetectable for a period ≤ 5 seconds” , the period can also be zero second. Then it means associatedSSB always remain detectable. Vivo provided two commas change for easy reading. Huawei had circulated the updated version#2. |
| OPPO | OK with the wording in the corresponding CR: If the associatedSSB, which has been detectable at least for the time period Tidentify_intra_with_index defined in clause 9.2.5.1, becomes undetectable for a period ≤ 5 seconds and then the associatedSSB becomes detectable again with the same spatial reception parameter provided the timing to that cell has not changed more than  3200 Tc, PSS/SSS detection time and time period used to acquire the SFN information are equal to 0. |

Topic #2: CSI-RS RRM performance requirements

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2109082 | CATT | DraftCR for CSI-RSRQ |
| R4-2109083 | CATT | DraftCR for CSI-RSINR |
| R4-2109086 | CATT,OPPO | Big CR |
| R4-2109179 | MediaTek Inc. | Proposal 1: Specify Es/Iot upper bound for the 2nd side condition of CSI-SINR accuracy requirement at 4dB. |
| R4-2109507 | CMCC | Proposal 1: for CSI-SINR measurement accuracy requirements with side condition #2 (¦TΔ¦≤ CP), we can accept Es/Iot ≤ [6] dB to move forward. |
| R4-2110057 | OPPO | CR |
| R4-2110905 | Huawei, HiSilicon | Proposal: The Es/Iot upper bound for side condition #2 in CSI-SINR accuracy requirements is 4dB. |
| R4-2110906 | Huawei, HiSilicon | CR |
| R4-2109929 | vivo | CR |
| R4-2109930 | vivo | Cat A CR |

Open issues summary

Sub-topic 2-1 Valid condition of CSI-RS configuration in the specification

Moderator: It was agreed in previous meeting to define CSI-RS based measurement requirements for CSI-RS configuration with {D=3 with PRBs ≥ 48}. But for the valid condition of CSI-RS configuration in CSI-RS based measurement accuracy requirements (including CSI-RSRP, CSI-RSRQ and CSI-SINR), there are different descriptions in different CRs. This issue is to unify the woding in the specification.

Issue 2-1: How to specify valid condition of CSI-RS configuration condition for accuracy requirements?

Proposals:

- Option 1: (OPPO, Huawei, CATT)

  - The accuracy requirements in Table 10.1.2.3.2-1 are valid under the following conditions:

    - The bandwidth of CSI-RS resource is no less than 48PRB when density is 3.

- Option 2: (In current CSI-RSRP accuracy in big CR)

  - The accuracy requirements in Table 10.1.2.3.2-1 are valid under the following conditions:

    - The bandwidth of CSI-RS resource is 48PRB when density is 3.

- Recommended WF

  - Need more discussion.

| Issue 2-1: How to specify valid condition of CSI-RS configuration condition for accuracy requirements? |  |
| --- | --- |
| Company | Comments |
| Huawei | Option 1. If option 2 is adopted, it means the accuracy requirements apply only for 48 RB and there is no requirement for other RB numbers. We believe this is not the intention. |
| CATT | Support option 1. The accuracy requirements should be applied when CSI-RS bandwidth is no less than 48PRB. |
| OPPO | Prefer Option 1. CSI-RS configuration condition for accuracy requirements should be aligned with the previous agreements that {BW ≥ 48PRB with D = 3}. |
| Nokia | We support Option 2. This condition is defined to indicate where/when the accuracy performance is derived. When CSI-RS is configured with a bandwidth higher than 48PRBs, we can expect a performance better than the value in the table. This has been discussed when defining the other measurement performance. For instance, we have the same for CSI-RS based L1-RSRP measurement and CLI measurements. Probably we can follow the same here. - The bandwidth of CSI-RS is 48 PRBs and the density is 3. |
| Xiaomi | Option 1, the performance requirements should be applied to larger CSI-RS bandwidth. |
| Qualcomm | Option1 is fine. More RBs shall help meeting with the RAN4 accuracy, which is the minimum requirement as we understand. |
| MTK | Support Option 1 |
| Apple | Prefer to option 1 |
| CMCC | We are OK with option 1, and we think the similar wording is already adopted in the agreed delay requirements (see below) |
| vivo | Option 1 is more generic. |

Sub-topic 2-2 CSI-SINR measurement accuracy requirements

Agreements in RAN4#98bis-e meeting:

  - Specify CSI-SINR accuracy requirement based on the following conditions on timing offset between the reference measurement timing and the target CSI-RS (TΔ) and Es/Iot side condition(s)

    - Side condition #1:

      - |TΔ|≤ CP/2 and Es/Iot ≤ 15 dB

    - Side condition #2

      - |TΔ|≤ CP and Es/Iot ≤ [6] dB

      - Note: the value of Es/Iot can be revisited based on additional simulation results in next meeting.

      - No dedicated test cases will be introduced for side condition #2

Issue 2-2: Es/Iot upper bound for side condition #2 of CSI-SINR accuracy requirements?

Proposals

- Option 1: (CMCC)

  - 6dB

- Option 2: (MTK, Huawei)

  - 4dB

- Recommended WF

  - Need more discussion.

| Issue 2-2: Es/Iot upper bound for side condition #2 of CSI-SINR accuracy requirements? |  |
| --- | --- |
| Company | Comments |
| Huawei | Support option 2 based on our simulation results. |
| CATT | Prefer option 1. From the simulation results in last meeting and this meeting, there are following values for the Es/Iot value of side condition #2: 12dB, 10dB, 6dB, 4dB, 0dB. We think 6dB is already a compromise value considering all the simulation results. |
| OPPO | No strong view. |
| Xiaomi | Option 2. |
| Qualcomm | Option2 can be supported as UE vendors have concerns. |
| MTK | Support option 2 based on our simulation results. |
| Apple | Either option is OK |
| CMCC | Our preference is option 1, but considering the concern from UE vendors and limited timeline, we can compromise to option 2 to move forward. |
| Vivo | Either option is fine. |

Companies views’ collection for 1st round

Open issues

CRs/TPs comments collection

Major close to finalize Wis and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going Wis, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2109082 (CATT) | Qualcomm: The notes about reference measurement timing seems missing in 10.1.8.2.1 and 10.1.10.2.1? |
| R4-2109083 (CATT) | Qualcomm: Suggest hold until the topic of CSI-SINR accuracy side conditions is concluded. Thanks |
| R4-2110057 (OPPO) | CATT: the updates on timing offset was already included in the endorsed draft big CR in last meeting. The CSI-RS configuration depends on the conclusion of sub-topic 2-1 |
|  | OPPO: Thanks for the comments. We can focus on the CSI-RS configuration. |
|  | Nokia: For intra-frequency measurements, it would be clearer to indicate the reference timing is serving cell timing as we anyway assume single FFT in Rel16. |
| R4-2110906 (Huawei) | Nokia: This depends on the conclusion of Subtopic 2-2. |
| R4-2109929 (vivo) | CATT: no formal CR is needed for performance part, it has been included in the endorsed draft big CR. |
|  | Vivo: okay. The CR can then be merged. |

Summary for 1st round

Open issues

|  | Status summary |
| --- | --- |
| Sub-topic #2-1 | Tentative agreements: None Candidate options: - Option 1: (Huawei, CATT, OPPO, Xiaomi, Qualcomm, MTK, Apple, CMCC, vivo) - The accuracy requirements in Table 10.1.2.3.2-1 are valid under the following conditions: - The bandwidth of CSI-RS resource is no less than 48PRB when density is 3. - Option 2: (Nokia) - The accuracy requirements in Table 10.1.2.3.2-1 are valid under the following conditions: - The bandwidth of CSI-RS resource is 48PRB when density is 3. Recommendations for 2nd round: Continue discussion in 2nd round. |
| Sub-topic #2-2 | Tentative agreements: Es/Iot upper bound for side condition #2 of CSI-SINR accuracy requirements is 4dB. Candidate options: None Recommendations for 2nd round: None. |

CRs/TPs

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Sub-topic 2-1 Valid condition of CSI-RS configuration in the specification

- Option 1: (Huawei, CATT, OPPO, Xiaomi, Qualcomm, MTK, Apple, CMCC, vivo)

  - The accuracy requirements in Table 10.1.2.3.2-1 are valid under the following conditions:

    - The bandwidth of CSI-RS resource is no less than 48PRB when density is 3.

- Option 2: (Nokia)

  - The accuracy requirements in Table 10.1.2.3.2-1 are valid under the following conditions:

    - The bandwidth of CSI-RS resource is 48PRB when density is 3.

  - Agreements in GTW:

    - The accuracy requirements in Table 10.1.2.3.2-1 are valid under the following conditions:

      - The bandwidth of CSI-RS is 48 PRBs and the density is 3.

      - The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 10.1.2.3.2-1.

| Sub-topic 2-1 Valid condition of CSI-RS configuration in the specification |  |
| --- | --- |
| Company | Comments |
| Xiaomi | Option 1 |
| Qualcomm | Follow the guidance of chairman’s notes after the GTW. |
| Nokia | Follow GTW agreements. |
| OPPO | Follow GTW agreements. And ok to update the CRs based on such agreements. |

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on CSI-RS based L3 measurement requirements | CATT | Including core part and performance part |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2109078 | CR on CSI-RS intra-frequency requirement and scheduling restriction | CATT | Revised |  |
| R4-2109927 | CR to 38.133 Correction on core requirements for CSI-RS based measurement | vivo | Revised |  |
| R4-2110386 | Adding intra-frequency CSI-RS measurement in CSSF | Huawei | Agreeable | The comment seems not for the CR contents. |
| R4-2109079 | CR on CSI-RS based measurement requirements | CATT | Revised |  |
| R4-2109177 | CR for clarification on frequency layer merging R16 | MTK | Return to | Check whether it is OK after clarification. If no further comments, it will be agreed in 2nd round. |
| R4-2109553 | 38.133 CR on the CSI-RS based measurement requirements | Nokia | Revised |  |
| R4-2110365 | CR on time validity of the detected associatedSSB | Huawei | Revised |  |
| R4-2110903 | CR on CSI-RS measurement window | Huawei | Revised |  |
| R4-2111412 | CR on CSSF for CSI-RS L3 RRM R16 | Apple | Revised |  |
| R4-2109082 | draft CR on performance requirement for CSI-RSRQ | CATT | Revised |  |
| R4-2109083 | draft CR on performance requirement for CSI-SINR | CATT | Merged |  |
| R4-2110057 | CR to TS 38.133 on performance requirements for CSI-RS based L3 measurement | OPPO | Revised |  |
| R4-2110906 | draftCR on CSI-SINR accuracy requirements | Huawei | Revised |  |
| R4-2109929 | CR to 38.133 on SA event triggered reporting tests with gap for NR neighbor cell in FR2 | vivo | Not pursued |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2108317 | WF on CSI-RS based L3 measurement requirements | CATT | Agreeable |  |
| R4-2108318 | CR on CSI-RS intra-frequency requirement and scheduling restriction | CATT | Agreeable |  |
| R4-2108319 | CR to 38.133 Correction on core requirements for CSI-RS based measurement | vivo | Agreeable |  |
| R4-2108320 | CR on CSI-RS based measurement requirements | CATT | Agreeable |  |
| R4-2109177 | CR for clarification on frequency layer merging R16 | MTK | Agreeable |  |
| R4-2109553 | 38.133 CR on the CSI-RS based measurement requirements | Nokia | Postponed |  |
| R4-2108322 | CR on time validity of the detected associatedSSB | Huawei | Agreeable |  |
| R4-2108323 | CR on CSI-RS measurement window | Huawei | Agreeable |  |
| R4-2111412 | CR on CSSF for CSI-RS L3 RRM R16 | Apple | Postponed |  |
| R4-2108324 | CR on CSSF for CSI-RS L3 RRM R16 | Apple | Withdrawn |  |
| R4-2108325 | draft CR on performance requirement for CSI-RSRQ | CATT | Agreeable |  |
| R4-2108326 | CR to TS 38.133 on performance requirements for CSI-RS based L3 measurement | OPPO | Agreeable |  |
| R4-2108327 | draftCR on CSI-SINR accuracy requirements | Huawei | Agreeable |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents