3GPP TSG-RAN WG4 Meeting # 99-e 													R4-2108409

Electronic Meeting, 19th – 27th May 2021

Agenda item:			9.20.1

Source:	Moderator (Ericsson)

Title:	Email discussion summary for [99-e][237] NR_pos_enh_RRM

Document for:	Information

# Introduction

The document contains discussion related to the RRM core requirements for gNB positioning measurements:

The document contains the following main topics:

- Topic #1: General aspects (Agenda item: 9.20.1)

- Topic #2: LS on gNB/UE Rx/Tx timing error mitigation (Agenda item: 9.20.1)

- Topic #3: Work plan for RRM core requirements (Agenda item: 9.20.1)

# Topic #1: General aspects

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2109103 | CATT | Observation 1: RAN4 need some feasibility study on the method of mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays provided by RAN1, and provide some possible solutions on accuracy improving if the method is not feasible. Observation 2: Some methods related to measurement and MG for reducing positioning measurement latency can be discussed and defined in RAN4 (e.g. CSSF improving, gap enhancement etc.). Details can be FFS. Observation 3: UE behaviour (e.g. the receiving of PRS and transmission of SRS) to support positioning in RRC_INACTIVE state is in RAN2 scope. Proposal 1: RAN4 should focus on the DL NR positioning first for the support of positioning in RRC_INACTIVE state. Proposal 2: RAN4 need to specify the measurement requirements and accuracy requirements for positioning in RRC_INACTIVE state after the behaviour is specified in RAN2. Proposal 3: RAN4 need to specify the performance requirements and conformance test for the supported signals in A-GNSS positioning for NR and E-UTRAN. |
| R4-2109224 | Intel Corporation | Observation 1: RAN4 is not supposed to enhance any current accuracy requirements related to mitigation of UE Rx/Tx and/or gNB Rx/Tx timing delays. Proposal 1: RAN4 considers specifying an enhanced set of accuracy requirements with much higher SINR side condition targeting latency reduction in positioning in general. Proposal 2: RAN4 to specify new measurement gap patterns dedicated to positioning with shorter MGRP. Observation 2: Potential FR2 enhancements in measurement period requirements can be considered in RAN4. Proposal 3: Specify INACTIVE mode measurement delay and accuracy requirements and potential reporting requirements in RAN4, subjecting to RAN1 design. Proposal 4: Add the newly supported A-GNSS systems in TS 38.171: BDS B2a, BDS B3I and NavIC. |
| R4-2109945 | vivo | Proposal 1: RAN4 should specify RRM requirements for UE DL measurements without measurement gap if corresponding procedures is enabled. Proposal 2: RAN4 should specify RRM requirements for UE DL measurements with reduced UE measurement time if corresponding enhanced procedures are defined. Proposal 3: RAN4 should specify RRM requirements for UE DL measurements with measurement gap with reduced UE measurement time if corresponding enhanced procedures are defined. Proposal 4: RAN4 is to discuss feasibility of TEG. Proposal 5: RAN4 is to specify RRM requirements for UE measurements in RRC-INACTIVE state. Proposal 6: RAN4 is to specify RRM requirements for UL/gNB measurements in RRC-INACTIVE state if it is to be introduced in Rel-17. Proposal 7: A-GNSS minimum performance requirements are specified for BeiDou B2a and BeiDou B3I navigation signals. Proposal 8: A-GNSS minimum performance requirements are specified for NavIC. |
| R4-2110016 | Nokia, Nokia Shanghai Bell | Proposal 1 : RAN4 needs to study characteristics of timing delays if TEG appears static or semi-static or dynamic in TX/RX scenarios with considering various front-end parameters and conditions including source of time delay. Proposal 2 : RAN4 may first visit studies to find latency bottleneck during positioning measurements including measurement gap configuration. For the latency study, RAN4 needs to refer to latency analysis from RAN2. Observation 1 : The on-demand PRS support can improves the positioning measurement latency, but RAN1/RAN2 discussions are not completed yet. Observation 2 : It is early to discuss UE measurement behaviors in inactive mode, since RAN1/2 discussion are on going. Proposal 3 : RAN4 starts with analysis on PRS resource configuration, positioning measurement period and DRX behaviors in the UE RRC_INACTIVE state. Proposal 4 : RAN4 starts discussions on performance requirements of the additional BDS and NavIC signals as extension of chapter 5,6 requirements in TS38.171 and TS36.171. |
| R4-2110231 | Ericsson | - Determine impact on positioning measurement accuracies after RAN1 progress on gNB/UE Rx/Tx timing delay aspects. - Possible reduction of measurement period through performing measurements without MG Possible reduction of measurement period through definition of new MG pattern Solution to UE positioning measurement period reduction is dependent on agreements in RAN1 and RAN2 Requirements should be defined to ensure reasonable compromise between UE power saving in RRC_INACTIVE and measurement periods Wait for further progress on the enhancements done by RAN2 and RAN3 - Positioning measurements without gaps will impact RRM requirements due to gap sharing between RRM requirements and positioning measurements - Larger MGL to accommodate reduction of measurement period may also impact serving cell procedures |
| R4-2110917 | Huawei, HiSilicon | Proposal 1: RAN4 not to define new RRM requirements or update existing RRM requirements for timing error mitigation. Proposal 2: RAN4 to consider the following enhancements for latency reduction - Measurement period based on multiple sets of {N,T} capabilities - MG-less PRS measurement Proposal 3: For DL positioning methods in RRC_INACTIVE, RAN4 to define UE requirements for PRS measurement in RRC_INACTIVE. Proposal 4: For UL and DL+UL positioning methods in RRC_INACTIVE, RAN4 to wait for the conclusions from RAN1/2 before discussing RRM impacts. Proposal 5: RAN4 to add related conditions and requirements for BDS B2a, BDS B3I and NavIC in 36.171 and 38.171. |

## Open issues summary

### Sub-topic 1-1 Mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays

According to RP-210903

- Specify methods, measurements, signalling, and procedures for improving positioning accuracy of the Rel-16 NR positioning methods by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays, including [RAN1, RAN2, RAN3, RAN4]

  - DL, UL and DL+UL positioning methods

  - UE-based and UE-assisted positioning solutions

Open issues and candidate options before e-meeting:

Issue 1-1-1: Definition of new or updated RRM requirements in relation to timing error mitigation

- Proposals

  - Option 1: Do not define or update RRM requirements based on timing error mitigation (Intel, Huawei)

  - Option 2: Feasibility study on mitigation techniques needed to be able to improve positioning accuracy (CATT)

  - Option 3: RAN4 to study characteristics on timing delay (Nokia)

  - Option 4: Determine impact on RAN4 requirements after further timing error mitigation progress from RAN1 (Ericsson)

- Recommended WF

  - Discuss the options

### Sub-topic 1-2 Latency reductions

From RP-210903:

- Specify the enhancements of signalling, and procedures for improving positioning latency of the Rel-16 NR positioning methods, for DL and DL+UL positioning methods, including:

  - Latency reduction related to the request and response of location measurements or location estimate and positioning assistance data; [RAN2, RAN3, RAN1]

  - Latency reduction related to the time needed to perform UE measurements; [RAN1, RAN4]

  - Latency reduction related to the measurement gap; [RAN1, RAN4, RAN2]

Open issues and candidate options before e-meeting:

Issue 1-2-1: General issues with latency reduction

- Proposals

  - Option 1: RAN4 to find latency bottleneck for positioning measurements including MG configuration and RAN2 latency analysis (Nokia)

  - Option 2: None

- Recommended WF

  - Discuss the topic

Issue 1-2-2: Latency enhancements in relation to UE measurement time

- Proposals

  - Option 1: RAN4 to consider specifying enhanced set of accuracy requirements with much higher SINR side condition to reduce general positioning latency (Intel)

  - Option 2: Define {N,T} capabilities to indicate measurement/processing time (T) in relation to reference signal duration (T) (Huawei)

- Recommended WF

  - Discuss the topic and the options

Issue 1-2-3: Latency enhancements in relation to measurement gaps

- Proposals

  - Option 1: Specify new measurement gap patterns with shorter repetition period (Intel)

  - Option 2: MG-less PRS measurement (Huawei)

  - Option 3: RRM requirement definition for reduction of measurement period by defining new MG configurations or MG-less measurement is dependent on agreements in RAN1/RAN2 (Ericsson, vivo)

- Recommended WF

  - Discuss the options

### Sub-topic 1-3 Inactive state positioning measurements

From RP-210903:

- Specify methods, measurements, signalling and procedures to support positioning for UEs in RRC_ INACTIVE state, for UE-based and UE-assisted positioning solutions, including [RAN2, RAN1, RAN3,RAN4]:

  - DL NR positioning methods and RAT-independent positioning methods

    - Support of UE positioning measurements for UEs in RRC_INACTIVE state

    - Reporting of positioning measurement or location estimate performed in RRC_INACTIVE when the UE is in RRC_INACTIVE state

Note: this work will be coordinated with the SDT WI.

  - As 2nd priority:

    - UL and DL+UL NR positioning methods

    - Support of gNB positioning measurements for UEs in RRC_INACTIVE state

Open issues and candidate options before e-meeting:

Issue 1-3-1: Specify RRM requirements for positioning measurements in RRC_INACTIVE state

- Proposals

  - Option 1: Focus on DL measurements first and specify measurement and accuracy requirements after RAN2 specifies behaviour in RRC_INACTIVE (CATT)

  - Option 2: Specify measurement delay and accuracy and potential reporting requirements in RAN4 based on outcome of other WGs (Intel, vivo)

  - Option 3: Define UE requirements for PRS measurement for DL positioning methods with RRC_INACTIVE and wait for RAN1/RAN2 conclusions on UL/DL+UL positioning methods (Huawei)

  - Option 4: RAN4 to analyse PRS resource configuration, positioning measurement period and DRX behaviour in UR RRC_INACTIVE state (Nokia)

- Recommended WF

  - Discuss the issue and its options

### Sub-topic 1-4 GNSS enhancements

- Support the following enhancements of A-GNSS positioning  [RAN2, RAN3, RAN4]

  - Specify support for BDS B2a signal

  - Specify support for BDS B3I signal

  - Specify support for NavIC to NR

Note: This objective is applicable to NR and E-UTRA.

Open issues and candidate options before e-meeting:

Issue 1-4-1: A-GNSS positioning enhancement

- Proposals

  - Option 1: RAN4 to specify performance requirements and conformance test for supported signals in A-GNSS positioning for NR and E-UTRAN (CATT)

  - Option 2: Add BDS B2a, BDS B3I and NavIC in TS 38.171 (Intel, vivo, Huawei)

  - Option 3: Start discussion on performance requirements of additional BDS and NavIC signals to be added in TS 38.171 and TS 36.171 (Nokia)

  - Option 4: Wait on further progress on enhancements by RAN2 and RAN3 (Ericsson)

- Recommended WF

  - Discuss above options

### Sub-topic 1-5 New RRM positioning measurement and procedural requirements and their impact on existing RAN4 requirements

- Discuss and specify new as well as the impact on the existing RAN4 requirements for positioning and other RRM measurements and corresponding procedures [RAN4]

Open issues and candidate options before e-meeting:

Issue 1-5-1: Gapless measurement

- Proposals

  - Option 1: RAN4 to discuss impact of gapless measurements on RRM requirements due to gap sharing (Ericsson)

  - Option 2: None

- Recommended WF

  - TBA

Issue 1-5-2: Larger MGL

- Proposals

  - Option 1: RAN4 to discuss impact of larger measurement gap length usage on serving cell procedures (Ericsson)

  - Option 2: None

- Recommended WF

  - TBA

## Companies views’ collection for 1st round

### Open issues

Sub topic 1-1-1 Definition of new or updated RRM requirements in relation to timing error mitigation

| Company | Comments |
| --- | --- |
| Ericsson | In our view, timing error mitigation should not impact any positioning RRM requirements. However, RAN4 can further monitor progress in RAN1 and RAN2 until the next meeting before discussing any impact on requirements. |
| CATT | We think no RRM requirements update related to timing error mitigating is needed in RAN4 so far. However, some feasibility studies are needed for the mitigation techniques provided by RAN1. |
| vivo | We agree with Ericsson. At current stage, there is no impact on RRM requirements of timing error mitigation mechanism agreed in RAN1. We should however follow further progress in other WGs to see if there would be impact on RRM requirements. In the meanwhile, feasibility of TEG should be studied in RAN4. |
| Qualcomm | Support options 3 and 4. We agree that further progress of mitigation techniques in RAN1 would be needed to determine impact on RRM requirements. At the same time RAN4 can start studying some of the assumptions in the solutions being proposed in RAN1, e.g. different timing error sources and their behavior. |
| Nokia | In fact, the questions include two sub-questions : - Updating Rel-16 RRM requirements - Definition of new RRM requirements on TE report We share the same view that TE mitigation does not impact on Rel-16 requirements on (i). But the issue of (ii) is still an open question up to RAN1 discussion. Therefore, we support option-2 and option 3 on TE. It may be too early to conclude if introducing requirements or not. RAN1 identifies the time delay is one obstacle to achieve cm-level accuracy. Therefore at least feasibility should be studied in RAN4. |
| Intel | Agree with Nokia views. |
| Huawei | We support option 1 because we understand the intention of defining TEG is to allow UE and TRP to provide more information to the LMF about the measurements, e.g. similar as time stamp. This will help to improve the positioning accuracy, but it does not aim at improving the measurement accuracy. On the other hand, since this is first meeting, we are also fine with option 2, 3 and 4 to do further study in RAN4 and wait for more RAN1 inputs before deciding the RRM impacts. We also agree with Nokia that there should be no impact to Rel-16 requirements. |
| OPPO | Support option 1 and option 2. We don’t see much clear impacts on RRM requirements due to timing error mitigation at current stage, especially for core part. |

Sub topic 1-2-1 General issues with latency reduction

| Company | Comments |
| --- | --- |
| Ericsson | RAN4 requirements to reduce latency depends on RAN1 solution. So RAN4 should wait for RAN1 agreements before discussing requirements. |
| CATT | RAN4 can define requirements depending on RAN1 solutions on latency reduction and RAN4 can also find some methods related to measurement and MG to reduce latency. |
| vivo | We also think RAN4 work on latency reduction should be triggered by other WGs. It may also be fine that RAN4 can start to work if certain issues in RAN4 expertise are identified. |
| Qualcomm | RAN4 should focus on factors that impact the measurement period requirements, including MG and CSSF, number of samples for accuracy, UE processing capability, etc. |
| Nokia | We propose first to identify latency bottleneck. RAN4 can study RAN1/2 latency reductions as well as RAN4 own issues. This proposal is aligned with studies in issue 1-2-2, 1-2-3 or further latency sources. |
| Huawei | RAN4 should focus on the latency reduction in UE measurement and MG. RAN1 is the leading WG for those two sub-objective, so RAN4 should wait for RAN1 progress. On the other hand, we are open to discuss enhancements that are RAN4 specific, if identified. |
| OPPO | We can focus on RAN4-related issues such as MG and sharing mechanism firstly. Other methods should be triggered by RAN1/RAN2. |

Sub topic 1-2-2: Latency enhancements in relation to UE measurement time

| Company | Comments |
| --- | --- |
| Ericsson | We agree that side conditions (SINR) for requirements is RAN4 issue and RAN4 can further revisit them for enhancing requirements. However, RAN4 should identify list of side conditions and not just SINR which can lead to enhance requirements. The {N,T} capabilities to indicate measurement/processing time (T) should be within RAN1/2 scope not RAN4. |
| CATT | We think using much higher SINR condition is a method to improve accuracy, but how it can reduce latency? The {N,T} capability has been defined and how to report this capability is UE implementation. We think we cannot request UE to indicate different capability based on different PRS configuration and this is not in RAN4 scope. |
| vivo | For option 1, not sure how latency can be reduced by specifying accuracy requirements with high SINR. The latency reduction is more related to measurement period requirements. For option 2, it is more of RAN1/2 issue. |
| Qualcomm | Agree that RRM requirements, including accuracy requirements, many need to be revisited to support latency reduction objectives. Also agree with Ericsson’s comment that there are additional factors besides SINR side-condition that should be considered, e.g. accuracy improvements from timing error mitigation. |
| Nokia | For option-1, we understand the study intention that measurement period can be reduced at high SNR channels. If predicting possible outcomes, RAN4 may reach to difference measurement time period requirement depending on SNR condition. It wouldn’t be easy to slice SNR levels mapping to accuracy and measurement period. For option-2, this can be one RAN4 driven latency study. We expect different UE capability associated with measurement/processing time will appear in the market. RAN1 also considered PRS symbol design for various RX approaches. So, we agree that there can be some room to reduce latency from buffer/processing time in UE implementation. |
| Intel | Most of the comments above are feasible. Both options are possible. Let’s have further discussion. |
| Huawei | We suggest to keep both options. On option 1 we understand the intention is to shorten the measurement period in high SINR condition. We are open to discuss this. On option 2, we agree that the UE capability should be discussed and defined by RAN1. If it is introduced, RAN4 needs to adapt the measurement period requirements. |
| OPPO | Option 1 is reasonable, and the details, such as the reduced samplings number, SINR threshold to reduce the samplings number needs further study. For option 2, the PRS processing capability is defined by RAN1 and is out of RAN4 scope. |

Sub topic 1-2-3: Latency enhancements in relation to measurement gaps

| Company | Comments |
| --- | --- |
| Ericsson | We support option 3. RAN4 should wait for RAN1/2 agreements whether PRS measurements can be done without gaps. RAN4 can discuss new gap patterns from next meeting. However, they do not need to be of shorter MGRP. They may also have larger MGL. |
| CATT | We think whether to introduce gapless measurement should be defined in RAN1/2. The need for new measurement gap can be FFS. |
| vivo | We support option 3. Since it is to be discussed in RAN1/2 firstly, RAN4 would be better to wait until there is progress to trigger RAN4 work. |
| Qualcomm | Support option 3. |
| Nokia | In general, we support studies about MG related latency reduction. RAN1/2 already started related discussions. RAN1/2 agrees to feasibility studies of latency enhancements in relation to measurement gaps, RAN4 accordingly studies. We see there are a few frameworks from RAN1/2 of - MG-less positioning measurement, - Pre-configured MG-based measurement - Multiple concurrent gap measurement Further details of each of the framework seem still FFS in RAN1/2 discussions. RAN4 needs to monitor RAN1/2 discussion status. |
| Intel | Understand option3 but we would rather like to think that defining MG pattern may be RAN4 decision. |
| Huawei | We can support option 3 which includes option 2 from our side. On option 1, the smallest MGRP in existing MGPs is 20ms, do we still need an even shorter one? |
| OPPO | Support option 3. The benefits of introducing new gap pattern with short MGRP or long MGL are limited in the comparison with the cost of noticeable performance loss on data throughput, which is not desirable from our side. |

Sub topic 1-3-1: Specify RRM requirements for positioning measurements in RRC_INACTIVE state

| Company | Comments |
| --- | --- |
| Ericsson | We agree with option 1 and option 2, which are fundamentally the same. |
| CATT | Support option 1. And we think option 1, option 2 and option 3 are aligned. The PRS configuration and UE behavior should be RAN1/2 scope. |
| vivo | Option 1, option 2 and option 3 are quite similar. We think RAN4 work can start if there is some conclusions in RAN2/1. |
| Qualcomm | In our view, options 1, 2 and 3 are well aligned. The take-aways are to prioritize DL positioning methods and wait for outcome in other WGs (RAN1/RAN2). |
| Nokia | We support studying RRM requirements for positioning measurements in RRC_INACTIVE state. Option 1,2,3 are fine to start. |
| Huawei | We can support option 1, 2 and 3. Option 4 seems to be in a more detailed level which can be discussed later. |
| OPPO | We can support option 1 and option 2. |

Sub topic 1-4-1: A-GNSS positioning enhancement

| Company | Comments |
| --- | --- |
| Ericsson | We agree that performance requirements for additional BDS and NavIC signals need to be defined. However, RAN4 should wait for completion / progress of corresponding LPP signalling in RAN2. |
| CATT | We agree that the performance requirements should be defined in RAN4. Option 1, option 2 and option 3 are aligned. |
| vivo | We agree performance requirements for additional BDS and NavIC need to be specified. |
| Qualcomm | Option 2. It is within scope of the WI. The timing of the work should be specified in the WP and subject to progress in other WGs. |
| Nokia | Option 1,2,3 are aligned. Performance requirements of the additional BDS and NavIC signals can be defined accordingly as signaling framework in RAN2/3. |
| Huawei | We can support option 1, 2 and 3. |

Sub topic 1-5-1: Gapless measurement

| Company | Comments |
| --- | --- |
| Ericsson | If gapless PRS measurements are defined based on RAN1 agreement, then RAN4 needs to assess impact on RRM requirements. This issue can be addressed after latency enhancements related measurement procedures have agreed/progressed in RAN1/2 (related to issues 1-2-1-1.2.3). |
| CATT | Whether to introduce gapless measurement should be discussed first. |
| vivo | It may be considered as part of work for gapless measurement if issue is identified. |
| Qualcom | See our response to issue 1-2-3. |
| Nokia | As answered from issue 1-2-3, there are a few framework discussions regarding latency reduction due to MG. Which framework is more feasible is up to RAN1/2 discussion. We are ok to include the discussions on MG-less positioning measurement, but it seems early stage to understand the feature. |
| Huawei | We understand this is already discussed in Issue 1-2-3. |

Sub topic 1-5-2: Larger MGL

| Company | Comments |
| --- | --- |
| Ericsson | Related to issue 1-2-3. New MG patterns can be discussed in RAN4. |
| CATT | Whether to introduce new gap pattern should be discussed first. |
| vivo | Not clear now why longer MGL is needed. |
| Qualcomm | Option 2. Unclear if it’s needed at this point. |
| Nokia | We need more information to understand the proposal. Larger MGL has pro and cons. More dynamic MG or MG-less are preferred. |
| Huawei | The necessity to introduce larger MGL should be clarified first. |
| OPPO | Does option 1 try to discuss the MG with MGL larger than 20ms, which is the maximum MGL value defined in current spec? If so, we have concerns that it will cause long interruptions on data transmission. |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1 | Issue 1-1-1: Definition of new or updated RRM requirements in relation to timing error mitigation Tentative agreements: Monitor progress of other RAN WGs to determine impact of timing error mitigation on existing RRM requirements or the need to define new RRM requirements. No impact on current RRM specification can be identified. Candidate options: Study feasibility of proposed timing error mitigation mechanisms by RAN1 in RAN4 - Option 1: Yes - Option 2: TBA Recommendations for 2nd round: Discuss feasibility study on timing error enhancements. |
| Sub-topic #2 | Issue 1-2-1: General issues with latency reduction Tentative agreements: RAN4 requirements definition on latency reduction are reliant on RAN1 solutions, which RAN4 needs to discuss after RAN1 has made agreement. Candidate options: Analyze factors that impact measurement requirements from RAN4 perspective to identify possible enhancements regarding latency reduction - Option 1: Yes - Option 2: TBA Recommendations for 2nd round: Discuss further |
| Sub-topic #2 | Issue 1-2-2: Latency enhancements in relation to UE measurement time Tentative agreements: none Candidate options: Define {N,T} capabilities to indicate measurement/processing time (T) in relation to reference signal duration (T): - Option 1: Yes - Option 2: No, not in RAN4 scope Recommendations for 2nd round: Discuss and identify (further) side conditions which can lead to enhanced requirements regarding UE measurement time |
| Sub-topic #2 | Issue 1-2-3: Latency enhancements in relation to measurement gaps Tentative agreements: Wait on outcome of RAN1/2 on gapless measurement. Introducing new measurement gap patterns is FFS and shall be addressed in the next meeting. Candidate options: none Recommendations for 2nd round: none |
| Sub-topic #3 | Issue 1-3-1: Specify RRM requirements for positioning measurements in RRC_INACTIVE state Tentative agreements: Specify measurement delay and accuracy and potential reporting requirements in RAN4 based on outcome of other WGs - Define UE requirements for PRS measurement for DL positioning methods with RRC_INACTIVE and wait for RAN1/RAN2 conclusions on UL/DL+UL positioning methods Candidate options: none Recommendations for 2nd round: Discuss UE requirements for PRS measurement for DL positioning methods with RRC_INACTIVE |
| Sub-topic #4 | Issue 1-4-1: A-GNSS positioning enhancement Tentative agreements: Performance requirements for additional BDS and NavIC signals need to be defined based on progress of RAN2 work. Candidate options: none Recommendations for 2nd round: none |
| Sub-topic #5 | Issue 1-5-1: Gapless measurement Tentative agreements: Wait for RAN1/2 framework agreements on gapless measurements Candidate options: none Recommendations for 2nd round: merge with issue 1-2-3 |
| Sub-topic #5 | Issue 1-5-2: Larger MGL Tentative agreements: none Candidate options: Introduction of new measurement gap patterns with larger MGL - Yes - No Recommendations for 2nd round: Discuss and address if MGL > 20ms concerns if needed. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |

## Discussion on 2nd round (if applicable)

### Sub-topic 1-1 Mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays

Issue 1-1-1: Definition of new or updated RRM requirements in relation to timing error mitigation

Tentative agreements:

Monitor progress of other RAN WGs to determine impact of timing error mitigation on existing RRM requirements or the need to define new RRM requirements. No impact on current RRM specification can be identified.

Candidate options:

Study feasibility of proposed timing error mitigation mechanisms by RAN1 in RAN4

- Option 1: Yes

- Option 2: TBA

Recommendations for 2nd round:

Discuss feasibility study on timing error enhancements.

| Company | Comments |
| --- | --- |
| Ericsson | This is aligned with discussion in topic #2, therefore we are fine to further study the proposed timing error mitigation mechanisms by RAN1 in RAN4. |
| Huawei | We are open to the feasibility study, so option 1 is fine. |
| Nokia | Support option-1. |

### Sub-topic 1-2 Latency reductions

Issue 1-2-1: General issues with latency reduction

Tentative agreements:

RAN4 requirements definition on latency reduction are reliant on RAN1 solutions, which RAN4 needs to discuss after RAN1 has made agreement.

Candidate options:

Analyze factors that impact measurement requirements from RAN4 perspective to identify possible enhancements regarding latency reduction

- Option 1: Yes

- Option 2: TBA

Recommendations for 2nd round: Discuss further

| Company | Comments |
| --- | --- |
| Huawei | We are open to discuss possible RAN4 specific enhancement, so option 1 is fine. |
| Nokia | We are also open that RAN4 analyzes latency bottlenecks or factors from RAN4 perspective. Companies can discuss issues from Rel-16 frameworks for Rel-17 improvement. |

Issue 1-2-2: Latency enhancements in relation to UE measurement time

Tentative agreements:

none

Candidate options:

Define {N,T} capabilities to indicate measurement/processing time (T) in relation to reference signal duration (T):

- Option 1: Yes

- Option 2: No, not in RAN4 scope

Recommendations for 2nd round:

Discuss and identify (further) side conditions which can lead to enhanced requirements regarding UE measurement time

| Company | Comments |
| --- | --- |
| Ericsson | Option 2. |
| Huawei | We suggest to remove the bullets related to capabilities {N,T} in the WF rather than concluding it is not in RAN4 scope. Depending on RAN1 outcome, this may have some impacts on RAN4 requirements, and we can start the discussion when RAN1 has enough agreements. |
| Nokia | We support studying with option-1. RAN4 can further discuss enhancement from UE capability {N,T} under Rel-16 framework in Rel-17 phase. Also, RAN1/2 enhancement scope is included here. |

Issue 1-2-3: Latency enhancements in relation to measurement gaps

Tentative agreements:

Wait on outcome of RAN1/2 on gapless measurement. Introducing new measurement gap patterns is FFS and shall be addressed in the next meeting.

Candidate options:

none

Recommendations for 2nd round:

none

### Sub-topic 1-3 Inactive state positioning measurements

Issue 1-3-1: Specify RRM requirements for positioning measurements in RRC_INACTIVE state

Tentative agreements:

Specify measurement delay and accuracy and potential reporting requirements in RAN4 based on outcome of other WGs

- Define UE requirements for PRS measurement for DL positioning methods with RRC_INACTIVE and wait for RAN1/RAN2 conclusions on UL/DL+UL positioning methods

Candidate options:

none

Recommendations for 2nd round:

Discuss UE requirements for PRS measurement for DL positioning methods with RRC_INACTIVE

| Company | Comments |
| --- | --- |
| Huawei | There is not enough input from RAN1 or RAN2. RAN4 needs to wait before discussing the exact requirements. |
| Nokia | Wait for RAN2/1 inputs for further discussion |

### Sub-topic 1-4 GNSS enhancements

Tentative agreements:

Performance requirements for additional BDS and NavIC signals need to be defined based on progress of RAN2 work.

Candidate options:

none

Recommendations for 2nd round:

none

### Sub-topic 1-5 New RRM positioning measurement and procedural requirements and their impact on existing RAN4 requirements

Issue 1-5-1: Gapless measurement

Tentative agreements:

Wait for RAN1/2 framework agreements on gapless measurements

Candidate options:

none

Recommendations for 2nd round:

merged with issue 1-2-3

Issue 1-5-2: Larger MGL

Tentative agreements:

none

Candidate options:

Introduction of new measurement gap patterns with larger MGL

- Yes

- No

Recommendations for 2nd round:

Discuss and address if MGL > 20ms concerns if needed.

| Company | Comments |
| --- | --- |
| Ericsson | New MG patterns including longer MGL can be discussed in RAN4 |
| Huawei | We do not see this is in the scope of the WI. |
| Nokia | It is unclear if larger MGL is regarded as measurement gap enhancement. Benefits need to be identified with consensus. |

# Topic #2: LS on gNB/UE Rx/Tx timing error mitigation

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2109101 | CATT | Proposal 1: The issues related to the definitions of the timing errors and the Rx/Tx/RxTx TEGs: - The remaining Rx/Tx time delays after the calibration cannot be evaluated by the UE/TRP. So it is unrealistic for UE/TRP to provide the remaining Rx/Tx time delays; - FFS: Whether UE/TRP can provide the differences of the remaining Rx/Tx time delays - If the uncalibrated time delay is used, it can only be estimated by UE/TRP itself and has been compensated to the Rx/Tx reference point. The calibration error (remaining Rx/Tx time delay) is included in the reported measurement and may be also in the uncalibrated time delay. - UE/TRP may group the (remaining/uncalibrated) Rx/Tx time delays based on the RF chains and antenna panel used for receiving the DL PRS or transmitting the UL SRS. In this case, the timing errors in the same Rx/Tx/RxTx TEGs will be within the same margin. However, UE/TRP may not be able to ensure the timing errors in different Rx/Tx/RxTx TEGs are not within the same margin. - The issue applies to all the definitions of the UE/TRP Rx/Tx/RxTx TEG. Proposal 2: Possible solution for enhancing the method of accuracy improving provided in LS: e.g. support a TRP providing the association information of DL PRS resources with Tx TEGs to the LMF if the TRP has multiple TEGs which are associated with different antenna panels or antenna arrays. And the solution can also be used for TRP Rx if needed but not applied for UE Rx/Tx. |
| R4-2109945 | vivo | Proposal 4: RAN4 is to discuss feasibility of TEG. Proposal 9: RAN4 to discuss and conclude whether UE Rx timing error can be grouped based on following factors, e.g., Antenna panel, RF chain design, frequency, baseband sampling rate (or PRS BW and SCS) etc. Proposal 10: RAN4 to discuss and conclude whether UE Tx timing error can be grouped based on following factors, e.g., Antenna panel, RF chain design, frequency, baseband sampling rate (or PRS BW and SCS), SRS antenna switching etc. |
| R4-2110016 | Nokia, Nokia Shanghai Bell | Observation 3 : RAN1 has agreed to support TEG studies firstly for DL-TDOA measurement case. It is expected that RAN1 continues to discuss the time delay impacts for other time-based methods. Observation 4 : A goal of TEG study is to achieve cm-level positioning accuracy by mitigating this timing error. It may be related with RF margin in accuracy requirements. Observation 5 : If TEG dynamically changes, measuring the absolute Tx/Rx timing errors in DL-TDOA may be difficult as they are residual errors after calibration. Observation 6 : If TEG statically or semi-statically changes, then measuring the absolute Tx/Rx timing errors in DL-TDOA may be possible depending on TX / RX scenarios. Observation 7 : RAN1 question in LS seems to have broad scopes, and TEG measurement feasibility is one of RAN4 study topics in Rel-17 NR positioning phase. As for now, it may be too early to clearly answer to RAN1 question. Proposal 5 : Propose possible replies from RAN4 as - RAN4 studies if TEG appears static or semi-static or dynamic in TX/RX scenarios with considering various front-end parameters and conditions in Rel-17 phase. - If TEG statically or semi-statically changes and measurable, feasibility study on the absolute Tx/Rx timing error estimation in DL-TDOA can be possible. |
| R4-2110233 | Ericsson | - TRPs have distinct non-moving location and known beam direction i.e. sector orientation - The timing error differences between multiple T/RX chains within one TRP are known timing errors due to phase calibration and are accounted for by adjusted TRX timing - Residual timing error differences between multiple T/RX chains within one TRP after phase calibration are small and therefore it is not necessary to distinguish via TEG reporting - A TRP will only feature one TEG, therefore reporting of TEG is unnecessary. - Multiple gNB Rx and Tx TEG is not supported. Send reply LS to RAN1 on the issue. |
| R4-2110917 | Huawei, HiSilicon | Proposal 6: RAN4 does not need to send reply LS for R1-2104111. |

## Open issues summary

According to R1-2104111:

Agreement:

- Support the following for mitigating TRP Tx timing errors and/or UE Rx timing errors for DL TDOA

  - Support a UE to provide the association information of RSTD measurements with UE Rx TEG(s) to the LMF when the UE reports the RSTD measurements to the LMF if the UE has multiple TEGs

  - Support a TRP providing the association information of DL PRS resources with Tx TEGs to the LMF if the TRP has multiple TEGs

  - Support the LMF to provide the association information of DL PRS resources with Tx TEGs to a UE for UE-based positioning if the TRP has multiple TEGs

  - FFS: the details of the signalling, procedures, and UE capability

- Send an LS to RAN4 to check if there is any issue to support the above enhancements

### Sub-topic 2-1 LS discussion

Open issues and candidate options before e-meeting:

Issue 2-1-1: Send reply LS to RAN1

- Proposals

  - Option 1: Not needed (Huawei)

  - Option 2: Yes (Ericsson, Nokia, CATT)

- Recommended WF

  - Discuss TEG feature feasibility to reach conclusion if reply LS is needed and what to include

### Sub-topic 2-2 Discussion on TEG and feasibility

Open issues and candidate options before e-meeting:

Issue 2-2-1: Calibration error/residual timing error after calibration

- Proposals

  - Option 1: The remaining Rx/Tx time delays after the calibration are unknown to the UE/TRP and therefore cannot be provided (CATT)

  - Option 2: TBA

- Recommended WF

  - Discuss topic

Issue 2-2-2: TEG grouping

- Proposals

  - Option 1: (CATT)

    - UE/TRP may group the timing error (with or without calibration) based on RF chains and antenna panel, such that timing errors in the same group are within certain margin

    - UE/TRP may not be able to ensure that timing errors are within the same margin

  - Option 2: Discuss and conclude whether UE Rx and UE Tx timing error can be grouped based on antenna panel, RF chain, frequency, baseband sampling rate, SRS antenna switching, etc. (vivo)

- Recommended WF

  - Discuss topic and options

Issue 2-2-3: Time variant behaviour of TEG

- Proposals

  - Option 1: (Nokia)

    - RAN4 to study if TEG appears static, semi-static or dynamic in TX/RX scenarios considering various front-end parameters and conditions

    - Feasibility study on absolute Tx/Rx timing error estimation in DL-TDOA if TEG changes measurably statically or semi-statically.

  - Option 2: TBA

- Recommended WF

  - Discuss

Issue 2-2-4: Applicability of TEG with gNB/TRP

- Proposals

  - Option 1: Support TRP to provide association information of DL PRS resources with Tx and Rx TEG to LMF if TRP has multiple TEG (i.e. different antenna panels or arrays) (CATT)

  - Option 2: TRP comprising multiple TEG is unlikely, therefore multiple Rx and Tx TEG is not supported (Ericsson)

- Recommended WF

  - Discuss topic and its options

Issue 2-2-5: Applicability of TEG with UE

- Proposals

  - Option 1: Not applicable (CATT)

- Recommended WF

  - Discuss

## Companies views’ collection for 1st round

### Open issues

Sub topic 2-1-1: Send reply LS to RAN1

| Company | Comments |
| --- | --- |
| Ericsson | Option 2, based on our view and some others companies’ views. |
| CATT | Support option 2. We think before supporting to report associated information of TEGs, the feasibility on the reported information should be discussed first. And we understand there are some issues on the TEG definition which should be provided to RAN1. |
| vivo | Agree with recommended WF. |
| Qualcomm | Support the recommended WF. There were no direct questions to RAN4 in the LS so an immediate response is not necessary. RAN4 may reply with comments and recommendations once it has studied and discussed the feasibility of timing error mitigation via TEGs. |
| Nokia | Support option2. |
| Intel | Slightly prefer option1. We don’t see at least currently there can be anything meaningful for RAN4 to reply to RAN1. Agree with the recommended WF. |
| Huawei | Option 1, at least for this meeting. Agree with QC that there is direct question or action to RAN4, so it’s unclear to us what RAN4 should reply. Technically, we do not see any feasibility issue in the RAN1 agreements. On the other hand, since this is first meeting, we are open to have further discussions in RAN4, and if we identify any issue or reach any conclusion that has RAN1 impact, we can send LS to inform RAN1. |
| OPPO | Support the recommended WF. |

Sub topic 2-2-1: Calibration error/residual timing error after calibration

| Company | Comments |
| --- | --- |
| Ericsson | We agree with option 1 that in fact the measurement result comprises information about the timing after certain timing calibration has taken place. Therefore, the residual error after calibration can be classified as an unknown timing error per se, for which no distinction can be guaranteed. Only topological knowledge provided could classify the belonging to a certain TEG. See issue 2-2-2 |
| CATT | Support option 1. UE/TRP has no idea on the remaining time delay after calibration. |
| vivo | When defining accuracy requirements for RSTD and UE Rx-Tx time difference measurement, RF calibration margin is added. Therefore, at least the worst case of calibration error should be known to some extent. |
| Qualcomm | We are not aware of a way to completely eliminate calibration error. Agree with Option 1. |
| Nokia | We don’t know yet about option-1. The option proposed by CATT is one study point of RAN4. It needs to clarify if TEG can be measurable or statistically estimated or can be grouped per some RF settings etc. |
| Huawei | We agree with option 1 that the exact timing error is unknown, but we also share same view as vivo that the timing error cannot be unlimited. We do not quite understand the relevance of option 1 to the RAN1 agreements. The TEG framework does not require UE or TRP to be aware of exact timing error. |
| OPPO | We agree with option 1 and share the same view with vivo that the residual timing error after calibration should be limited within a known range, which will serve as the RF margin when defining accuracy requirements. |

Sub topic 2-2-2: TEG grouping

| Company | Comments |
| --- | --- |
| Ericsson | It is unsure if grouping of certain topological and/or signal configurations leads to distinctiveness for which TEG could be formed so we agree with option 2, that RAN4 should analyze how TEGs could be defined based on certain parameters. |
| CATT | Support option 1. We think generally the timing delay within the same RF chain or same antenna panel should be similar and can be grouped within a certain margin. But UE/TRP cannot ensure the margin difference between different groups. So we are also fine to further discuss how UE/TRP can group the timing delay based on certain parameters. |
| Vivo | Since RAN1 agrees TEG approach is used to improve accuracy, feasibility should be studied in RAN4. For option 2, we would like to revise a little bit to address difference for UE Rx timing error and UE Tx timing error. At least SRS antenna switching will have impact on UE Tx timing error only. Option 2a Discuss and conclude whether UE Rx timing error can be grouped based on antenna panel, RF chain, frequency, baseband sampling rate etc. Discuss and conclude whether UE Tx timing error can be grouped based on antenna panel, RF chain, frequency, baseband sampling rate, SRS antenna switching, etc. |
| Qualcomm | We understand that RAN4 needs to define error margins associated with different TEGs. It would be beneficial to discuss scenarios were TEGs may be applied to determine achievable error margins. Note that the error margins apply to measurements associated with one TEG. Our understanding is that there is no expectation of maintaining an error margin across two different TEGs. |
| Nokia | We support to study the issues addressed in option-2. |
| Huawei | The definition of TEG is given in the RAN1 LS, and we understand, as long as the relative timing error between the measurements or transmissions with the same TEG is within a margin, how UE/TRP associates different measurements or transmissions to different TEGs should be up to implementation. In this sense, we do not support option 2. Option 1 is fine but again we do not see how it is relevant to the RAN1 LS. |
| OPPO | Support option 2 to firstly study the feasibility of TEG grouping. |

Sub topic 2-2-3: Time variant behaviour of TEG

| Company | Comments |
| --- | --- |
| Ericsson | Agree with option 1, analysis is needed to ensure that if TEG reporting and further analysis based on TEG would take place, how time variant behavior would impact such reporting and following analysis by network instances. |
| CATT | We are fine to further study the timing delay estimation and time variant of TEGs. |
| vivo | Study on this aspect may be needed. However, we are not expecting dynamic TEG grouping. |
| Qualcomm | We agree that time varying aspects of timing errors should be considered when studying the feasibility of TEGs. |
| Nokia | Support option 1. Time delay we aim to study is the residual timing error after RF calibration. Therefore, it may have dependency on RF noise or RF impairment sources. If it is unknown from dynamic sources, it would be hard to estimate it. |
| Huawei | We would like to clarify whether option 1 means to study the time variance of timing error or TEG? If it is for the latter, could proponent explain what is meant by “TEG change”? It is noted that TEG is associated with a measurement or a transmission. |
| OPPO | Agree with option 1. |

Sub topic 2-2-4: Applicability of TEG with gNB/TRP

| Company | Comments |
| --- | --- |
| Ericsson | Prefer option 2. As stated in our contribution, it is unlikely that a gNB will comprise otherwise undistinguishable antenna panels / TRX chains with large differences in residual timing error after calibration, since other features demand high phase accuracy, e.g. beamforming. |
| CATT | We think this is related to issue 2-2-2. We agree that TRP cannot group the residual timing delay after calibration. But as analyzed in our paper, through grouping the uncalibrated delay, TRP can have the information that the resources are transmitted in the same antenna panel and the more accurate location of antenna can be used to improve accuracy. Anyway we are fine to further study the grouping of timing errors. |
| Qualcomm | Needs to be studied further. |
| Nokia | It is early to conclude between the two options. |
| Huawei | On option 1, we understand TRP only transmits PRS, so it can only provide Tx TEG association info to the LMF, but it cannot provide Rx TEG info. On option 2, we understand if a TRP has multiple Tx or Rx antennas, then it is possible to have multiple TEGs. Option 2 is too restrictive for NW implementation. |

Sub topic 2-2-5: Applicability of TEG with UE

| Company | Comments |
| --- | --- |
| Ericsson | Based on provided reasoning in R4-2109103, we can agree with option 1. |
| CATT | Support option 1. We think the difference between different antennas for UE is small and no need to distinguish different groups. |
| Vivo | The issue is not clear to us. What is the relationship to Issue 2-2-2 for feasibility of TEG grouping? |
| Ericsson | After further internal discussion we like to revise our previous comment. We do not agree with option 1 at this moment and would like to address and discuss issue 2-2-2 first. |
| Qualcomm | This needs to be studied further. |
| Nokia | A baseline assumption of RAN1 is that it is applicable to both gNB and UE, accordingly RAN4 starts with the same assumption. |
| Huawei | Same as comment on option 2 for Issue 2-2-4, we cannot agree to option 1 because it is too restrictive for UE implementation. |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Sub topic 2-1-1: Send reply LS to RAN1 Tentative agreements: Discuss TEG feature feasibility to reach conclusion if reply LS is needed and what to include Candidate options: none Recommendations for 2nd round: Further discuss TEG feasibility |
| Sub-topic#1 | Sub topic 2-2-1: Calibration error/residual timing error after calibration Tentative agreements: none Candidate options: none Recommendations for 2nd round: Further discuss |
| Sub-topic#1 | Sub topic 2-2-2: TEG grouping Tentative agreements: Discuss and conclude whether UE Rx and UE Tx timing error can be grouped based on antenna panel, RF chain, frequency, baseband sampling rate, SRS antenna switching, etc. Candidate options: none Recommendations for 2nd round: Discuss topic according to TEG definition by RAN1 |
| Sub-topic#1 | Sub topic 2-2-3: Time variant behaviour of TEG Tentative agreements: FFS: RAN4 to study if TEG appears static, semi-static or dynamic in TX/RX scenarios considering various front-end parameters and conditions FFS: Feasibility study on absolute Tx/Rx timing error estimation in DL-TDOA if TEG changes measurably statically or semi-statically. Candidate options: none Recommendations for 2nd round: Discuss formulation of proposed study topic, e.g. study time variance of timing error compared to time variance of TEG, according to TEG definition by RAN1 |
| Sub-topic#1 | Sub topic 2-2-4: Applicability of TEG with gNB/TRP Tentative agreements: Needs further study Candidate options: none Recommendations for 2nd round: Discuss |
| Sub-topic#1 | Sub topic 2-2-5: Applicability of TEG with UE Tentative agreements: Needs further study Candidate options: none Recommendations for 2nd round: Discuss |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

### Sub-topic 2-1 LS discussion

Sub topic 2-1-1: Send reply LS to RAN1

Tentative agreements:

Discuss TEG feature feasibility to reach conclusion if reply LS is needed and what to include

Candidate options:

none

Recommendations for 2nd round:

Further discuss TEG feasibility

| Company | Comments |
| --- | --- |
| Ericsson | From RAN1 agreements and based on the LS to RAN4, after further internal discussions on the topic, it should be understood that TEG association is based on a per antenna panel baseline. This means that the TEG association grants information based on the differences of timing errors related to two measurements and/or transmissions. This means if τ1 and τ2 are the timing errors related to two measurements/transmissions that are associated with the same TEG then ¦ τ1 - τ2 ¦ < δ. If such information of association is made known to the LMF, it can for example form the difference of measurements conducted with the same TEG which guarantees that the timing error of said TRX chain/antenna panel that belongs to that TEG does not change between two measurements. Therefore, when the difference of the measurements conducted with the same TEG is performed, the timing error difference gets cancelled (within margin δ). This can be visualized in the following example for RSTD measurements: In above example RSTD measurements for d1 and d3 have receive timing error that cannot be cancelled if the antenna panel used is unknown. If TEG grouping for Panel a is made possible and information by association is granted, the LMF can form RSTD3-RSTD1, where the receive timing errors will cancel. RSTD1=d1-drefc+τRX,a-τRX,b RSTD2=d2-drefc RSTD3=d3-drefc+τRX,a-τRX,b This will lead to RSTD3-RSTD1=d3-d1c In reality timing errors will not cancel completely but have some residual timing error which is however smaller than the margin delta. However, the raised questions in the first round regarding TEG association did incorporate some different views concerning the understanding of TEG. As we understand it, the reasoning behind TEG is not to group multiple antenna panels and TRX chains with similar timing error before or after group delay calibration, rather to be able to provide association information without limiting implementation, to ensure that the timing error difference between measurements/transmissions associated with the same TAG are within a margin delta. Therefore, it needs to be discussed if there are any feasibility issues with TEG reporting not primarily with grouping multiple antenna panels together but rather other topics that are still valid for the case that a TEG normally only addresses one antenna panel / TRX chain, such as time variant behavior of the timing error. Further discussion is in 2-2-3. In conclusion, we think a reply LS is needed at some stage but RAN4 firstly has to align. |
| Huawei | We do not see any feasibility issue, but we are open to further discussions. We do not think RAN4 has any consensus in this meeting based on which an LS can be sent. |
| Nokia | RAN4 concludes first TEG estimation/measurement feasibility in RAN4, and then LS can be replied. It is fine to hold LS reply until the next meeting. |

### Sub-topic 2-2 Discussion on TEG and feasibility

Sub topic 2-2-1: Calibration error/residual timing error after calibration

Tentative agreements: none

Candidate options: none

Recommendations for 2nd round:  Further discuss

| Company | Comments |
| --- | --- |
| Ericsson | As per discussion in 2-1-1, we don’t think that the residual calibration error matters, under the assumption that the error would be time-invariant. If TEG association shall point out that that the timing errors which affect the measured timing accuracy has not changed in between conducting measurements/transmission, it is not necessary to gain knowledge of the absolute value of the residual timing error. Whether or not the time-invariance holds true needs to be discussed (issue 2-2-3). |
| Huawei | We do not quite understand the relevance of the discussion to the feasibility of TEG mechanism. The TEG framework does not require UE or TRP to be aware of exact timing error. |
| Nokia | RAN1 already defines TE is residual error after calibration in agreements. RAN4 starts with the same assumption with RAN1. Further properties will be studied through other sub-topics. |

Sub topic 2-2-2: TEG grouping

Tentative agreements:

Discuss and conclude whether UE Rx and UE Tx timing error can be grouped based on antenna panel, RF chain, frequency, baseband sampling rate, SRS antenna switching, etc.

Candidate options:

none

Recommendations for 2nd round:

Discuss topic according to TEG definition by RAN1

| Company | Comments |
| --- | --- |
| Ericsson | To our understanding no grouping according to UE Rx and UE Tx timing errors will take place, rather defining association information for which transmission/reception (chain) the timing error difference between transmissions/receptions are within a margin delta. |
| Huawei | How UE/TRP associates different measurements or transmissions to different TEGs should be up to implementation. We are not sure if RAN4 needs to discuss this. |
| Nokia | First, RAN4 needs conclude if TEG can be estimated or measured, and then further TEG grouping can be discussed logically as a second step. |

Sub topic 2-2-3: Time variant behaviour of TEG

Tentative agreements:

FFS: RAN4 to study if TEG appears static, semi-static or dynamic in TX/RX scenarios considering various front-end parameters and conditions

FFS: Feasibility study on absolute Tx/Rx timing error estimation in DL-TDOA if TEG changes measurably statically or semi-statically.

Candidate options:

none

Recommendations for 2nd round:

Discuss formulation of proposed study topic, e.g. study time variance of timing error compared to time variance of TEG, according to TEG definition by RAN1

| Company | Comments |
| --- | --- |
| Ericsson | One question arises is, that since TEG association is based on information about a single antenna panel and verifying that in between measurements the timing error difference is still smaller than a certain threshold, then what would time variant behavior mean? Would time variant behavior be relatively small, such that ¦ τ1 - τ2 ¦ < δ still holds true and therefore, no “change” in TEG association is needed. If it would not be relatively small, one could argue that the calibration itself was unmeaningful in the first place, however this discussion is reliant on how big or small the threshold δ shall be. Therefore, considering δ, we are fine to study if residual timing error differences after calibration are static, semi-static or dynamic and what implications with regards to TEG there would be assuming certain threshold δ to apply association with certain TEG. |
| Huawei | This is a valid issue and we are fine to further discuss. |
| Nokia | We support this study. |

Sub topic 2-2-4: Applicability of TEG with gNB/TRP

Tentative agreements: Needs further study

Candidate options: none

Recommendations for 2nd round: Discuss

| Company | Comments |
| --- | --- |
| Ericsson | In our view, most TRPs which would even fall under consideration to be made distinguishable via TEG association would not comprise more than a single TEG, therefore reporting such identification aid is unnecessary. This follows the assumption that gNB/TRP compared to UEs generally speaking are rather fixed base, known location, constant sector hull/envelope types of deployments which do not have multiple antenna panels with opposing orientation. In the case of DL PRS, furthermore the gNB/TRP would only be eligible to provide association information for the TX TEG, which is furthermore unnecessary, since a TRP even if equipped with multiple panels can be configured as multiple TRPs with different DL PRS and therefore are distinguishable anyway. To answer 1st round comments on using TEG for gNBs to differentiate antenna panels with certain distance by using the uncalibrated group delay, our understanding is that compensation for difference in position of different antennas has been discussed in RAN1 but has been excluded (R1-2007172). Since there was not much traction on Rx diversity issue in RAN1 during SI stage, this was not included in the WID scope for Rel-17. |
| Huawei | We understand if a TRP has multiple Tx or Rx antennas, then it is possible to have multiple TEGs |
| Nokia | We are ok with the tentative agreement. |

Sub topic 2-2-5: Applicability of TEG with UE

Tentative agreements: Needs further study

Candidate options: none

Recommendations for 2nd round: Discuss

| Company | Comments |
| --- | --- |
| Ericsson | Following the comments made in previous issues, we think that TEG association is applicable for UEs. It is noted that TEG association is based on identification of more than one measurement which was conducted while the difference of residual timing errors of all measurements with identified TEG are smaller than some threshold value. In the case of RSTD measurements evaluation by LMF, TEG association information can be used to tackle the orientation problem and furthermore allow for more positioning accuracy by combining multiple RSTD measurements in such a way that common timing errors cancel, at least to some extent, namely the threshold for timing error differences between multiple measurements which is used to define TEG. |
| Huawei | We understand if a UE has multiple Tx or Rx antennas, then it is possible to have multiple TEGs |
| Nokia | We are ok with the tentative agreement. |

# Topic #3: Work plan for RRM core requirements

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2110232 | Ericsson | Work plan for RRM core requirements |

## Open issues summary

### Sub-topic 2-1 RRM core requirements work plan

Open issues and candidate options before e-meeting:

Issue 3-1-1: Work plan can be approved?

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - Discuss issues with work plan if any and agree on final work plan.

## Companies views’ collection for 1st round

### Open issues

Sub topic Issue 3-1-1: Work plan approval

| Company | Comments |
| --- | --- |
| Ericsson | Option 1 |
| CATT | Option 1. Fine with the work plan. |
| vivo | In general, the framework of work plan is fine. For the following bullet, we think it should be removed from work plan. It is up to progress and should be allowed even if it is not included in the work plan. - Send LS to RAN2 if need for any new signaling is identified. In addition, the completion date for performance requirements should be at least 6 months after the completion of core part. |
| Qualcomm | We’d like clarification on the timeline. The WP says “Finalization positioning measurement accuracy requirements” by RAN4#102 in Feb 2022. Our understanding is that the performance part extends through August 2022. Also, agreeing on simulation assumptions in RAN4#100 does not seem realistic nor necessary given our understanding of the WI timeline. We would not expect to finalize simulation assumptions in the second meeting of the WI. |
| Nokia | We prefer to take the work plan for reference. It includes sending LS at a certain stage or specific items. |
| Intel | Option 1. |
| Huawei | We suggest to remove descriptions related to “measurement accuracy improvement for Rel-16 positioning measurements”, as we should change Rel-16 requirements for this Rel-17 WI. Even we are going to improve accuracy requirements in Rel-17 this should be Perf part. If the core part completion is targeted at March 2022, then RAN4#102 should be for core part finalization. The current WP looks like the core part already completes at RAN4#101bis. |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Sub topic Issue 3-1-1: Work plan approval Tentative agreements: Work plan will be revised, to incorporate companies’ comments. Candidate options: none Recommendations for 2nd round: Discuss and agree on revised workplan |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |

## Discussion on 2nd round (if applicable)

Sub topic Issue 3-1-1: Work plan approval

Tentative agreements:

Work plan will be revised, to incorporate companies’ comments.

Recommendations for 2nd round: Discuss and agree on revised workplan

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on Rel-17 positioning enhancements | Ericsson | WF document to capture agreements from thread |
| Reply LS on gNB/UE Rx/Tx timing error mitigation | CATT | To: RAN_1; In regard to LS in R4-2107610. Only applicable if issue 2-1-1 is resolved and sending reply LS is agreed. |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2110232 | Work plan for RRM core requirements | Ericsson | Revised | Add revision to allow proposed changes by companies addressed in Topic #3 |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2108360 | WF on Rel-17 positioning enhancements | Ericsson | Agreeable |  |
| R4-2108361 | Reply LS on gNB/UE Rx/Tx timing error mitigation | CATT | Postponed |  |
| R4-2108362 | Work plan for RRM core requirements | Ericsson | Agreeable | Revision of R4-2110232 |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents