3GPP TSG-RAN WG4 Meeting # 99-e 													R4-2108426

Electronic Meeting, 19th – 27th May 2021

Agenda item:			4.1.4, 5.1.7.1, 4.2.1, 5.2.2.1

Source:	Moderator (Ericsson)

Title:	Email discussion summary for [99-e][301] BSRF_Maintenance

Document for:	Information

# Introduction

The e-mail discussion covers Rel-15 and Rel-16 BS RF maintenance for NR, LTE and MS BS, i.e. Agenda items 4.1.4, 5.1.7.1, 4.2.1, 5.2.2.1.

All contributions submitted are CRs, and divided into the following Topics:

- Regional requirements in Japan

- ETC2 correction

- In-band blocking for multi-band Base Stations

- Other maintenance

# Topic #1: 1.	Regional requirements in Japan

In RAN4#98, CRs to be aligned with BWA system in Japan were agreed for 36 and 38 series. This is to update the 37 series for the corresponding changes.

## Companies’ contributions summary

(Cat A CRs are not listed)

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| 37.104: R4-2109898 (Rel-15) R4-2109899 (Rel-16) 37.141: R4-2109904 (Rel-15) R4-2109905 (Rel-16) | NEC, SoftBank Corp., KDDI Corporation, Nokia | CR to 37.104 & 37.141: Regional requirements for band 41, n41, and n90 in Japan, (Rel-15 and Rel-16) Summary of change: Updated the regional requirements list. Added regional requirements for rated output power for Band n41 and n90. Added text to indicate emission limits may be applied to the sum of the emission power over all antenna connectors for Band n41 and n90. Updated additional BS spurious emissions limits for Band 41, n41, and n90. Deleted the additional OBUE limits for Band 41 and n90. Updated the intermodulation requirements for Band 41, n41, and n90. |
| 37.105: R4-2109901 (Rel-15) 37.145-1: R4-2109907 (Rel-15) 37.145-2 R4-2109910 (Rel-15) |  | CR to 37.105, 37.145-1 & 37.145-2: Regional requirements for band 41 in Japan, Rel-15 Summary of change: Updated the regional requirements list. Deleted additional requirements related to Band 41 in Japan. |

## Companies views’ collection for 1st round

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| 37.104: R4-2109898 (Rel-15) R4-2109899 (Rel-16) 37.141: R4-2109904 (Rel-15) R4-2109905 (Rel-16) | Ericsson: The MSR spec CRs use band designation n41 (and n90 in Rel-16) for the bands in question. These are NR specific designations which should not be used in the MSR specifications, where the bands are designated as Band 41 and 90. If a requirement is specific to NR, it could e.g. be stated as “for NR operation in Band 41”. |
|  | NEC: Agree the Ericsson comments on band numbers. Need updates accordingly. |
| 37.105: R4-2109901 (Rel-15) 37.145-1: R4-2109907 (Rel-15) 37.145-2: R4-2109910 (Rel-15) | Ericsson: It is noted that the CRs for AAS specs removes the regional requirements for Band 41, with the following reason for change: “Additional requirements for Band 41 are not aligned with the regional requirements in Japan”. Some more explanation and background would however be very helpful. Is the intention that AAS BS specs will not be used in Japan for these bands, since the regional requirements are removed? |
|  | NEC: AAS requirements for LTE are not specified in BWA specifications (band 41) in Japan. It is the background of these CRs. |

## Summary for 1st round

### CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2109898 R4-2109899 R4-2109904 R4-2109905 | To be revised based on discussion. |
| R4-2109901 R4-2109907 R4-2109910 R4-2109902 R4-2109903 R4-2109908 R4-2109909 R4-2109911 R4-2109912 | The CRs are agreeable. The corresponding Cat A CRs are agreeable. |

## Discussion on 2nd round (if applicable)

# Topic #2: 2.	ETC2 correction

The CR concerns ETC2 that is used to test contiguous CA occupied bandwidth only.

## Companies’ contributions summary

(Cat A CRs are not listed)

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2109825 | Nokia, Nokia Shanghai Bell | CR to TS 36.141: ETC2 correction Summary of change: Instead of testing all carrier bandwidth combinations with different sum of channel bandwidth, only smallest and largest sum of channel bandwidth is tested. |

## Companies views’ collection for 1st round

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2109825 | Huawei: agree in principle, but it may be good to clarify on the CC number, e.g. by adding text “irrespective of the number of component carriers”. Related CRs in [302] to be aligned. |

## Summary for 1st round

### CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2109825 | To be revised. Updates to be aligned with related CR in [302]. |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

# Topic #3: 3.	In-band blocking for multi-band Base Stations

The CRs concern in-band blocking for multi-band BS (LTE, MSR, AAS and NR base stations), where some additions and corrections are proposed..

## Companies’ contributions summary

(Cat A CRs are not listed)

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2111154 R4-2111113 R4-2111116 | Ericsson | CRs to 38.104, 38.141-1 & 38.141-2: In-band blocking for multi-band Base Stations Summary of change: A note is introduced for multi-band operation in in-band-blocking, similar to the note used in 37 series specs for MSR BS. |
| R4-2111119 R4-2111122 R4-2111125 R4-2111173 R4-2111131 R4-2111134 R4-2111137 | Ericsson | CRs to 36.104, 36.141, 37.104, 37.141, 37.105, 37.145-1 & 37.145-2: In-band blocking for multi-band Base Stations Summary of change: The NOTE for multi-band operation is updated to clarify that “adjacent and overlapping” refers to the in-band blocking frequency ranges of the bands in question. |

## Companies views’ collection for 1st round

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2111113 | Nokia: Wording would be more clear as: 'or in the in-band blocking frequency range of an adjacent or overlapping band'. |
|  | NEC: Proposed change specifies the requirements for BS capable of multi-band operation. However, requirements for single-band BS will be not clear. |
|  | Huawei: it looks that the wording of the Note is aligned with the legacy specifications. Do we want to introduce mis-alignment as per Nokia comment? |
| R4-2111119 | Nokia: Wording would be more clear as: 'or in the in-band blocking frequency range of an adjacent or overlapping band'. |
|  | NEC: Identical text is preferable for the corresponding notes in specifications. |
|  | Huawei: same as for R4-2111113. |

## Summary for 1st round

### CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2111154 R4-2111113 R4-2111116 | Return to. Discussion regarding how to word the revised notes to be continued in 2nd round. |
| R4-2111119 R4-2111122 R4-2111125 R4-2111173 R4-2111131 R4-2111134 R4-2111137 | Return to. Discussion regarding how to word the revised notes to be continued in 2nd round. |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

# Topic #4: 4.	Other maintenance

CRs on other diverse topics are covered under “Other maintenance”.:

- Correction of NR bands for MSR BS

- Correction of the channel raster of n259 for TS 38.104

- CR of updating the subPRB BS aspect

## Companies’ contributions summary

(Cat A CRs are not listed)

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2111140 R4-2111143 | Ericsson | CRs to 37.104 and 37.141: Correction of NR bands for MSR BS Summary of change: Three new notes are introduced (Notes 11, 12 and 13) that can be used as replacements for Notes 2, 3 and 4, in case of bands where NR is not supported. |
| R4-2109029 | CATT | CR to 38.104: Correction of the channel raster of n259 for TS 38.104 Summary of change: Correct the NREF for n259 120 kHz SCS in Table 5.4.2.3-2 |
| R4-2111202 | Ericsson | CR to 36.141: CR of updating the subPRB BS aspect Summary of change: Add the declaration of BS for subPRB support for LTE-M and related test specificaton updates. |

## Companies views’ collection for 1st round

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2111140 | Nokia: propose to introduce new column with E-UTRA/NB-IoT band rather than complicate table further with additional notes; also some bands like 18 are added to later releases, so band list isn Cat A CRs should be adjusted accordingly. |
|  | Company B |
| R4-2111202 | Nokia: cover page has proposed change affects 'ME'; the 'BL/CE' should be added to Abbreviations clause 3.3; duplicate ',' and 'for' in added text in clause 7.2.4.2 |
|  | Company B |
| R4-2109029 | Ericsson: The correction from 2270832 2270833 for n259 is correct. There are two issues however: - A CR would also be needed for TS 38.101-2, which has the same table. - There seems to two more errors in the table that needs correction. The band n258 range should start at 2016666 (for 60 kHz raster) and the n259 range (for 60 kHz raster) should also start at 2270833. Operating Band; ΔFRaster (kHz); Uplink and Downlink Range of NREF (First – <Step size> – Last). n257; 60; 2054166 – <1> – 2104165. 120; 2054167 – <2> – 2104165. n258; 60; 2016667 – <1> – 2070832. 120; 2016667 – <2> – 2070831. n259; 60; 2270832 – <1> – 2337499. 120; 2270832– <2> – 2337499. n260; 60; 2229166 – <1> – 2279165. 120; 2229167 – <2> – 2279165. n261; 60; 2070833 – <1> – 2084999. 120; 2070833 – <2> – 2084999. |
|  | CATT: Thanks Ericsson for the comments. For the first comment, we submitted CRs for TS 38.101-2 in this meeting. The CR is R4-2109027 and R4-2109028, until now, there’re no comments received in thread [103]. For the second comment, I response as below. The Range of NREF for FR2 is 2016667 – 3279165 according to Table 5.4.2.1-1. So n258 starts from 2016667. And also according to the FREF = FREF-Offs + ΔFGlobal (NREF – NREF-Offs), n258 starts from 2016667 because FREF-Offs is 24250.08 and NREF-Offs is 2016667. For n259, we agree with the comment, 2270832 is in 39499.98 MHz which is not in the range of n259. We can revise the CR if we can align the understanding. And also I’ll ask the revision for UE CR. |

## Summary for 1st round

### CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2109029 | To be revised. Continue discussion of the NR-ARFCN range for bands n258 and n259. |
| R4-2111140 R4-2111143 R4-2111141 R4-2111142 R4-2111144 R4-2111145 | To be revised. Continue discussion on how to correct the NR applicability and the proposal to add a new column. Change from Cat A to Cat F CRs, since changes will apply to new bands. |
| R4-2111202 | To be revised, based on discussions. |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2109898 | CR to TS 37.104: Regional requirements for band 41 and n41 in Japan, Rel-15 | NEC, SoftBank, KDDI, Nokia | Revised |  |
| R4-2109899 | CR to TS 37.104: Regional requirements for band 41, n41, and n90 in Japan, Rel-16 | NEC, SoftBank, KDDI, Nokia | Revised |  |
| R4-2109904 | CR to TS 37.141: Regional requirements for band 41 and n41 in Japan, Rel-15 | NEC, SoftBank, KDDI, Nokia | Revised |  |
| R4-2109905 | CR to TS 37.141: Regional requirements for band 41, n41, and n90 in Japan, Rel-16 | NEC, SoftBank, KDDI, Nokia | Revised |  |
| R4-2109901 | CR to TS 37.105: Regional requirements for band 41 in Japan, Rel-15 | NEC, SoftBank, KDDI, Nokia | Agreeable |  |
| R4-2109902 | CR to TS 37.105: Regional requirements for band 41 in Japan, Rel-16 | NEC, SoftBank, KDDI, Nokia | Agreeable |  |
| R4-2109903 | CR to TS 37.105: Regional requirements for band 41 in Japan, Rel-17 | NEC, SoftBank, KDDI, Nokia | Agreeable |  |
| R4-2109907 | CR to TS 37.145-1: Regional requirements for band 41 in Japan, Rel-15 | NEC, SoftBank, KDDI, Nokia | Agreeable |  |
| R4-2109908 | CR to TS 37.145-1: Regional requirements for band 41 in Japan, Rel-16 | NEC, SoftBank, KDDI, Nokia | Agreeable |  |
| R4-2109909 | CR to TS 37.145-1: Regional requirements for band 41 in Japan, Rel-17 | NEC, SoftBank, KDDI, Nokia | Agreeable |  |
| R4-2109910 | CR to TS 37.145-2: Regional requirements for band 41 in Japan, Rel-15 | NEC, SoftBank, KDDI, Nokia | Agreeable |  |
| R4-2109911 | CR to TS 37.145-2: Regional requirements for band 41 in Japan, Rel-16 | NEC, SoftBank, KDDI, Nokia | Agreeable |  |
| R4-2109912 | CR to TS 37.145-2: Regional requirements for band 41 in Japan, Rel-17 | NEC, SoftBank, KDDI, Nokia | Agreeable |  |
| R4-2109825 | CR to TS 36.141: ETC2 correction | Nokia, Nokia Shanghai Bell | Revised |  |
| R4-2109029 | Correction of the channel raster of n259 for TS 38.104 | CATT | Revised |  |
| R4-2111140 | CR to 37.104: Correction of NR bands for MSR BS | Ericsson | Revised |  |
| R4-2111141 | CR to 37.104: Correction of NR bands for MSR BS | Ericsson | Change from Cat A to Cat F |  |
| R4-2111142 | CR to 37.104: Correction of NR bands for MSR BS | Ericsson | Change from Cat A to Cat F |  |
| R4-2111143 | CR to 37.141: Correction of NR bands for MSR BS | Ericsson | Revised |  |
| R4-2111144 | CR to 37.141: Correction of NR bands for MSR BS | Ericsson | Change from Cat A to Cat F |  |
| R4-2111145 | CR to 37.141: Correction of NR bands for MSR BS | Ericsson | Change from Cat A to Cat F |  |
| R4-2111202 | CR of updating the subPRB BS aspect | Ericsson | Revised |  |

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-210xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-210xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents