3GPP TSG-RAN4 Meeting #99-e	R4-2108595

# Electronic meeting, 19th – 27th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.174 | CR |  | rev |  | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.174: Introduction of IAB-DU performance requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Perf |  |  |  |  |  | Date: |  |  | 2021-05-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The IAB spec currently does not contain IAB-DU performance requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | IAB-DU performance requirements are introduced according to agreements. The requirements are adapted from the BS specification 38.104. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No IAB-DU performance requirements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 8	Conducted performance requirements

## 8.1	IAB-DU performance requirements

### 8.1.1	General

Conducted performance requirements specify the ability of the IAB type 1-H to correctly demodulate signals in various conditions and configurations. Conducted performance requirements are specified at the TAB connector(s).

Conducted performance requirements for the IAB-DU are specified for the fixed reference channels defined in annex A and the propagation conditions in annex G. The requirements only apply to those FRCs that are supported by the IAB-DU.

Unless stated otherwise, performance requirements apply for a single carrier only. Performance requirements for an IAB-DU supporting carrier aggregation are defined in terms of single carrier requirements.

The SNR used in this clause is specified based on a single carrier and defined as:

SNR = S / N

Where:

S	is the total signal energy in the slot on a single TAB connector.

N	is the noise energy in a bandwidth corresponding to the transmission bandwidth over the duration of a slot on a single TAB connector.

### 8.1.2	Performance requirements for PUSCH

#### 8.1.2.1	Requirements for PUSCH with transform precoding disabled

##### 8.1.2.1.1	General

The performance requirement of PUSCH is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in annex A. The performance requirements assume HARQ retransmissions.

Table: 8.1.2.1.1-1 Test parameters for testing PUSCH

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| Cyclic prefix |  | Normal |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0}, {0, 1} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain | PUSCH mapping type | A, B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
| assignment | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL pattern. |  |  |

##### 8.1.2.1.	Minimum requirements

The throughput shall be equal to or larger than 70% of the maximum throughput for the FRCs stated in tables 8.1.2.1.2-1 to 8.1.2.1.2-14 at the given SNR for 1Tx or for 2Tx two-layer spatial multiplexing transmission. FRCs are defined in annex A.

Table 8.1.2.1.2-1: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 5 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of Demodulation Branches | Propagation conditions and correlation matrix (Annex G) | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- |
|  |  | TDLB100-400 Low | D-FR1-A.2.1-1 | pos1 | -2.3 |
|  | 2 | TDLC300-100 Low | D-FR1-A.2.3-1 | pos1 | 10.1 |
|  |  | TDLA30-10 Low | D-FR1-A.2.4-1 | pos1 | 12.3 |
|  |  | TDLB100-400 Low | D-FR1-A.2.1-1 | pos1 | -5.8 |
| 1 | 4 | TDLC300-100 Low | D-FR1-A.2.3-1 | pos1 | 6.2 |
|  |  | TDLA30-10 Low | D-FR1-A.2.4-1 | pos1 | 8.8 |
|  |  | TDLB100-400 Low | D-FR1-A.2.1-1 | pos1 | -8.7 |
|  | 8 | TDLC300-100 Low | D-FR1-A.2.3-1 | pos1 | 3.0 |
|  |  | TDLA30-10 Low | D-FR1-A.2.4-1 | pos1 | 5.6 |
|  | 2 | TDLB100-400 Low | D-FR1-A.2.1-8 | pos1 | 1.0 |
|  |  | TDLC300-100 Low | D-FR1-A.2.3-8 | pos1 | 18.2 |
| 2 | 4 | TDLB100-400 Low | D-FR1-A.2.1-8 | pos1 | -2.3 |
|  |  | TDLC300-100 Low | D-FR1-A.2.3-8 | pos1 | 11.0 |
|  | 8 | TDLB100-400 Low | D-FR1-A.2.1-8 | pos1 | -5.3 |
|  |  | TDLC300-100 Low | D-FR1-A.2.3-8 | pos1 | 6.8 |

Table 8.1.2.1.2-2: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 10 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of Demodulation Branches | Propagation conditions and correlation matrix (Annex G) | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- |
|  |  | TDLB100-400 Low | D-FR1-A.2.1-2 | pos1 | -2.5 |
|  | 2 | TDLC300-100 Low | D-FR1-A.2.3-2 | pos1 | 10.2 |
|  |  | TDLA30-10 Low | D-FR1-A.2.4-2 | pos1 | 12.2 |
|  |  | TDLB100-400 Low | D-FR1-A.2.1-2 | pos1 | -6.0 |
| 1 | 4 | TDLC300-100 Low | D-FR1-A.2.3-2 | pos1 | 6.3 |
|  |  | TDLA30-10 Low | D-FR1-A.2.4-2 | pos1 | 8.6 |
|  |  | TDLB100-400 Low | D-FR1-A.2.1-2 | pos1 | -8.7 |
|  | 8 | TDLC300-100 Low | D-FR1-A.2.3-2 | pos1 | 3.1 |
|  |  | TDLA30-10 Low | D-FR1-A.2.4-2 | pos1 | 5.5 |
|  | 2 | TDLB100-400 Low | D-FR1-A.2.1-9 | pos1 | 1.7 |
|  |  | TDLC300-100 Low | D-FR1-A.2.3-9 | pos1 | 18.3 |
| 2 | 4 | TDLB100-400 Low | D-FR1-A.2.1-9 | pos1 | -2.0 |
|  |  | TDLC300-100 Low | D-FR1-A.2.3-9 | pos1 | 11.2 |
|  | 8 | TDLB100-400 Low | D-FR1-A.2.1-9 | pos1 | -5.5 |
|  |  | TDLC300-100 Low | D-FR1-A.2.3-9 | pos1 | 6.8 |

Table 8.1.2.1.2-3: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 20 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of Demodulation Branches | Propagation conditions and correlation matrix (Annex G) | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- |
|  |  | TDLB100-400 Low | D-FR1-A.2.1-3 | pos1 | -2.1 |
|  | 2 | TDLC300-100 Low | D-FR1-A.2.3-3 | pos1 | 10.0 |
|  |  | TDLA30-10 Low | D-FR1-A.2.4-3 | pos1 | 12.4 |
|  |  | TDLB100-400 Low | D-FR1-A.2.1-3 | pos1 | -5.5 |
| 1 | 4 | TDLC300-100 Low | D-FR1-A.2.3-3 | pos1 | 6.2 |
|  |  | TDLA30-10 Low | D-FR1-A.2.4-3 | pos1 | 8.6 |
|  |  | TDLB100-400 Low | D-FR1-A.2.1-3 | pos1 | -8.5 |
|  | 8 | TDLC300-100 Low | D-FR1-A.2.3-3 | pos1 | 3.0 |
|  |  | TDLA30-10 Low | D-FR1-A.2.4-3 | pos1 | 5.5 |
|  | 2 | TDLB100-400 Low | D-FR1-A2.1-10 | pos1 | 2.1 |
|  |  | TDLC300-100 Low | D-FR1-A.2.3-10 | pos1 | 18.3 |
| 2 | 4 | TDLB100-400 Low | D-FR1-A.2.1-10 | pos1 | -1.8 |
|  |  | TDLC300-100 Low | D-FR1-A.2.3-10 | pos1 | 11.1 |
|  | 8 | TDLB100-400 Low | D-FR1-A.2.1-10 | pos1 | -5.3 |
|  |  | TDLC300-100 Low | D-FR1-A.2.3-10 | pos1 | 6.9 |

Table 8.1.2.1.2-4: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 10 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of Demodulation Branches | Propagation conditions and correlation matrix (Annex G) | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- |
|  |  | TDLB100-400 Low | D-FR1-A.2.1-4 | pos1 | -2.3 |
|  | 2 | TDLC300-100 Low | D-FR1-A.2.3-4 | pos1 | 10.2 |
|  |  | TDLA30-10 Low | D-FR1-A.2.4-4 | pos1 | 12.8 |
|  |  | TDLB100-400 Low | D-FR1-A.2.1-4 | pos1 | -5.6 |
| 1 | 4 | TDLC300-100 Low | D-FR1-A.2.3-4 | pos1 | 6.4 |
|  |  | TDLA30-10 Low | D-FR1-A.2.4-4 | pos1 | 8.6 |
|  |  | TDLB100-400 Low | D-FR1-A.2.1-4 | pos1 | -8.6 |
|  | 8 | TDLC300-100 Low | D-FR1-A.2.3-4 | pos1 | 3.3 |
|  |  | TDLA30-10 Low | D-FR1-A.2.4-4 | pos1 | 5.5 |
|  | 2 | TDLB100-400 Low | D-FR1-A.2.1-11 | pos1 | 1.3 |
|  |  | TDLC300-100 Low | D-FR1-A.2.3-11 | pos1 | 18.4 |
| 2 | 4 | TDLB100-400 Low | D-FR1-A.2.1-11 | pos1 | -2.2 |
|  |  | TDLC300-100 Low | D-FR1-A.2.3-11 | pos1 | 11.2 |
|  | 8 | TDLB100-400 Low | D-FR1-A.2.1-11 | pos1 | -5.2 |
|  |  | TDLC300-100 Low | D-FR1-A.2.3-11 | pos1 | 7.0 |

Table 8.1.2.1.2-5: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 20 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of Demodulation Branches | Propagation conditions and correlation matrix (Annex G) | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- |
|  |  | TDLB100-400 Low | D-FR1-A.2.1-5 | pos1 | -2.9 |
|  | 2 | TDLC300-100 Low | D-FR1-A.2.3-5 | pos1 | 10.2 |
|  |  | TDLA30-10 Low | D-FR1-A.2.4-5 | pos1 | 12.5 |
|  |  | TDLB100-400 Low | D-FR1-A.2.1-5 | pos1 | -6.0 |
| 1 | 4 | TDLC300-100 Low | D-FR1-A.2.3-5 | pos1 | 6.4 |
|  |  | TDLA30-10 Low | D-FR1-A.2.4-5 | pos1 | 8.6 |
|  |  | TDLB100-400 Low | D-FR1-A.2.1-5 | pos1 | -8.8 |
|  | 8 | TDLC300-100 Low | D-FR1-A.2.3-5 | pos1 | 3.2 |
|  |  | TDLA30-10 Low | D-FR1-A.2.4-5 | pos1 | 5.5 |
|  | 2 | TDLB100-400 Low | D-FR1-A.2.1-12 | pos1 | 1.3 |
|  |  | TDLC300-100 Low | D-FR1-A.2.3-12 | pos1 | 18.1 |
| 2 | 4 | TDLB100-400 Low | D-FR1-A.2.1-12 | pos1 | -2.2 |
|  |  | TDLC300-100 Low | D-FR1-A.2.3-12 | pos1 | 11.3 |
|  | 8 | TDLB100-400 Low | D-FR1-A.2.1-12 | pos1 | -5.3 |
|  |  | TDLC300-100 Low | D-FR1-A.2.3-12 | pos1 | 6.9 |

Table 8.1.2.1.2-6: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 40 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of Demodulation Branches | Propagation conditions and correlation matrix (Annex G) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- |
|  |  | TDLB100-400 Low | pos1 | -2.5 |
|  | 2 | TDLC300-100 Low | pos1 | 10.0 |
|  |  | TDLA30-10 Low | pos1 | 12.4 |
|  |  | TDLB100-400 Low | pos1 | -5.8 |
| 1 | 4 | TDLC300-100 Low | pos1 | 6.3 |
|  |  | TDLA30-10 Low | pos1 | 8.5 |
|  |  | TDLB100-400 Low | pos1 | -8.7 |
|  | 8 | TDLC300-100 Low | pos1 | 3.1 |
|  |  | TDLA30-10 Low | pos1 | 5.4 |
|  | 2 | TDLB100-400 Low | pos1 | 1.3 |
|  |  | TDLC300-100 Low | pos1 | 19.5 |
| 2 | 4 | TDLB100-400 Low | pos1 | -2.3 |
|  |  | TDLC300-100 Low | pos1 | 11.3 |
|  | 8 | TDLB100-400 Low | pos1 | -5.2 |
|  |  | TDLC300-100 Low | pos1 | 6.9 |

Table 8.1.2.1.2-7: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 100 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of Demodulation Branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Normal | TDLB100-400 Low | D-FR1-A.2.1-7 | pos1 | -2.8 |
|  | 2 | Normal | TDLC300-100 Low | D-FR1-A.2.3-7 | pos1 | 10.2 |
|  |  | Normal | TDLA30-10 Low | D-FR1-A.2.4-7 | pos1 | 13.0 |
|  |  | Normal | TDLB100-400 Low | D-FR1-A.2.1-7 | pos1 | -5.8 |
| 1 | 4 | Normal | TDLC300-100 Low | D-FR1-A.2.3-7 | pos1 | 6.5 |
|  |  | Normal | TDLA30-10 Low | D-FR1-A.2.4-7 | pos1 | 9.0 |
|  |  | Normal | TDLB100-400 Low | D-FR1-A.2.1-7 | pos1 | -8.7 |
|  | 8 | Normal | TDLC300-100 Low | D-FR1-A.2.3-7 | pos1 | 3.2 |
|  |  | Normal | TDLA30-10 Low | D-FR1-A.2.4-7 | pos1 | 5.8 |
|  | 2 | Normal | TDLB100-400 Low | D-FR1-A.2.1-14 | pos1 | 1.4 |
|  |  | Normal | TDLC300-100 Low | D-FR1-A.2.3-14 | pos1 | 19.2 |
| 2 | 4 | Normal | TDLB100-400 Low | D-FR1-A.2.1-14 | pos1 | -2.2 |
|  |  | Normal | TDLC300-100 Low | D-FR1-A.2.3-14 | pos1 | 11.6 |
|  | 8 | Normal | TDLB100-400 Low | D-FR1-A.2.1-14 | pos1 | -5.2 |
|  |  | Normal | TDLC300-100 Low | D-FR1-A.2.3-14 | pos1 | 7.1 |

Table 8.1.2.1.2-8: Minimum requirements for PUSCH with 70% of maximum throughput, Type B, 5 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of Demodulation Branches | Propagation conditions and correlation matrix (Annex G) | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- |
|  |  | TDLB100-400 Low | D-FR1-A.2.1-1 | pos1 | -2.3 |
|  | 2 | TDLC300-100 Low | D-FR1-A.2.3-1 | pos1 | 10.2 |
|  |  | TDLA30-10 Low | D-FR1-A.2.4-1 | pos1 | 12.5 |
|  |  | TDLB100-400 Low | D-FR1-A.2.1-1 | pos1 | -5.7 |
| 1 | 4 | TDLC300-100 Low | D-FR1-A.2.3-1 | pos1 | 6.3 |
|  |  | TDLA30-10 Low | D-FR1-A.2.4-1 | pos1 | 8.9 |
|  |  | TDLB100-400 Low | D-FR1-A.2.1-1 | pos1 | -8.7 |
|  | 8 | TDLC300-100 Low | D-FR1-A.2.3-1 | pos1 | 3.0 |
|  |  | TDLA30-10 Low | D-FR1-A.2.4-1 | pos1 | 5.7 |
|  | 2 | TDLB100-400 Low | D-FR1-A.2.1-8 | pos1 | 1.5 |
|  |  | TDLC300-100 Low | D-FR1-A.2.3-8 | pos1 | 18.3 |
| 2 | 4 | TDLB100-400 Low | D-FR1-A.2.1-8 | pos1 | -2.3 |
|  |  | TDLC300-100 Low | D-FR1-A.2.3-8 | pos1 | 11.1 |
|  | 8 | TDLB100-400 Low | D-FR1-A.2.1-8 | pos1 | -5.4 |
|  |  | TDLC300-100 Low | D-FR1-A.2.3-8 | pos1 | 6.8 |

Table 8.1.2.1.2-9: Minimum requirements for PUSCH with 70% of maximum throughput, Type B, 10 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of Demodulation Branches | Propagation conditions and correlation matrix (Annex G) | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- |
|  |  | TDLB100-400 Low | D-FR1-A.2.1-2 | pos1 | -2.3 |
|  | 2 | TDLC300-100 Low | D-FR1-A.2.3-2 | pos1 | 10.5 |
|  |  | TDLA30-10 Low | D-FR1-A.2.4-2 | pos1 | 12.6 |
|  |  | TDLB100-400 Low | D-FR1-A.2.1-2 | pos1 | -5.7 |
| 1 | 4 | TDLC300-100 Low | D-FR1-A.2.3-2 | pos1 | 6.5 |
|  |  | TDLA30-10 Low | D-FR1-A.2.4-2 | pos1 | 8.9 |
|  |  | TDLB100-400 Low | D-FR1-A.2.1-2 | pos1 | -9.0 |
|  | 8 | TDLC300-100 Low | D-FR1-A.2.3-2 | pos1 | 3.2 |
|  |  | TDLA30-10 Low | D-FR1-A.2.4-2 | pos1 | 5.8 |
|  | 2 | TDLB100-400 Low | D-FR1-A.2.1-9 | pos1 | 2.0 |
|  |  | TDLC300-100 Low | D-FR1-A.2.3-9 | pos1 | 18.7 |
| 2 | 4 | TDLB100-400 Low | D-FR1-A.2.1-9 | pos1 | -2.3 |
|  |  | TDLC300-100 Low | D-FR1-A.2.3-9 | pos1 | 11.3 |
|  | 8 | TDLB100-400 Low | D-FR1-A.2.1-9 | pos1 | -5.2 |
|  |  | TDLC300-100 Low | D-FR1-A.2.3-9 | pos1 | 7.0 |

Table 8.1.2.1.2-10: Minimum requirements for PUSCH with 70% of maximum throughput, Type B, 20 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of Demodulation Branches | Propagation conditions and correlation matrix (Annex G) | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- |
|  |  | TDLB100-400 Low | D-FR1-A.2.1-3 | pos1 | -2.1 |
|  | 2 | TDLC300-100 Low | D-FR1-A.2.3-3 | pos1 | 10.4 |
|  |  | TDLA30-10 Low | D-FR1-A.2.4-3 | pos1 | 12.3 |
|  |  | TDLB100-400 Low | D-FR1-A.2.1-3 | pos1 | -5.7 |
| 1 | 4 | TDLC300-100 Low | D-FR1-A.2.3-3 | pos1 | 6.3 |
|  |  | TDLA30-10 Low | D-FR1-A.2.4-3 | pos1 | 8.8 |
|  |  | TDLB100-400 Low | D-FR1-A.2.1-3 | pos1 | -8.5 |
|  | 8 | TDLC300-100 Low | D-FR1-A.2.3-3 | pos1 | 3.1 |
|  |  | TDLA30-10 Low | D-FR1-A.2.4-3 | pos1 | 5.7 |
|  | 2 | TDLB100-400 Low | D-FR1-A2.1-10 | pos1 | 1.6 |
|  |  | TDLC300-100 Low | D-FR1-A.2.3-10 | pos1 | 18.1 |
| 2 | 4 | TDLB100-400 Low | D-FR1-A.2.1-10 | pos1 | -2.0 |
|  |  | TDLC300-100 Low | D-FR1-A.2.3-10 | pos1 | 11.2 |
|  | 8 | TDLB100-400 Low | D-FR1-A.2.1-10 | pos1 | -5.3 |
|  |  | TDLC300-100 Low | D-FR1-A.2.3-10 | pos1 | 6.9 |

Table 8.1.2.1.2-11: Minimum requirements for PUSCH with 70% of maximum throughput, Type B, 10 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of Demodulation Branches | Propagation conditions and correlation matrix (Annex G) | FRC (Annex A) | Additional DM-RS position |  | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | TDLB100-400 Low | D-FR1-A.2.1-4 |  | pos1 | -2.4 |
|  | 2 | TDLC300-100 Low | D-FR1-A.2.3-4 |  | pos1 | 10.1 |
|  |  | TDLA30-10 Low | D-FR1-A.2.4-4 |  | pos1 | 12.5 |
|  |  | TDLB100-400 Low | D-FR1-A.2.1-4 |  | pos1 | -5.7 |
| 1 | 4 | TDLC300-100 Low | D-FR1-A.2.3-4 |  | pos1 | 6.4 |
|  |  | TDLA30-10 Low | D-FR1-A.2.4-4 |  | pos1 | 8.6 |
|  |  | TDLB100-400 Low | D-FR1-A.2.1-4 |  | pos1 | -8.8 |
|  | 8 | TDLC300-100 Low | D-FR1-A.2.3-4 |  | pos1 | 3.2 |
|  |  | TDLA30-10 Low | D-FR1-A.2.4-4 |  | pos1 | 5.6 |
|  | 2 | TDLB100-400 Low | D-FR1-A.2.1-11 |  | pos1 | 1.1 |
|  |  | TDLC300-100 Low | D-FR1-A.2.3-11 |  | pos1 | 18.5 |
| 2 | 4 | TDLB100-400 Low | D-FR1-A.2.1-11 |  | pos1 | -2.5 |
|  |  | TDLC300-100 Low | D-FR1-A.2.3-11 |  | pos1 | 11.3 |
|  | 8 | TDLB100-400 Low | D-FR1-A.2.1-11 |  | pos1 | -5.6 |
|  |  | TDLC300-100 Low | D-FR1-A.2.3-11 |  | pos1 | 7.0 |

Table 8.1.2.1.2-12: Minimum requirements for PUSCH with 70% of maximum throughput, Type B, 20 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of Demodulation Branches | Propagation conditions and correlation matrix (Annex G) | FRC (Annex A) | Additional DM-RS position |  | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | TDLB100-400 Low | D-FR1-A.2.1-5 |  | pos1 | -2.9 |
|  | 2 | TDLC300-100 Low | D-FR1-A.2.3-5 |  | pos1 | 10.1 |
|  |  | TDLA30-10 Low | D-FR1-A.2.4-5 |  | pos1 | 12.5 |
|  |  | TDLB100-400 Low | D-FR1-A.2.1-5 |  | pos1 | -6.0 |
| 1 | 4 | TDLC300-100 Low | D-FR1-A.2.3-5 |  | pos1 | 6.3 |
|  |  | TDLA30-10 Low | D-FR1-A.2.4-5 |  | pos1 | 8.6 |
|  |  | TDLB100-400 Low | D-FR1-A.2.1-5 |  | pos1 | -9.0 |
|  | 8 | TDLC300-100 Low | D-FR1-A.2.3-5 |  | pos1 | 3.1 |
|  |  | TDLA30-10 Low | D-FR1-A.2.4-5 |  | pos1 | 5.6 |
|  | 2 | TDLB100-400 Low | D-FR1-A.2.1-12 |  | pos1 | 1.3 |
|  |  | TDLC300-100 Low | D-FR1-A.2.3-12 |  | pos1 | 18.2 |
| 2 | 4 | TDLB100-400 Low | D-FR1-A.2.1-12 |  | pos1 | -2.3 |
|  |  | TDLC300-100 Low | D-FR1-A.2.3-12 |  | pos1 | 11.2 |
|  | 8 | TDLB100-400 Low | D-FR1-A.2.1-12 |  | pos1 | -5.4 |
|  |  | TDLC300-100 Low | D-FR1-A.2.3-12 |  | pos1 | 7.0 |

Table 8.1.2.1.2-13: Minimum requirements for PUSCH with 70% of maximum throughput, Type B, 40 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of Demodulation Branches | Propagation conditions and correlation matrix (Annex G) | FRC (Annex A) | Additional DM-RS position |  | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | TDLB100-400 Low | D-FR1-A.2.1-6 |  | pos1 | -2.5 |
|  | 2 | TDLC300-100 Low | D-FR1-A.2.3-6 |  | pos1 | 10.0 |
|  |  | TDLA30-10 Low | D-FR1-A.2.4-6 |  | pos1 | 12.5 |
|  |  | TDLB100-400 Low | D-FR1-A.2.1-6 |  | pos1 | -5.8 |
| 1 | 4 | TDLC300-100 Low | D-FR1-A.2.3-6 |  | pos1 | 6.2 |
|  |  | TDLA30-10 Low | D-FR1-A.2.4-6 |  | pos1 | 8.7 |
|  |  | TDLB100-400 Low | D-FR1-A.2.1-6 |  | pos1 | -8.8 |
|  | 8 | TDLC300-100 Low | D-FR1-A.2.3-6 |  | pos1 | 3.0 |
|  |  | TDLA30-10 Low | D-FR1-A.2.4-6 |  | pos1 | 5.5 |
|  | 2 | TDLB100-400 Low | D-FR1-A.2.1-13 |  | pos1 | 1.7 |
|  |  | TDLC300-100 Low | D-FR1-A.2.3-13 |  | pos1 | 18.7 |
| 2 | 4 | TDLB100-400 Low | D-FR1-A.2.1-13 |  | pos1 | -2.1 |
|  |  | TDLC300-100 Low | D-FR1-A.2.3-13 |  | pos1 | 11.2 |
|  | 8 | TDLB100-400 Low | D-FR1-A.2.1-13 |  | pos1 | -5.2 |
|  |  | TDLC300-100 Low | D-FR1-A.2.3-13 |  | pos1 | 6.9 |

Table 8.1.2.1.2-14: Minimum requirements for PUSCH with 70% of maximum throughput, Type B, 100 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of Demodulation Branches | Propagation conditions and correlation matrix (Annex G) | FRC (Annex A) | Additional DM-RS position |  | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | TDLB100-400 Low | D-FR1-A.2.1-7 |  | pos1 | -2.5 |
|  | 2 | TDLC300-100 Low | D-FR1-A.2.3-7 |  | pos1 | 10.1 |
|  |  | TDLA30-10 Low | D-FR1-A.2.4-7 |  | pos1 | 13.1 |
|  |  | TDLB100-400 Low | D-FR1-A.2.1-7 |  | pos1 | -5.8 |
| 1 | 4 | TDLC300-100 Low | D-FR1-A.2.3-7 |  | pos1 | 6.3 |
|  |  | TDLA30-10 Low | D-FR1-A.2.4-7 |  | pos1 | 9.2 |
|  |  | TDLB100-400 Low | D-FR1-A.2.1-7 |  | pos1 | -8.7 |
|  | 8 | TDLC300-100 Low | D-FR1-A.2.3-7 |  | pos1 | 3.1 |
|  |  | TDLA30-10 Low | D-FR1-A.2.4-7 |  | pos1 | 5.9 |
|  | 2 | TDLB100-400 Low | D-FR1-A.2.1-14 |  | pos1 | 1.6 |
|  |  | TDLC300-100 Low | D-FR1-A.2.3-14 |  | pos1 | 19.3 |
| 2 | 4 | TDLB100-400 Low | D-FR1-A.2.1-14 |  | pos1 | -2.2 |
|  |  | TDLC300-100 Low | D-FR1-A.2.3-14 |  | pos1 | 11.6 |
|  | 8 | TDLB100-400 Low | D-FR1-A.2.1-14 |  | pos1 | -5.3 |
|  |  | TDLC300-100 Low | D-FR1-A.2.3-14 |  | pos1 | 7.1 |

#### 8.1.2.2	Requirements for PUSCH with transform precoding enabled

##### 8.1.2.2.1	General

The performance requirement of PUSCH is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in annex A. The performance requirements assume HARQ retransmissions.

Table 8.1.2.2.1-1: Test parameters for testing PUSCH

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Enabled |
| Cyclic Prefix |  | Normal |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | 0 |
|  | DM-RS sequence generation | NID0=0, group hopping and sequence hopping are disabled |
| Time domain | PUSCH mapping type | A, B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | 15 kHz SCS: 25 PRBs in the middle of the test bandwidth 30 kHz SCS: 24 PRBs in the middle of the test bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL patterns. |  |  |

##### 8.1.2.2.2	Minimum requirements

The throughput shall be equal to or larger than 70% of the maximum throughput for the FRCs stated in tables 8.1.2.2.2-1 to 8.1.2.2.2-4 at the given SNR. FRCs are defined in annex A.

Table 8.1.2.2.2-1: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 5 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of Demodulation Branches | Propagation conditions and correlation matrix (Annex G) | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- |
|  | 2 | TDLB100-400 Low | D-FR1-A.2.1-15 | pos1 | -2.4 |
| 1 | 4 | TDLB100-400 Low | D-FR1-A.2.1-15 | pos1 | -5.7 |
|  | 8 | TDLB100-400 Low | D-FR1-A.2.1-15 | pos1 | -8.5 |

Table 8.1.2.2.2-2: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 10 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of Demodulation Branches | Propagation conditions and correlation matrix (Annex G) | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- |
|  | 2 | TDLB100-400 Low | D-FR1-A.2.1-16 | pos1 | -2.5 |
| 1 | 4 | TDLB100-400 Low | D-FR1-A.2.1-16 | pos1 | -5.7 |
|  | 8 | TDLB100-400 Low | D-FR1-A.2.1-16 | pos1 | -8.4 |

Table 8.1.2.2.2-3: Minimum requirements for PUSCH with 70% of maximum throughput, Type B, 5 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of Demodulation Branches | Propagation conditions and correlation matrix (Annex G) | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- |
|  | 2 | TDLB100-400 Low | D-FR1-A.2.1-15 | pos1 | -2.3 |
| 1 | 4 | TDLB100-400 Low | D-FR1-A.2.1-15 | pos1 | -5.8 |
|  | 8 | TDLB100-400 Low | D-FR1-A.2.1-15 | pos1 | -8.6 |

Table 8.1.2.2.2-4: Minimum requirements for PUSCH with 70% of maximum throughput, Type B, 10 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of Demodulation Branches | Propagation conditions and correlation matrix (Annex G) | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- |
|  | 2 | TDLB100-400 Low | D-FR1-A.2.1-16 | pos1 | -2.7 |
| 1 | 4 | TDLB100-400 Low | D-FR1-A.2.1-16 | pos1 | -6.0 |
|  | 8 | TDLB100-400 Low | D-FR1-A.2.1-16 | pos1 | -8.8 |

#### 8.1.2.3	Requirements for UCI multiplexed on PUSCH

##### 8.1.2.3.1	General

In the tests for UCI multiplexed on PUSCH, the UCI information only contains CSI part 1 and CSI part 2 information, and there is no HACK/ACK information transmitted.

The CSI part 1 block error probability (BLER) is defined as the probability of incorrectly decoding the CSI part 1 information when the CSI part 1 information is sent as follow:

BLERCSI part 1= #(false CSI part 1)#(CSI part 1)

where:

-	#(false CSI part 1) denotes the number of incorrectly decoded CSI part 1 information transmitted occasions

-	#(CSI part 1) denotes the number of CSI part 1 information transmitted occasions.

The CSI part 2 block error probability is defined as the probability of incorrectly decoding the CSI part 2 information when the CSI part 2 information is sent as follows:

BLERCSI part 2= #(false CSI part 2)#(CSI part 2)

where:

-	#(false CSI part 2) denotes the number of incorrectly decoded CSI part 2 information transmitted occasions

-	#(CSI part 2) denotes the number of CSI part 2 information transmitted occasions.

The number of UCI information bit payload per slot is defined for two cases as follows:

-	5 bits in CSI part 1, 2 bits in CSI part 2

-	20 bits in CSI part 1, 20 bits in CSI part 2

The 7bits UCI case is further defined with the bitmap [c0 c1 c2 c3 c4] = [0 1 0 1 0] for CSI part 1 information, where c0 is mapping to the RI information, and with the bitmap [c0 c1] = [1 0] for CSI part2 information.

The 40bits UCI information case is assumed random information bit selection.

In both tests, PUSCH data, CSI part 1 and CSI part 2 information are transmitted simultaneously.

Table 8.1.2.3.1-1: Test parameters for testing UCI on PUSCH

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| Cyclic Prefix |  | Normal |
| HARQ | Maximum number of HARQ transmissions | 1 |
|  | RV sequence | 0 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | Single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID=0 |
| Time domain | PUSCH mapping type | A,B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
| assignment | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
|  | Number of CSI part 1 and CSI part 2 information bit payload | {5,2},{20,20} |
|  | scaling | 1 |
| UCI | betaOffsetACK-Index1 | 11 |
|  | betaOffsetCSI-Part1-Index1 and betaOffsetCSI-Part1-Index2 | 13 |
|  | betaOffsetCSI-Part2-Index1 and betaOffsetCSI-Part2-Index2 | 13 |
|  | UCI partition for frequency hopping | Disabled |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL patterns. |  |  |

##### 8.1.2.3.2	Minimum requirements

The CSI part 1 block error probability shall not exceed 0.1% at the SNR in table 8.2.2.3.2-1 and table 8.2.2.3.2-2.The CSI part 2 block error probability shall not exceed 1% at the SNR given in table 8.2.2.3.2-3 and table 8.2.2.3.2-4.

Table 8.1.2.3.2-1: Minimum requirements for UCI multiplexed on PUSCH, Type A, CSI part 1, 10 MHz Channel Bandwidth, 30 kHz SCS

| Number of TX antennas | Number of Demodulation Branches | Propagation conditions and correlation matrix (Annex G) | UCI bits (CSI part 1, CSI part 2) | Additional DM-RS position | FRC (Annex A) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | TDLC300-100 Low | 7(5,2) | pos1 | D-FR1-A.2.3-4 | 5.4 |
|  | 2 | TDLC300-100 Low | 40(20,20) | pos1 | D-FR1-A.2.3-4 | 4.3 |

Table 8.1.2.3.2-2: Minimum requirements for UCI multiplexed on PUSCH, Type B, CSI part 1, 10 MHz Channel Bandwidth, 30 kHz SCS

| Number of TX antennas | Number of Demodulation Branches | Propagation conditions and correlation matrix (Annex G) | UCI bits (CSI part 1, CSI part 2) | Additional DM-RS position | FRC (Annex A) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | TDLC300-100 Low | 7(5,2) | pos1 | D-FR1-A.2.3-4 | 5.8 |
|  | 2 | TDLC300-100 Low | 40(20,20) | pos1 | D-FR1-A.2.3-4 | 4.1 |

Table 8.1.2.3.2-3: Minimum requirements for UCI multiplexed on PUSCH, Type A, CSI part 2, 10 MHz Channel Bandwidth, 30 kHz SCS

| Number of TX antennas | Number of Demodulation Branches | Propagation conditions and correlation matrix (Annex G) | UCI bits (CSI part 1, CSI part 2) | Additional DM-RS position | FRC (Annex A) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | TDLC300-100 Low | 7(5,2) | pos1 | D-FR1-A.2.3-4 | -0.2 |
|  | 2 | TDLC300-100 Low | 40(20,20) | pos1 | D-FR1-A.2.3-4 | 2.4 |

Table 8.1.2.3.2-4: Minimum requirements for UCI multiplexed on PUSCH, Type B, CSI part 2, 10 MHz Channel Bandwidth, 30 kHz SCS

| Number of TX antennas | Number of Demodulation Branches | Propagation conditions and correlation matrix (Annex G) | UCI bits (CSI part 1, CSI part 2) | Additional DM-RS position | FRC (Annex A) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | TDLC300-100 Low | 7(5,2) | pos1 | D-FR1-A.2.3-4 | 0.3 |
|  | 2 | TDLC300-100 Low | 40(20,20) | pos1 | D-FR1-A.2.3-4 | 2.6 |

### 8.1.3	Performance requirements for PUCCH

#### 8.1.3.1	DTX to ACK probability

##### 8.1.3.1.1	General

The DTX to ACK probability, i.e. the probability that ACK is detected when nothing was sent:

ProbPUCCH DTX→Ack bits= #(false ACK bits)#PUCCH DTX*#(ACK/NACK bits)

where:

-	#(false ACK bits) denotes the number of detected ACK bits.

-	#(ACK/NACK bits) denotes the number of encoded bits per slot

-	#(PUCCH DTX) denotes the number of DTX occasions

##### 8.1.3.1.2     Minimum requirement

The DTX to ACK probability shall not exceed 1% for all PUCCH formats carrying ACK/NACK bits:

ProbPUCCH DTX→Ack bits ≤ 10-2

#### 8.1.3.2	Performance requirements for PUCCH format 0

##### 8.1.3.2.1	General

The ACK missed detection probability is the probability of not detecting an ACK when an ACK was sent.

Table 8.1.3.2.1-1: Test Parameters

| Parameter | Test |
| --- | --- |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | N/A for 1 symbol Enabled for 2 symbols |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 13 for 1 symbol 12 for 2 symbols |

[The transient period as specified in TS 38.101-1 [3] clause 6.3.3.1 is not taken into account for performance requirement testing, where the RB hopping is symmetric to the CC centre, i.e. intra-slot frequency hopping is enabled.]

##### 8.1.3.2.2	Minimum requirement

The ACK missed detection probability shall not exceed 1% at the SNR given in table 8.1.3.2.2-1 and in table 8.1.3.2.2-2.

Table 8.1.3.2.2-1: Minimum requirements for PUCCH format 0 and 15 kHz SCS

| Number of | Number of RX | Propagation conditions and | Number of | Channel bandwidth / SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| TX antennas | antennas | correlation matrix (Annex G) | OFDM symbols | 5 MHz | 10 MHz | 20 MHz |
| 1 | 2 | TDLC300-100 Low | 1 | 9.4 | 8.8 | 9.3 |
|  |  |  | 2 | 2.8 | 3.7 | 3.3 |
| 1 | 4 | TDLC300-100 Low | 1 | 3.0 | 2.9 | 3.2 |
|  |  |  | 2 | -1.0 | -0.5 | -0.8 |
| 1 | 8 | TDLC300-100 Low | 1 | -1.1 | -1.1 | -1.1 |
|  |  |  | 2 | -4.1 | -3.9 | -4.0 |

Table 8.2.3.2.2-2: Minimum requirements for PUCCH format 0 and 30 kHz SCS

| Number of | Number of RX | Propagation conditions and | Number of | Channel bandwidth / SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TX antennas | antennas | correlation matrix (Annex G) | OFDM symbols | 10 MHz | 20 MHz | 40 MHz | 100 MHz |
| 1 | 2 | TDLC300-100 Low | 1 | 9.8 | 9.8 | 9.5 | 9.2 |
|  |  |  | 2 | 4.2 | 3.6 | 3.8 | 3.5 |
| 1 | 4 | TDLC300-100 Low | 1 | 3.4 | 3.4 | 3.0 | 3.3 |
|  |  |  | 2 | -0.3 | -0.4 | -0.5 | -0.8 |
| 1 | 8 | TDLC300-100 Low | 1 | -1.0 | -1.0 | -1.1 | -1.0 |
|  |  |  | 2 | -3.7 | -3.8 | -4.0 | -3.9 |

#### 8.1.3.3	Performance requirements for PUCCH format 1

##### 8.1.3.3.1	NACK to ACK requirements

##### 8.1.3.3.1.1	General

The NACK to ACK detection probability is the probability that an ACK bit is falsely detected when an NACK bit was sent on the particular bit position, where the NACK to ACK detection probability is defined as follows:

,

where:

-	denotes the total number of NACK bits transmitted

-	denotes the number of NACK bits decoded as ACK bits at the receiver, i.e. the number of received ACK bits

-	NACK bits in the definition do not contain the NACK bits which are mapped from DTX, i.e. NACK bits received when DTX is sent should not be considered.

Random codeword selection is assumed.

Table 8.1.3.3.1.1-1: Test Parameters

| Parameter | Test |
| --- | --- |
| Cyclic prefix | Normal |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |

[The transient period as specified in TS 38.101-1 [3] clause 6.3.3.1 is not taken into account for performance requirement testing, where the RB hopping is symmetric to the CC centre, i.e. intra-slot frequency hopping is enabled.]

##### 8.1.3.3.1.	Minimum requirements

The NACK to ACK probability shall not exceed 0.1% at the SNR given in table 8.1.3.3.1.2-1 and table 8.1.3.3.1.2-2.

Table 8.1.3.3.1.2-1: Minimum requirements for PUCCH format 1 with 15 kHz SCS

| Number of | Number of | Propagation | Channel bandwidth / SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- |
| TX antennas | Demodulation Branches | conditions and correlation matrix (Annex G) | 5 MHz | 10 MHz | 20 MHz |
|  | 2 | TDLC-300-100 Low | -3.8 | -3.6 | -3.6 |
| 1 | 4 | TDLC-300-100 Low | -8.4 | -7.6 | -8.4 |
|  | 8 | TDLC-300-100 Low | -11.8 | -11.4 | -11.4 |

Table 8.1.3.3.1.2-2: Minimum requirements for PUCCH format 1 with 30 kHz SCS

| Number of | Number of | Propagation | Channel bandwidth / SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| TX antennas | Demodulation Branches | conditions and correlation matrix (Annex G) | 10 MHz | 20 MHz | 40 MHz | 100 MHz |
|  | 2 | TDLC-300-100 Low | -2.8 | -3.3 | -3.9 | -3.5 |
| 1 | 4 | TDLC-300-100 Low | -8.1 | -8.3 | -7.5 | -8.0 |
|  | 8 | TDLC-300-100 Low | -11.5 | -11.2 | -11.6 | -11.3 |

##### 8.1.3.3.2	ACK missed detection requirements

##### 8.1.3.3.2.1	General

The ACK missed detection probability is the probability of not detecting an ACK when an ACK was sent. The test parameters in table 8.3.3.1.1-1 are configured.

[The transient period as specified in TS 38.101-1 [3] clause 6.3.3.1 is not taken into account for performance requirement testing, where the RB hopping is symmetric to the centre, i.e. intra-slot frequency hopping is enabled.]

##### 8.1.3.3.2.2	Minimum requirements

The ACK missed detection probability shall not exceed 1% at the SNR given in table 8.1.3.3.2.2-1 and in table 8.1.3.3.2.2-2.

Table 8.1.3.3.2.2-1: Minimum requirements for PUCCH format 1 with 15 kHz SCS

| Number of | Number of | Propagation | Channel bandwidth / SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- |
| TX antennas | Demodulation Branches | conditions and correlation matrix (Annex G) | 5 MHz | 10 MHz | 20 MHz |
|  | 2 | TDLC-300-100 Low | -5.0 | -4.4 | -5.0 |
| 1 | 4 | TDLC-300-100 Low | -8.6 | -8.2 | -8.5 |
|  | 8 | TDLC-300-100 Low | -11.6 | -11.5 | -11.5 |

Table 8.1.3.3.2.2-2: Minimum requirements for PUCCH format 1 with 30 kHz SCS

| Number of | Number of | Propagation | Channel bandwidth / SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| TX antennas | Demodulation Branches | conditions and correlation matrix (Annex G) | 10 MHz | 20 MHz | 40 MHz | 100 MHz |
|  | 2 | TDLC-300-100 Low | -3.9 | -4.4 | -4.4 | -4.2 |
| 1 | 4 | TDLC-300-100 Low | -8.0 | -8.1 | -8.4 | -8.3 |
|  | 8 | TDLC-300-100 Low | -11.4 | -11.4 | -11.4 | -11.4 |

#### 8.1.3.4	Performance requirements for PUCCH format 2

##### 8.1.3.4.1	NACK to ACK requirements

##### 8.1.3.4.1.	General

The ACK missed detection probability is the probability of not detecting an ACK when an ACK was sent.

The ACK missed detection requirement only applies to the PUCCH format 2 with 4 UCI bits.

Table 8.1.3.4.1.1-1: Test Parameters

| Parameter | Value |
| --- | --- |
| Cyclic Prefix | Normal |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | N/A |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 4 |
| Number of symbols | 1 |
| The number of UCI information bits | 4 |
| First symbol | 13 |
| DM-RS sequence generation | NID0=0 |

[The transient period as specified in TS 38.101-1 [3] clause 6.3.3.1 is not taken into account for performance requirement testing, where the RB hopping is symmetric to the CC center, i.e. intra-slot frequency hopping is enabled.]

##### 8.1.3.4.1.2	Minimum requirements

The ACK missed detection probability shall not exceed 1% at the SNR given in table 8.1.3.4.1.2-1 and table 8.1.3.4.1.2-2 for 4UCI bits.

Table 8.1.3.4.1.2-1: Minimum requirements for PUCCH format 2 with 15 kHz SCS

| Number of | Number of | Propagation | Channel bandwidth / SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- |
| TX antennas | Demodulation Branches | conditions and correlation matrix (Annex G) | 5 MHz | 10 MHz | 20 MHz |
|  | 2 | TDLC300-100 Low | 5.8 | 5.6 | 5.9 |
| 1 | 4 | TDLC300-100 Low | 0.4 | 0.5 | 0.3 |
|  | 8 | TDLC300-100 Low | -3.5 | -3.5 | -3.5 |

Table 8.1.3.4.1.2-2: Minimum requirements for PUCCH format 2 with 30 kHz SCS

| Number of | Number of | Propagation | Channel bandwidth / SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| TX antennas | Demodulation Branches | conditions and correlation matrix (Annex G) | 10 MHz | 20 MHz | 40 MHz | 100 MHz |
|  | 2 | TDLC300-100 Low | 5.5 | 5.6 | 5.5 | 5.7 |
| 1 | 4 | TDLC300-100 Low | 0.3 | 0.2 | 0.3 | 0.4 |
|  | 8 | TDLC300-100 Low | -3.6 | -3.6 | -3.5 | -3.3 |

##### 8.1.3.4.2	UCI BLER performance requirements

##### 8.1.3.4.2.	General

The UCI block error probability (BLER) is defined as the probability of incorrectly decoding the UCI information when the UCI information is sent. The UCI information does not contain CSI part 2.

[The transient period as specified in TS 38.101-1 [3] clause 6.3.3.1 is not taken into account for performance requirement testing, where the RB hopping is symmetric to the CC centre, i.e. intra-slot frequency hopping is enabled.]

The UCI block error probability performance requirement only applies to the PUCCH format 2 with 22 UCI bits.

Table 8.1.3.4.2.1-1: Test Parameters

| Parameter | Value |
| --- | --- |
| Cyclic Prefix | Normal |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| Frist PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |

##### 8.1.3.4.2.2	Minimum requirement

The UCI block error probability shall not exceed 1% at the SNR given in table 8.2.3.4.2.2-1 and table 8.1.3.4.2.2-2 for 22 UCI bits.

Table 8.1.3.4.2.2-1: Minimum requirements for PUCCH format 2 with 15 kHz SCS

| Number of | Number of | Channel bandwidth / SNR (dB) |  |  |
| --- | --- | --- | --- | --- |
| TX antennas | Demodulation Branches | 5 MHz | 10 MHz | 20 MHz |
|  | 2 | 0.2 | 0.8 | 1.2 |
| 1 | 4 | -3.6 | -3.2 | -3.2 |
|  | 8 | -6.8 | -6.7 | -6.8 |

Table 8.1.3.4.2.2-2: Minimum requirements for PUCCH format 2 with 30 kHz SCS

| Number of | Number of | Cyclic Prefix | Propagation | Channel bandwidth / SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TX antennas | Demodulation Branches |  | conditions and correlation matrix (Annex G) | 10 MHz | 20 MHz | 40 MHz | 100 MHz |
|  | 2 | Normal | TDLC300-100 Low | 0.5 | 1.1 | 0.4 | 0.3 |
| 1 | 4 | Normal | TDLC300-100 Low | -3.3 | -2.9 | -3.3 | -3.4 |
|  | 8 | Normal | TDLC300-100 Low | -5.8 | -5.8 | -6.7 | -5.9 |

#### 8.1.3.5	Performance requirements for PUCCH format 3

##### 8.1.3.5.1	General

The performance is measured by the required SNR at UCI block error probability not exceeding 1%.

The UCI block error probability is defined as the conditional probability of incorrectly decoding the UCI information when the UCI information is sent. The UCI information does not contain CSI part 2.

[The transient period as specified in TS 38.101-1 [3] clause 6.3.3.1 is not taken into account for performance requirement testing, where the RB hopping is symmetric to the centre, i.e. intra-slot frequency hopping is enabled.]

Table 8.1.3.5.1-1: Test Parameters

| Parameter | Test 1 | Test 2 |
| --- | --- | --- |
| Cyclic Prefix | Normal |  |
| Modulation order | QPSK |  |
| First PRB prior to frequency hopping | 0 |  |
| Intra-slot frequency hopping | enabled |  |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |  |
| Group and sequence hopping | neither |  |
| Hopping ID | 0 |  |
| Number of PRBs | 1 | 3 |
| Number of symbols | 14 | 4 |
| The number of UCI information bits | 16 | 16 |
| First symbol | 0 | 0 |

##### 8.1.3.5.2	Minimum requirements

The UCI block error probability shall not exceed 1% at the SNR given in Table 8.2.3.5.2-1 and Table 8.2.3.5.2-2.

Table 8.2.3.5.2-1: Minimum requirements for PUCCH format 3 with 15 kHz SCS

| Test | Number of | Number of | Additional | Channel bandwidth / SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Number | TX antennas | Demodulation Branches | DM-RS configuration | 5 MHz | 10 MHz | 20 MHz |
| 1 | 1 | 2 | No additional DM-RS | 0.2 | 1.1 | 0.3 |
|  |  |  | Additional DM-RS | -0.1 | 0.5 | -0.1 |
|  |  | 4 | No additional DM-RS | -3.8 | -3.3 | -3.8 |
|  |  |  | Additional DM-RS | -4.3 | -4.0 | -4.0 |
|  |  | 8 | No additional DM-RS | -7.0 | -6.7 | -6.9 |
|  |  |  | Additional DM-RS | -7.7 | -7.5 | -7.7 |
| 2 | 1 | 2 | No additional DM-RS | 1.4 | 2.2 | 2.0 |
|  |  | 4 | No additional DM-RS | -3.1 | -2.5 | -2.5 |
|  |  | 8 | No additional DM-RS | -6.5 | -6.0 | -6.2 |

Table 8.1.3.5.2-2: Minimum requirements for PUCCH format 3 with 30 kHz SCS

| Test | Number | Number | Cyclic | Propagation | Additional | Channel bandwidth / SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number | of TX antennas | of Demodulation Branches | Prefix | conditions and correlation matrix (Annex G) | DM-RS configuration | 10 MHz | 20 MHz | 40 MHz | 100 MHz |
| 1 | 1 | 2 | Normal | TDLC300-100 Low | No additional DM-RS | 0.9 | 0.6 | 0.6 | 0.9 |
|  |  |  |  |  | Additional DM-RS | 0.5 | 0.3 | 0.0 | 0.1 |
|  |  | 4 | Normal | TDLC300-100 Low | No additional DM-RS | -3.1 | -3.4 | -3.2 | -3.5 |
|  |  |  |  |  | Additional DM-RS | -3.7 | -4.1 | -4.0 | -4.2 |
|  |  | 8 | Normal | TDLC300-100 Low | No additional DM-RS | -6.6 | -6.7 | -6.8 | -6.8 |
|  |  |  |  |  | Additional DM-RS | -7.5 | -7.6 | -7.6 | -7.7 |
| 2 | 1 | 2 | Normal | TDLC300-100 Low | No additional DM-RS | 1.8 | 2.0 | 2.0 | 1.5 |
|  |  | 4 | Normal | TDLC300-100 Low | No additional DM-RS | -2.9 | -3.0 | -2.4 | -3.0 |
|  |  | 8 | Normal | TDLC300-100 Low | No additional DM-RS | -6.4 | -6.0 | -6.4 | -6.2 |

#### 8.1.3.6	Performance requirements for PUCCH format 4

##### 8.1.3.6.1	General

The performance is measured by the required SNR at UCI block error probability not exceeding 1%.

The UCI block error probability is defined as the conditional probability of incorrectly decoding the UCI information when the UCI information is sent. The UCI information does not contain CSI part 2.

[The transient period as specified in TS 38.101-1 [3] clause 6.3.3.1 is not taken into account for performance requirement testing, where the RB hopping is symmetric to the centre, i.e. intra-slot frequency hopping is enabled.]

Table 8.1.3.6.1-1: Test parameters

| Parameter | Value |
| --- | --- |
| Cyclic Prefix | Normal |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Number of PRBs | 1 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of symbols | 14 |
| The number of UCI information bits | 22 |
| First symbol | 0 |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |

##### 8.1.3.6.2	Minimum requirement

The UCI block error probability shall not exceed 1% at the SNR given in Table 8.2.3.6.2-1 and Table 8.2.3.6.2-2.

Table 8.1.3.6.2-1: Required SNR for PUCCH format 4 with 15 kHz SCS

| Number of | Number of | Propagation | Additional | Channel bandwidth / SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| TX antennas | Demodulation Branches | conditions and correlation matrix (Annex G) | DM-RS configuration | 5 MHz | 10 MHz | 20 MHz |
| 1 | 2 | TDLC300-100 Low | No additional DM-RS | 1.8 | 2.6 | 2.2 |
|  |  |  | Additional DM-RS | 1.6 | 2.4 | 1.8 |
|  | 4 | TDLC300-100 Low | No additional DM-RS | -2.3 | -1.9 | -2.2 |
|  |  |  | Additional DM-RS | -2.9 | -2.6 | -2.7 |
|  | 8 | TDLC300-100 Low | No additional DM-RS | -5.9 | -5.7 | -5.8 |
|  |  |  | Additional DM-RS | -6.6 | -6.4 | -6.3 |

Table 8.1.3.6.2-2: Required SNR for PUCCH format 4 with 30 kHz SCS

| Number | Number | Propagation | Additional | Channel bandwidth / SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| of TX antennas | of Demodulation Branches | conditions and correlation matrix (Annex G) | DM-RS configuration | 10 MHz | 20 MHz | 40 MHz | 100 MHz |
| 1 | 2 | TDLC300-100 Low | No additional DM-RS | 3.1 | 2.8 | 3.1 | 2.8 |
|  |  |  | Additional DM-RS | 2.8 | 2.3 | 3.1 | 2.2 |
|  | 4 | TDLC300-100 Low | No additional DM-RS | -1.7 | -1.9 | -1.7 | -2.1 |
|  |  |  | Additional DM-RS | -2.0 | -2.5 | -2.5 | -2.4 |
|  | 8 | TDLC300-100 Low | No additional DM-RS | -5.6 | -5.5 | -5.5 | -5.5 |
|  |  |  | Additional DM-RS | -6.2 | -6.1 | -6.4 | -6.2 |

#### 8.1.3.7	Performance requirements for multi-slot PUCCH

##### 8.1.3.7.1	General

##### 8.1.3.7.2	Performance requirements for multi-slot PUCCH format 1

##### 8.1.3.7.2.	ACK to NACK requirements

##### 8.1.3.7.2.1.1	General

The NACK to ACK detection probability is the probability that an ACK bit is falsely detected when an NACK bit was sent on the particular bit position, where the NACK to ACK detection probability is defined as follows:

,

where:

-	denotes the total number of NACK bits transmitted

-	denotes the number of NACK bits decoded as ACK bits at the receiver, i.e. the number of received ACK bits

-	NACK bits in the definition do not contain the NACK bits which are mapped from DTX, i.e. NACK bits received when DTX is sent should not be considered.

Random codeword selection is assumed.

Table 8.1.3.7.2.1.1-1: Test Parameters for multi-slot PUCCH format 1

| Parameter | Test |
| --- | --- |
| Cyclic Prefix | Normal |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | disabled |
| Inter-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Number of slots for PUCCH repetition | 2 |

##### 8.1.3.7.2.1.2	Minimum requirements

The multi-slot NACK to ACK probability shall not exceed 0.1% at the SNR given in table 8.2.3.7.2.1.2-1.

Table 8.1.3.7.2.1.2-1: Minimum requirements for multi-slot PUCCH format 1 with 30kHz SCS

| Number of TX | Number of RX | Propagation conditions and correlation matrix | Channel bandwidth / SNR (dB) |
| --- | --- | --- | --- |
| antennas | antennas | (Annex G) | 40 MHz |
| 1 | 2 | TDLC-300-100 Low | -6.3 |

##### 8.1.3.7.2.2	ACK missed detection requirements

##### 8.1.3.7.2.2.1	General

The ACK missed detection probability is the probability of not detecting an ACK when an ACK was sent. The test parameters in table 8.2.3.7.2.1.1-1 are configured.

##### 8.1.3.7.2.2.2	Minimum requirements

The multi-slot ACK missed detection probability shall not exceed 1% at the SNR given in table 8.2.3.7.2.2.2-1.

Table 8.1.3.7.2.2.2-1: Minimum requirements for multi-slot PUCCH format 1 with 30kHz SCS

| Number of TX | Number of RX | Propagation conditions and correlation matrix | Channel bandwidth / SNR (dB) |
| --- | --- | --- | --- |
| antennas | antennas | (Annex G) | 40 MHz |
| 1 | 2 | TDLC-300-100 Low | -7.6 |

### 8.1.4	Performance requirements for PRACH

#### 8.1.4.1	PRACH false alarm probability

##### 8.1.4.1.1	General

The false alarm requirement is valid for any number of receive antennas, for any channel bandwidth.

The false alarm probability is the conditional total probability of erroneous detection of the preamble (i.e. erroneous detection from any detector) when input is only noise.

##### 8.1.4.1.2	Minimum requirement

The false alarm probability shall be less than or equal to 0.1%.

#### 8.1.4.2	PRACH detection requirements

##### 8.1.4.2.1	General

The probability of detection is the conditional probability of correct detection of the preamble when the signal is present. There are several error cases – detecting different preamble than the one that was sent, not detecting a preamble at all or correct preamble detection but with the wrong timing estimation. A timing estimation error occurs if the estimation error of the timing of the strongest path is larger than the time error tolerance given in Table 8.1.4.2.1-1.

Table 8.1.4.2.1-1: Time error tolerance

| PRACH | PRACH SCS | Time error tolerance |
| --- | --- | --- |
| preamble | (kHz) |  |
| 0 | 1.25 | 2.55 us |
| A1, A2, A3, B4, | 15 | 2.03 us |
| C0, C2 | 30 | 1.77 us |

The test preambles for normal mode are listed in table A.2.5-1 and the test parameter msg1-FrequencyStart is set to 0.

##### 8.1.4.2.2	Minimum requirements for normal mode

The probability of detection shall be equal to or exceed 99% for the SNR levels listed in Tables 8.1.4.2.2-1 to 8.1.4.2.2-3.

Table 8.1.4.2.2-1: PRACH missed detection requirements for Normal Mode, 1.25 kHz SCS

| Number of TX | Number of RX | Propagation conditions and | Frequency offset | SNR (dB) |
| --- | --- | --- | --- | --- |
| antennas | antennas | correlation matrix (Annex G) |  | Burst format 0 |
| 1 | 2 | TDLC300-100 Low | 400 Hz | -6.6 |
|  | 4 | TDLC300-100 Low | 400 Hz | -11.9 |
|  | 8 | TDLC300-100 Low | 400 Hz | -15.8 |

Table 8.1.4.2.2-2: PRACH missed detection requirements for Normal Mode, 15 kHz SCS

| Number of |  | Propagation | Frequency | SNR (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TX antennas | Number of Demodulation Branches | conditions and correlation matrix (Annex G) | offset | Burst format A1 | Burst format A2 | Burst format A3 | Burst format B4 | Burst format C0 | Burst format C2 |
| 1 | 2 | TDLC300-100 Low | 400 Hz | -2.1 | -4.8 | -6.6 | -8.8 | 0.8 | -4.9 |
|  | 4 | TDLC300-100 Low | 400 Hz | -7.3 | -10.3 | -11.7 | -13.8 | -4.3 | -10.2 |
|  | 8 | TDLC300-100 Low | 400 Hz | -11.0 | -13.9 | -15.2 | -17.3 | -8.1 | -13.9 |

Table 8.1.4.2.2-3: PRACH missed detection requirements for Normal Mode, 30 kHz SCS

| Number of |  | Propagation | Frequency | SNR (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TX antennas | Number of Demodulation Branches | conditions and correlation matrix (Annex G) | offset | Burst format A1 | Burst format A2 | Burst format A3 | Burst format B4 | Burst format C0 | Burst format C2 |
| 1 | 2 | TDLC300-100 Low | 400 Hz | -2.8 | -5.7 | -7.4 | -9.9 | 0.1 | -5.6 |
|  | 4 | TDLC300-100 Low | 400 Hz | -7.2 | -10.4 | -12.0 | -14.5 | -4.5 | -10.4 |
|  | 8 | TDLC300-100 Low | 400 Hz | -10.7 | -13.7 | -15.1 | -17.6 | -7.8 | -13.7 |