3GPP TSG-RAN4 Meeting #99-e	R4-2108806

Online, , 19th May 2021 - 27th May 2021

Source: 	Nokia, Nokia Shanghai Bell

Title: 					On increasing UE maximum power high limit for NR uplink inter band CA

Agenda Item: 			8.35.2

Document for:	Approval

# 1	Introduction

RAN4#98-bis-e further discussed how to fully take advantage of the hardware capability already present in the UE and ways to lift the restriction on maximum output power imposed by the power class for uplink inter-band CA. The idea was originally proposed in [1] in RAN4#97-e. Though RAN4#98-bis-e was not able to approve an WF of [2], the content was quite close to an agreement. We share our views on each option listed in [2].

  - Remove PPowerClass from PCMAX_H

  - Replace PPowerClass with sum or modified sum

    - in both PCMAX_H and PCMAX_L, or

    - only in PCMAX_H if still this UE reports PC for CA or

    - in both PCMAX_H and PCMAX_L and relax PCMAX_L by the sum - MAX(PC for Band A, PC for Band B) if the UE doesn’t report PC for CA(R4-2104572)

  - Define a new power class

  - Power boosting

# 2	Scope of the discussion

The WF of [2] referred to CA (inter band CA, intra band CA) and MR-DC in the slide 2 in [2]. The impact of this new idea on the existing requirements are quite different between uplink intra band contiguous/non-contiguous CA and uplink inter band CA. And a concern on the scope was raised in RAN4#98-bis-e so that the WF of [2] was not approved. In addition, the WI (NR_SAR_PC2_interB_SUL_2BUL) itself is for PC2 uplink “inter” band CA. Hence, to make more progress, it would be better to focus on NR uplink inter band CA at least as far as RAN4 discusses this topic under this WI.

Proposal 1: Focus on increasing UE maximum power high limit for NR uplink inter band CA under this WI and revise the WID to accommodate this topic in the objective accordingly.

# 3	Views on identified solutions in [2]

## 3.1	Remove PPowerClass from PCMAX_H

PCMAX_H

In [1], in order to lift the restriction on maximum output power, removing the PPowerClass,CA term within the PCMAX_H for uplink inter-band CA was proposed. Now since the UEs with this feature does not need a PPowerClass,CA term in PCMAX_H,  the UEs does not report the PPowerClass,CA for uplink inter-band CA. If the UE reported the PPowerClass,CA to a network, there would be no reason to remove the PPowerClass,CA term within the PCMAX_H. Because if the UE did so, it would be straightforward to report 27.8dBm as new PPowerClass,CA. Hence, the UE does not report PPowerClass,CA for uplink inter band CA and the gNBs would consider that the UEs’ power class for the uplink inter-band CA is the default PC, i.e., PC3. That means the gNBs cannot distinguish the existing PC3 UEs for uplink inter-band CA and the new UEs with this new feature so that it was pointed out that there is a backward compatible issue.

Observation 1: If removing PPowerClass,CA from PCMAX_H, there is a backward compatibility issue. Because gNBs would consider the UEs supporting the default PC for the uplink inter-band CA. Hence, the gNBs cannot distinguish legacy UEs without this feature, i.e., PC3 and new UEs with it.

In our understanding, the above observed issue can be resolved by introducing a new capability to inform the gNB of the new feature to increase UE maximum power high limit for NR uplink inter band CA.

Observation 2: The backward compatibility issue in the Observation 1 can be resolved by introducing a new capability.

Though now the aforementioned backward compatible issue is resolved, this would raise a new issue that the UE’s maximum output power ability is unknown, and the UE is allowed to transmit at any higher power, i.e., no upper boundary. A contribution of [1] mentioned that this aspect can be resolved because still PCMAX_H shall take MIN{10 log10 ∑ pEMAX,c , PEMAX,CA}. It is true that PEMAX,CA is the value indicated by p-NR-FR1 or by p-UE-FR1 so that the NW can cap control each of the UEs’ total output power by p-UE-FR1. Still, in terms of UE specification perspective, UE’s intrinsic ability seems no upper limit. Hence, the solution in [1] will generate an inconsistency between UEs’ real ability and PCMAX_H.

Observation 3: If removing PPowerClass,CA from PCMAX_H and if it means PPowerClass,CA is not reported, PCMAX_H cannot reflect UE’s real ability and/or the meaning of PC may be ambiguous.

PCMAX_L

If the UE does not report PPowerClass,CA, PCMAX_L does not hold true as follows. Because the unit of the last term becomes dB without PPowerClass,CA while the unit of the others is dBm.

PCMAX_L  = MIN{10 log10 ∑ pEMAX,c  - TC , PEMAX,CA,PPowerClass,CA – MAX(MAX(MPR, A-MPR) + ΔTIB,c + TC + TRxSRS, P-MPRc ) }

Observation 4: If removing PPowerClass,CA from PCMAX_H and if it means PPowerClass,CA is not reported, PCMAX_L does not hold true.

## 3.2	Replace PPowerClass,CA with sum

PCMAX_H

The original motivation of this discussion is lifting the restriction on maximum output power of a UE possessing ability to transmit 23dBm for a band and 26dBm for the other band within an uplink inter band CA. Hence, the total power must not exceed the sum of the individual power classes for the respective bands within the CA. And the power classes are for the respective bands within the CA surely reported to a network. Hence, it is possible for the network to consider the UE’s total power as uplink inter band CA is the sum of them if the UE also report a capability to indicate that the UE has ability to do so instead of reporting the conventional power class for the uplink inter band CA. Hence, the power class of the uplink band CA can be considered as pPowerClass,CA = ∑ pPowerClass,c., where pPowerClass,CA is the linear value of PPowerClass,CA and pPowerClass,c is the linear value of the maximum UE power for serving cell c specified in Table 6.2.1-1 without taking into account the tolerance.

PCMAX_L

It is straightforward to apply the same method for PCMAX_H to that for PCMAX_L.

Observation 5: For a UE with this new feature, by replacing pPowerClass,CA with ∑ pPowerClass,c , the UE can be handled as if the UE has a new power class without introducing it. And this can be applied to any combinations of power classes within a band combination such that 23dBm+26dBm, 23dBm+29dBm, 26dBm+29dBm etc.

## 3.3	new PC and power boosting

In our understanding, the essential point of the discussion is that there are UEs which have intrinsic ability to transmit output power more than some of the specified power classes, but the delta is around 1.8dB(27.8dBm - 26dBm) for a certain case and some companies do not prefer to introduce new power classes only due to 1.8dB difference.

Observation 6: The introduction of new power classes is not suitable to the original purpose of this discussion.

With respect to power boosting, at least this selection shall be avoided. Because in order to utilize power boosting, we need to have a reference. For a case of 23dBm+26dBm, the UE has to report a power class of 26dBm for uplink inter band CA as well as the boosting information of 1.8dB. This is completely not reasonable. There is no reason to dare to split 27.8dBm into 26dBm and 1.8dB and report them separately.

Observation 7: The introduction of power boosting is making the specification more complicated than any other listed options.

Proposal 2: For a UE with this new feature, select replacing pPowerClass,CA with ∑ pPowerClass,c

# 4	Views on Possible impacts

## 4.1	Possible impact to regulations

Even now there are countries where PC2 and/or PC1.5 are not allowed to use them. 3GPP, however, has introduced these new power classes. In those countries, the power of the UEs with PC2 and/or PC1.5 is capped by P-Max and the UEs fallback to PC3. The same applies to the UE with a new feature to increase UE maximum power high limit for NR uplink inter band.

Observation 8: For possible impact on regulations, there is nothing new to be discussed. The same principle of PC2/1.5 UEs applies to the UEs with this new feature. That is, if the IE P-Max as defined in TS 38.331 is provided and set to the maximum output power of the default power class or lower, shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified in clause 6.2A.4.

## 4.2	Possible impact to 3GPP specifications

Signalling

Whichever method is selected, a new signalling is needed. Removing PPowerClass,CA from PCMAX_H or replacing PPowerClass,CA with the sum of PPowerClass,c requires a new UE capability to indicate that the UE has this new feature. A new power class definition simply requires a new power class capability. The difference is that the former can offer more versatility than the latter. Once we introduce this feature, we don’t need to introduce new capabilities for 23dBm+26dBm for MR-DC or 23dBm+29dBm, 26dBm+29dBm for not only CA but also MR-DC etc in the future.

Observation 9: A new signalling is needed for each method. Removing PPowerClass,CA from PCMAX_H or replacing PPowerClass,CA with the sum of PPowerClass,c can offer more versatility than an introduction of a new PC, since the latter requires extra PCs in the future if RAN4 introduces 23dBm+26dBm for MR-DC or 23dBm+29dBm, 26dBm+29dBm for not only CA but also MR-DC etc in the future.

SAR

We don’t see something new that we need to especially address this feature. The same SAR method for normal PC2 uplink inter band CA can be used. Only the difference is the value contained in maxUplinkDutyCycle capability for 27.8dBm would be less than that for 26dBm. In addition, RAN4 has already specified PC2 uplink inter-band CA without waiting for the introduction of maxUplinkDutyCycle solution dedicated to uplink inter-band CA and has considered P-MPR as default solution. Therefore, P-MPR can be used as default.

Observation 10: For SAR, there is nothing particular to address for this new feature. The same SAR method for normal PC2 uplink inter band CA can be reused and the value for maxUplinkDutyCycle can be discussed . By the time the SAR method for PC2 uplink inter band CA is completed, P-MPR can be used.

MPR/MSD

The original purpose MPR is in principle band agnostic but Power class dependent. Regarding A-MPR, A-MPR itself is band and PC specific. Now what we are discussing is uplink inter-band CA where band and PC specific MPR and A-MPR apply to each band’s power class within the uplink inter-band CA regardless of the PC for uplink inter-band CA. This principle does not change. With respect to MSD, in our understanding, both MSDs due to intermodulation interference and that due to cross band isolation need to be re-evaluated. MSD due to UL harmonic harmonics does not require the evaluation.

Observation 11: For MPR/A-MPR, we can reuse the existing requirements. For MSD, re-evaluation is needed for MSD due to intermodulation interference and that due to cross band isolation.

# 5	Conclusion

In this contribution, we analysed the solution proposed in [1, 2]. As a result, we obtained following observations and a proposal.

Observation 1: If removing PPowerClass,CA from PCMAX_H, there is a backward compatibility issue. Because gNBs would consider the UEs supporting the default PC for the uplink inter-band CA. Hence, the gNBs cannot distinguish legacy UEs without this feature, i.e., PC3 and new UEs with it.

Observation 2: The backward compatibility issue in the Observation 1 can be resolved by introducing a new capability.

Observation 3: If removing PPowerClass,CA from PCMAX_H and if it means PPowerClass,CA is not reported, PCMAX_H cannot reflect UE’s real ability and/or the meaning of PC may be ambiguous.

Observation 4: If removing PPowerClass,CA from PCMAX_H and if it means PPowerClass,CA is not reported, PCMAX_L does not hold true.

Observation 5: For a UE with this new feature, by replacing pPowerClass,CA with ∑ pPowerClass,c , the UE can be handled as if the UE has a new power class without introducing it. And this can be applied to any combinations of power classes within a band combination such that 23dBm+26dBm, 23dBm+29dBm, 26dBm+29dBm etc.

Observation 6: The introduction of new power classes is not suitable to the original purpose of this discussion.

Observation 7: The introduction of power boosting is making the specification more complicated than any other listed options.

Observation 8: For possible impact on regulations, there is nothing new to be discussed. The same principle of PC2/1.5 UEs applies to the UEs with this new feature. That is, if the IE P-Max as defined in TS 38.331 is provided and set to the maximum output power of the default power class or lower, shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified in clause 6.2A.4.

Observation 9: A new signalling is needed for each method. Removing PPowerClass,CA from PCMAX_H or replacing PPowerClass,CA with the sum of PPowerClass,c can offer more versatility than an introduction of a new PC, since the latter requires extra PCs in the future if RAN4 introduces 23dBm+26dBm for MR-DC or 23dBm+29dBm, 26dBm+29dBm for not only CA but also MR-DC etc in the future.

Observation 10: For SAR, there is nothing particular to address for this new feature. The same SAR method for normal PC2 uplink inter band CA can be reused and the value for maxUplinkDutyCycle can be discussed . By the time the SAR method for PC2 uplink inter band CA is completed, P-MPR can be used.

Observation 11: For MPR/A-MPR, we can reuse the existing requirements. For MSD, re-evaluation is needed for MSD due to intermodulation interference and that due to cross band isolation.

Proposal 1: Focus on increasing UE maximum power high limit for NR uplink inter band CA under this WI and revise the WID to accommodate this topic in the objective accordingly.

Proposal 2: For a UE with this new feature, select replacing pPowerClass,CA with ∑ pPowerClass,c

# 6	References

[1] R4-2016439, Upper limits on output power for dual PA, Qualcomm Incorporated

[2] R4-2105342, WF on increasing UE maximum power high limit, Qualcomm Incorporated, OPPO, Nokia