3GPP TSG-WG4 Meeting #99-e	R4-2108893

# Online,  19th - 27th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 5741 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NB-IoT TDD RMCs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15 |  |  |  |  |  | Date: |  |  | 2021-05-25 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The notation of the repetition numbers for NB-IoT UL and DL RMCs is not aligned and creates a confusion when reading the spec. The notation needs to be aligned and should refer back to the definition of the parameter to avoid misunderstandings. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Aligned wording on number of repetitions for NPDSCH and NPUSCH. Add reference to 36.213. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect requirements remain in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 36.521-1 CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

Table A.3.2-2e Fixed Reference Channel for Receiver Requirements (TDD) – for CAT-NB1 and CAT-NB2

| Parameter |  |  |  | Unit | Value |
| --- | --- | --- | --- | --- | --- |
| Channel bandwidth |  | MHz | 0.2 |
| Number of subcarriers |  |  | 12 |
| Uplink-Downlink Configuration (Note 7) |  |  | 1 |
| Modulation |  |  | QPSK |
| Target Coding Rate |  |  | 1/3 |
| Number of HARQ Processes |  | Processes | 1 |
| Maximum number of HARQ transmissions |  |  | 1 |
| Transport block size |  | Bits | 88 |
| Number of Sub-Frames per transport block |  |  | 1 |
| Transport block CRC |  | Bits | 24 |
| Binary Channel Bits Per Sub-Frame |  | Bits | 320 |
| LTE CRS port |  |  | N/A |
| Number of NRS ports |  |  | 1 |
| Number of NPDSCH repetitions |  |  | 1 |
| UE DL Category |  |  | NB1 or NB2 |
| Note 1: Category NB1 or NB2 in stand-alone mode has been considered here. Note 2: Reference signal, Synchronization signals and NPBCH allocated as per TS 36.211. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) Note 4: Parameters related to NPDSCH scheduling are defined in Table A.3.2-1e to Table A.3.2-1g. Note 5: NPDCCH and information bit payload are not transmitted in the subframes used for transmission of SI messages. Note 6: SI messages transmission should be prioritized over NPDCCH transmission in case of collision. NPDCCH transmission is postponed until the next NB-IoT downlink subframe in case NPDCCH transmission occurs in a non NB-IoT downlink subframe, where an NB-IoT downlink subframe is a subframe that does not contain NPSS/NSSS/NPBCH/SIB1-NB transmission. Note 7: As per Table 4.2-2 in TS 36.211 [4] Note 8: Number of repetition NRep as defined in table 16.4.1.3-2 in TS 36.213 [6]. |  |  |  |  |  |

< End of changes >