3GPP TSG-RAN WG4 Meeting#99      	R4-2108935

E-meeting, 19th – 27th  May, 2021

Title:	TP to TR 38.717-03-01: CA_n2-n30-n66

Source:	Nokia, AT&T

Agenda item:	8.11.2

Document for:	Approval

# Introduction

This is a TP into TR 38.717-03-01 to introduce CA_n2-n30-n66.

********************** Start of TP *********************************

## 6.7	CA_n2-n30-n66

### 6.7.1	Operating bands for CA

Table 6.7.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n2-n30-n66 | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n30 | 2305 MHz | – | 2315 MHz | 2350 MHz | – | 2360 MHz | TDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

### 6.7.2	Channel bandwidths per operating band for CA

Table 6.7.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA Configuration | UL Config | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | BCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n2A-n30A-n66A | - | n30 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 |  |  | 40 |  |  |  |  |  |  |  |
|  |  | n2 | See CA_n2(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n2(2A)-n30A-n66A | - | n30 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 |  |  | 40 |  |  |  |  |  |  |  |
|  |  | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n2A-n30A-n66(2A) | - | n30 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.7.3	Co-existence studies

Table 6.7.3-1 summarizes frequency ranges where harmonics occur due to 3DL bands CA with 1 UL. There are no harmonic interference.

Table 6.7.3-1: Harmonic Interference for 3DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 2 | 1850 | 1910 | 1930 | 1990 | 3700 | 3820 | 5550 | 5730 | 7400 | 7640 |
| 30 | 2305 | 2315 | 2350 | 2360 | 4610 | 4630 | 6915 | 6945 | 9220 | 9260 |
| 66 | 1710 | 1780 | 2110 | 2200 | 3420 | 3560 | 5130 | 5340 | 6840 | 7120 |

Table 6.7.3-2 gives harmonic mixing issue for the 3DL bands CA with 1 UL. There are no harmonic mixing interference..

Table 6.7.3-2 Harmonic mixing for 3DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 2 | 1850 | 1910 | 1930 | 1990 | 3860 | 3980 | 5790 | 5970 | 7720 | 7960 |
| 30 | 2305 | 2315 | 2350 | 2360 | 4700 | 4720 | 7050 | 7080 | 9400 | 9440 |
| 66 | 1710 | 1780 | 2110 | 2200 | 4220 | 4400 | 6330 | 6600 | 8440 | 8800 |

For single uplink, the UE coexistence is already considered for these bands in TS 38.101-1.

### 6.7.4	∆TIB,c and ∆RIB,c values

It is proposed to re-use TIB,c and RIB,c  values from DC_2-30_n66.

Table 6.7.4-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n2-n30-n66 | n2 | 0.5 |
|  | n30 | 0.3 |
|  | n66 | 0.5 |

Table 6.7.4-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n2-n30-n66 | n2 | 0.4 |
|  | n30 | 0.5 |
|  | n66 | 0.4 |

### 6.7.5	REFSENS requirements

Compared to its fallback modes, there are no additional MSD requirements for this band combination.

********************** End of TP *********************************