3GPP RAN WG4 Meeting #99e		R4-2109131

Electronic Meeting, May 19-27, 2021

Agenda item:	8.2.1.2

Source:	Murata Manufacturing Co., Ltd.

Title:	Multi-band relaxation for band n262

WI/SI:	NR_47GHz_Band-Core

Release:	Rel-17

Document for:	Approval

1	Introduction

The work item introducing the new FR2 47GHz band was approved in RAN #88-e [1]. The values of min peak EIRP, spherical coverage, and MBR of 39GHz and 47GHz were determined in RAN #98-e [2]. And, it is noted that there might be further discussion for UEs only supporting 28/47GHz. This contribution shows the simulation result of antenna supporting 28/47GHz.

2	Discussion

We investigated the following two type antennas for this discussion.

- 28GHz single antenna which supports n257/n258.

- 28/47GHz stack antenna which supports n257/n258 and n262.

Antenna structure is assumed as the Figure 1.

Figure 1. Dimension of antenna

The following simulated result shows antenna gain at peak and 50%-tile CDF.

Table 1. Peak and 50%-tile CDF gain of 28/47GHz antenna

|  | 28GHz antenna |  | 28/47GHz antenna |  |  |
| --- | --- | --- | --- | --- | --- |
|  | n257 | n258 | n257 | n258 | n262 |
| Peak | 9.7dBi | 8.9dBi | 9.5dBi | 8.3dBi | 10.0dBi |
| 50%-tile | -1.5dBi | 0.1dBi | -0.5dBi | 0.4dBi | -2.7dBi |

Observation 1:	The decreased peak and 50%-tile CDF gain from 28GHz antenna to 28/47GHz antenna are smaller than MBR value, in band n257 and n258

| 28GHz antenna | 28/47GHz antenna |
| --- | --- |

Figure 2. CDF plots of each antennas

In the past discussion, we can check the peak gain by seeing realized antenna gain. And we can calculate 50%-tile CDF gain by subtracting EIRP gain drop from realized antenna gain. Table 2 shows the summary of past discussion results.

Table 2. Discussed peak and 50%-tile CDF gain of 28/47GHz antenna

|  | 28GHz antenna from [3] |  |  | 47GHz antenna from [2] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | min | average | max | min | average | max |
| Peak | 7.5dBi | 8.7dBi | 10.0dBi | 7.5dBi | 8.7dBi | 10.5dBi |
| 50%-tile | -3.4dBi | -2.2dBi | -0.9 | -5.6dBi | -4.4dBi | -2.6dBi |
| EIRP drop | 10.9dB |  |  | 13.1dB |  |  |

All simulated values of 28GHz antenna in Table 1 are higher than those average values of Table 2. Also, simulated n262 values in Table 1 are higher than those average values of Table 2, and it is close to maximum value, so we think this antenna could achieve enough performance in band n262.

Observation 2:	28/47GHz dual band antenna could achieve enough performance

Proposal 1:	There is no need changing MBR specification (MBP,n=0.7dB, MBS,n=0.7dB)

3	Conclusions

This contribution provides our view on the multi-band relaxation for n257/n258 and n262:

Observation 1:	The decreased peak and 50%-tile CDF gain from 28GHz antenna to 28/47GHz antenna are smaller than MBR value, in band n257 and n258

Observation 2:	28/47GHz dual band antenna could achieve enough performance

Proposal 1:	There is no need changing MBR specification (MBP,n=0.7dB, MBS,n=0.7dB)

4	References

RP-201129 "New WID on introduction of NR 47 GHz band”, T-Mobile

R4-2103312 “Email discussion summary for [98e][124] NR_47GHz_Band”, Nokia

R4-1712319 “UE power class for FR2”, Intel Corporation, Apple, OPPO, Samsung, LGE, Xiaomi, MediaTek, Motorola, Huawei