3GPP RAN WG4 Meeting #99e		R4-2109147

Electronic Meeting, 19-27 May, 2021

Agenda item:	8.41.1

Source:	Murata Manufacturing Co., Ltd.

Title:	On new FWA UE RF requirement

WI/SI:	NR_FR2_FWA_Bn259-Core

Release:	Rel-17

Document for:	Approval

1	Introduction

The work item introducing PC5 for band n259 was approved in RAN #91-e [1]. It was discussed during RAN4 #98bis-e [2] and a WF was approved [3] where companies are encouraged to provide the values for min peak EIRP and REFSENS. In this contribution we provide our inputs to min peak EIRP and REFSENS values for PC5 in band n259.

2	Discussion

In order to investigate minimum peak EIRP, we need to consider network performance as well as device characteristics point of view. Regarding network performance, in [4], the contribution showed that over 28dBm minimum peak EIRP would be preferred. And in [5], the contribution showed that minimum peak EIRP should be over 30dBm considering the link budget between UE and BTS. Though these contributions described network performance around 28GHz frequency range, we should consider that propagation loss is increased in n259 from that of n257/n258.

On the other hand, from device characteristics point of view, antenna elements assumption is very important to estimate minimum peak EIRP. Then we investigated the both 8 and 16 antenna elements cases for PC5 link budget as below.

2.1	Minimum peak EIRP

Table 1 shows Tx link budget to estimate minimum peak EIRP for band n259.

Table 1. The minimum peak EIRP evaluation for PC5 in band n259.

| Parameter | Unit | Nominal | Tolerance | Nominal | Tolerance |
| --- | --- | --- | --- | --- | --- |
| Frequency range | GHz | 39.5-43.5 |  | 39.5-43.5 |  |
| Pout per element | dBm | 11.0 |  | 11.0 |  |
| # of antennas in an array |  | 8 |  | 16 |  |
| Total conducted power per polarization | dBm | 20.0 |  | 23.0 |  |
| Avg antenna element gain | dBi | 4.0 |  | 4.0 |  |
| Antenna roll-off loss versus frequency | dB | 2.0 |  | 2.0 |  |
| Realized antenna array gain | dBi | 11.0 |  | 14.0 |  |
| Polarization gain | dB | 2.5 |  | 2.5 |  |
| Mismatch and transmission line loss including load pull | dB | 2 |  | 2 |  |
| Beam forming loss (phase shifter and amplitude error) | dB | 1 |  | 1 |  |
| Finite beam table | dB | 0.2 |  | 0.3 |  |
| Beam forming loss (one beam table fits all) | dB | 0.5 |  | 0.5 |  |
| Form factor integration losses | dB | 3.5 |  | 3.5 |  |
| Total implementation loss(nominal) | dB | 7.2 |  | 7.3 |  |
| Total implementation loss (worst case) | dB |  |  |  |  |
| Peak EIRP(Nominal) | dBm | 26.4 |  | 32.3 |  |
| Tolerance (+/-) | dB | 3.0 |  | 3.0 |  |
| Peak EIRP(Minimum) | dBm | 23.4 |  | 29.3 |  |
| Peak EIRP(Maximum) | dBm |  |  |  |  |

According to the above table, with 8 elements assumption, it is quite difficult to achieve over 28dBm minimum peak EIRP, so we think 16 elements assumption would be preferred.

Observation 1:	From network performance point of view, 16 elements assumption would be preferred

Proposal 1: 	29.3dBm is proposed as min peak EIRP requirement

2.2	REFSENS

Based on minimum peak EIRP investigation, we set the number of antenna elements as 16 for EIS estimation. Then the minimum peak EIS evaluation for PC5 in band n259 is shown in Table2.

Table 2. The minimum peak EIS evaluation for PC5 in band n259.

| Parameter | Unit | Value |
| --- | --- | --- |
| Band number |  | n259 |
| Frequency range | GHz | 39.5-43.5 |
| Modulation |  | QPSK |
| SNR requirement | dB | -1 |
| Bandwidth | MHz | 50 |
| Thermal noise | dBm/Hz | -174 |
| Noise Figure | dB | 12.5 |
| Number of antenna gain array |  | 16 |
| Array gain | dB | 14.0 |
| Element gain | dBi | 4 |
| Diversity gain | dB | 0 |
| Antenna gain roll-off over frequency | dB | 2 |
| Beamforming loss | dB | 1.8 |
| Total insertion loss | dB |  |
| [Mismatch and transmission line loss] | dB | 1 |
| [Form factor Integration losses] | dB | 3.5 |
| [Implementation loss] | dB | 6.3 |
| [Packaging loss] | dB |  |
| Tolerance | dB | 3 |
| REFSENS (50MHz) | dBm | -90.3 |
| REFSENS (100MHz) | dBm |  |
| REFSENS (200MHz) | dBm |  |
| REFSENS (400MHz) | dBm |  |

Proposal 2:	-90.3dBm is proposed as REFSENS specification

3	Conclusions

This contribution provides our view on the min peak EIRP and REFSENS values for PC5 in band n259.

Observation 1:	From network performance point of view, 16 elements assumption would be preferred

Proposal 1: 	29.3dBm is proposed as min peak EIRP requirement

Proposal 2:	-90.3dBm is proposed as REFSENS specification

4	References

RP-210875, “New WID on Introduction of FR2 FWA UE with maximum TRP of 23dBm for band n259,” SoftBank Corp., KDDI, NTT DOCOMO, Rakuten Mobile, Qualcomm, RAN #91e, March 2021

R4-2104493, "PC5 RF requirements in n259”, Qualcomm Incorporated

R4-2105462, " Email discussion summary for [98-bis-e][125] NR_FR2_FWA_Bn259”, Qualcomm Incorporated

R4-2011458, “On Japan FWA EIRP requirement”, Qualcomm Incorporated, RAN4#96-e, August 2020

R4-2011488, “On new FWA UE RF requirement for FR2”, Huawei, HiSilicon, RAN4#96-e, August 2020