3GPP TSG-RAN WG4 Meeting #99-e		R4-2109210

Electronic, 19nd – 27th May, 2021

Agenda item:	6.3.6

Source:	Intel Corporation

Title:	TP to TS 38.176-2: Demodulation manufacturer declarations

Document for:	Approval

# 1	Introduction

As discussed in [1], we proposed the following TP to TS 38.176-2 to define IAB manufacturer declarations that correspond to demodulation capabilities.

# 2	Text Proposal to TS 38.176-2

Start of the text proposal

## 4.6	Manufacturer's declarations

The following IAB manufacturer's declarations listed in tables 4.6-1, 4.6-2 when applicable to the IAB under test, are required to be provided by the manufacturer for radiated requirements testing for IAB type 1-H, IAB type 1-O and IAB type 2-O for IAB-DU and IAB-MT node respectively.

For the IAB type 1-H declarations required for the conducted requirements testing, refer to TS 38.176-1 [TBA], clause TBA.

Table 4.6-1 Manufacturers declarations for IAB-DU type 1-H, IAB-DU type 1-O and IAB-DU type 2-O radiated test requirements

| Declaration identifier | Declaration | Description | Applicability (Note 1) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | IAB type 1-H (Note 2) | IAB type 1-O | IAB type 2-O |
| D.TBA | PUSCH mapping type | Declaration of the supported PUSCH mapping type for FR1 as specified in [TBA], i.e., type A, type B or both. | c | x | n/a |
| D.TBA | PUSCH additional DM-RS positions | Declaration of the supported additional DM-RS position(s) for FR2, i.e., pos0, pos1, or both. | n/a | n/a | x |
| D.TBA | PUCCH format | Declaration of the supported PUCCH format(s) as specified in [20], i.e., format 0, format 1, format 2, format 3, format 4. | c | x | x |
| D.TBA | PRACH format and SCS | Declaration of the supported PRACH format(s) as specified in [TBA], i.e., format: 0, A1, A2, A3, B4, C0, C2. Declaration of the supported SCS(s) per supported PRACH format with short sequence, as specified in [TBA], i.e.: - For IAB type 1-O: 15 kHz, 30 kHz or both. - For IAB type 1-O: 60 kHz, 120 kHz or both. | c | x | x |
| D.TBA | Additional DM-RS for PUCCH format 3 | Declaration of the supported additional DM-RS for PUCCH format 3: without additional DM-RS, with additional DM-RS or both. | c | x | x |
| D.TBA | Additional DM-RS for PUCCH format 4 | Declaration of the supported additional DM-RS for PUCCH format 4: without additional DM-RS, with additional DM-RS or both. | c | x | x |
| D.TBA | PUSCH PT-RS | Declaration of PT-RS in PUSCH support: without PT-RS, with PT-RS or both. | n/a | n/a | x |
| D.TBA | PUCCH multi-slot | Declaration of multi-slot PUCCH support. | c | x | n/a |
| D.TBA | UL CA | For the highest supported SCS, declaration of the carrier combination with the largest aggregated bandwidth. If there is more than one combination, the carrier combination with the largest number of carriers shall be declared. | c | x | x |
| D.TBA | Modulation order | Declaration of the supported modulation orders: QPSK, 16QAM, 64QAM | TBA | TBA | TBA |
| D.TBA | Transorm precoding | Declararation on the supporting of transform precoding | TBA | TBA | TBA |
| Note 1: Manufacturer declarations applicable per IAB requirement set were marked as "x". Manufacturer declarations not applicable per IAB requirement set were marked as "n/a". |  |  |  |  |  |

Table 4.6-2 Manufacturers declarations for IAB-MT type 1-H, IAB type 1-O and IAB type 2-O radiated test requirements

| Declaration identifier | Declaration | Description | Applicability (Note 1) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | IAB type 1-H (Note 2) | IAB type 1-O | IAB type 2-O |
| D.TBA | 256QAM | Declaration on the supporting of 256QAM modulation order | TBA | TBA | TBA |
| Note 1: Manufacturer declarations applicable per IAB requirement set were marked as "x". Manufacturer declarations not applicable per IAB requirement set were marked as "n/a". |  |  |  |  |  |

End of the text proposal

# References

- R4-2103994 “WF on Rel-16 NR IAB demodulation requirements”, Nokia, Nokia Shanghai Bell, RAN4 #98e, February 2021