3GPP TSG-<TSG/RAN4> Meeting #99e	R4-2109419

# Electronic Meeting,  19th  -  27th May,2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | <CR#> | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for specifying REFSENS based on a unified equation method |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR1_35MHz_45MHz_BW-Core |  |  |  |  |  | Date: |  |  | 2021-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As a simplification, the proposal specifies UE REFSENS based on a unified equation for the sake of readibility and future proof, thus simplifies the REFSENS table. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Simplify the REFSENS table and add two notes (NOTE 8/9) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The REFSENS table is too big, unreadible, not future proof either. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change>

7.3.2	Reference sensitivity power level

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A.2.3.2, A3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.2-1 and Table 7.3.2-2.

Table 7.3.2-1: Two antenna port reference sensitivity QPSK PREFSENS

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| REFSENS (dBm) = 10*log10(N_RB*12*SCS(kHz)/4500)+∆R-100 |  |  |  |  |  |  |  |  |  |  |
| Operating Band |  |  | SCS kHz |  | Channel bandwidth (MHz) |  | ∆R |  | Duplex Mode |  |
| n1 | 15 | 5,10,15,20,25,30,40,50 | 0 | FDD |
|  | 30 | 10,15,20,25,30,40,50 |  |  |
|  | 60 | 10,15,20,25,30,40,50 |  |  |
| n2 | 15 | 5,10,15,20 | 2 | FDD |
|  | 30 | 10,15,20 |  |  |
|  | 60 | 10,15,20 |  |  |
| n3 | 15 | 5,10,15,20,25,30,40 | 3 | FDD |
|  | 30 | 10,15,20,25,30,40 |  |  |
|  | 60 | 10,15,20,25,30,40 |  |  |
| n5 | 15 | 5,10,15,20 | 2 | FDD |
|  | 30 | 10,15,20 |  |  |
|  | 60 |  |  |  |
| n71 | 15 | 5,10,15,20,25,30,40,50 | 2 | FDD |
|  | 30 | 10,15,20,25,30,40,50 |  |  |
|  | 60 | 10,15,20,25,30,40,50 |  |  |
| n8 | 15 | 5,10,15,20 | 3 | FDD |
|  | 30 | 10,15,20 |  |  |
|  | 60 |  |  |  |
| n12 | 15 | 5,10,15 | 3 | FDD |
|  | 30 | 10,15 |  |  |
|  | 60 |  |  |  |
| n14 | 15 | 5,10 | 3 | FDD |
|  | 30 | 10 |  |  |
|  | 60 |  |  |  |
| n18 | 15 | 5,10,15 | 0 | FDD |
|  | 30 | 10,15 |  |  |
|  | 60 |  |  |  |
| n20 | 15 | 5,10,15,20 | 3 | FDD |
|  | 30 | 10,15,20 |  |  |
|  | 60 |  |  |  |
| n25 | 15 | 5,10,15,20,25,30,40 | 3.5 | FDD |
|  | 30 | 10,15,20,25,30,40 |  |  |
|  | 60 | 10,15,20,25,30,40 |  |  |
| n26 | 15 | 5,10,15,20 | 2.44 | FDD |
|  | 30 | 10,15,20 |  |  |
| n28 | 15 | 5,10,15,20,30 | 1.5 | FDD |
|  | 30 | 10,15,20,30 |  |  |
|  | 60 |  |  |  |
| n29x | 15 | 5,10 | 3 | SDL |
|  | 30 | 10 |  |  |
|  | 60 |  |  |  |
| n30 | 15 | 5,10 | 1 | FDD |
|  | 30 | 10 |  |  |
|  | 60 |  |  |  |
| n34 | 15 | 5,10,15 | 0 | TDD |
|  | 30 | 10,15 |  |  |
|  | 60 | 10,15 |  |  |
| n381 | 15 | 5,10,15,20,25,30,40 | 0 | TDD |
|  | 30 | 10,15,20,25,30,40 |  |  |
|  | 60 | 10,15,20,25,30,40 |  |  |
| n39 | 15 | 5,10,15,20,25,30,40 | 0 | TDD |
|  | 30 | 10,15,20,25,30,40 |  |  |
|  | 60 | 10,15,20,25,30,40 |  |  |
| n40 | 15 | 5,10,15,20,25,30,40,50 | 0 | TDD |
|  | 30 | 10,15,20,25,30,40,50,60,80 |  |  |
|  | 60 | 10,15,20,25,30,40,50,60,80 |  |  |
| n411 | 15 | 10,15,20,30,40,50 | 2 | TDD |
|  | 30 | 10,15,20,30,40,50,60,80,90,10 |  |  |
|  | 60 | 10,15,20,30,40,50,60,80,90,10 |  |  |
| n481 | 15 | 5, 10, 15, 20, 40, 505 | 1 | TDD |
|  | 30 | 10, 15, 20, 40, 505, 605, 705, 805, 905, 1005 |  |  |
|  | 60 | 10, 15, 20, 40, 505, 605, 705, 805, 905, 1005 |  |  |
| n50 | 15 | 5, 10, 15, 20, 30, 40, 50 | 0 | TDD |
|  | 30 | 10, 15, 20, 30, 40, 50, 60, 80 |  |  |
|  | 60 | 10, 15, 20, 30, 40, 50, 60, 80 |  |  |
| n51 | 15 | 5 | 0 | TDD |
|  | 30 |  |  |  |
|  | 60 |  |  |  |
| n53 | 15 | 5, 10 | 0 | TDD |
|  | 30 | 10 |  |  |
|  | 60 | 10 |  |  |
| n65 | 15 | 5,10,15,20,50 | 0.5 | FDD |
|  | 30 | 10,15,20,50 |  |  |
|  | 60 | 10,15,20,50 |  |  |
| n66 | 15 | 5,10,15,20,25,30,40 | 0.5 | FDD |
|  | 30 | 10,15,20,25,30,40 |  |  |
|  | 60 | 10,15,20,25,30,40 |  |  |
| n70 | 15 | 5,10,15,20,25 | 0 | FDD |
|  | 30 | 10,15,20,25 |  |  |
|  | 60 | 10,15,20,25 |  |  |
| n71 | 15 | 5,10,15,20 | 2.8 | FDD |
|  | 30 | 10,15,20 |  |  |
|  | 60 |  |  |  |
| n74 | 15 | 5,10,15,20 | 0.47 | FDD |
|  | 30 | 10,15,20 |  |  |
|  | 60 | 10,15,20 |  |  |
| n757 | 15 | 5,10,15,20,25,30,40,50 | 0 | SDL |
|  | 30 | 10,15,20,25,30,40,50 |  |  |
|  | 60 | 10,15,20,25,30,40,50 |  |  |
| n767 | 15 | 5 | 0 | SDL |
|  | 30 |  |  |  |
|  | 60 |  |  |  |
| n771,4 | 15 | 10, 15, 20, 25, 30, 40, 50 | 1.5 | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| n781 | 15 | 10, 15, 20, 25, 30, 40, 50 | 1 | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| n791 | 15 | 40, 50 | 1 | TDD |
|  | 30 | 40, 50, 60, 80, 100 |  |  |
|  | 60 | 40, 50, 60, 80, 100 |  |  |
| n91 | 15 | 5 | 0 | FDD |
|  | 30 |  |  |  |
|  | 60 |  |  |  |
| n92 | 15 | 5,10,15,20 | 0 | FDD |
|  | 30 | 10,15,20 |  |  |
|  | 60 |  |  |  |
| n93 | 15 | 5 | 0 | FDD |
|  | 30 |  |  |  |
|  | 60 |  |  |  |
| n94 | 15 | 5,10,15,20 | 0 | FDD |
|  | 30 | 10,15,20 |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. |  |  |  |  |  |  |  |  |  |  |
| NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4 |  |  |  |  |  |  |  |  |  |  |
| NOTE 3: The requirement is modified by -0.5 dB when the assigned NR channel bandwidth is confined within 1475.9 - 1510.9 MHz. |  |  |  |  |  |  |  |  |  |  |
| NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. |  |  |  |  |  |  |  |  |  |  |
| NOTE 5: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. |  |  |  |  |  |  |  |  |  |  |
| NOTE 6: Values are modified by -0.5dB when carrier channel BW is between 865MHz and 894MHz. |  |  |  |  |  |  |  |  |  |  |
| NOTE 7: For SDL bands, the reference sensitivity requirements shall be verified by inter-band CA combinations with SDL band, which are supported by UE. |  |  |  |  |  |  |  |  |  |  |
| NOTE 8: NRB in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. NOTE 9: The REFSENS value is rounded to the nearest number down to one decimal point. |  |  |  |  |  |  |  |  |  |  |

<End of Change>