3GPP TSG-RAN WG4 Meeting #99-e	R4-2109595

Electronic meeting, 19th – 28th May, 2021

Source:	Ericsson

Title:	Simulation results for NR-U PRACH demodulation requirements

Agenda item:	6.1.7.4.4

Document for:	information

# Introduction

In RAN4#98-bis-e meeting, there are some simulation results are not available in our contributions. In this contribution, we re-submit all simulation results with missed ones.

# Simulation assumptions and results

Table 2.1.1 Simulation assumptions for large bandwidth PRACH

| PRACH | PRACH SCS | Time error tolerance |  |  |  |
| --- | --- | --- | --- | --- | --- |
| preamble | (kHz) | AWGN | TDLA30-10 | Lra | Ncs |
| A2, B4, C2 | 15 | 0.52 us | 0.67 us | 1151 | 164 |
|  | 30 | 0.26 us | 0.41 us | 571 | 190 |

The simulation for format B4 under TDLA30-10 is still ongoing by this contribution delivery.

Table 2.1.2 Ideal simulation results for PRACH with 15kHz SCS

| Number of |  | Propagation | Frequency | SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| TX antennas | Number of RX antennas | conditions and correlation matrix (Annex G) | offset | Burst format A2 | Burst format B4 | Burst format C2 |
| 1 | 2 | AWGN | 0Hz | -24.2 | -26.8 | -24.2 |
|  |  | TDLA30-10 Low | 400 Hz | -17.1 |  | -17.1 |

Table 2.1.3 Impairment simulation results for PRACH with 15kHz SCS

| Number of |  | Propagation | Frequency | SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| TX antennas | Number of RX antennas | conditions and correlation matrix (Annex G) | offset | Burst format A2 | Burst format B4 | Burst format C2 |
| 1 | 2 | AWGN | 0Hz | -21.7 | -24.3 | -21.7 |
|  |  | TDLA30-10 Low | 400 Hz | -14.6 |  | -14.6 |

Table 2.1.4 Ideal simulation results for PRACH with 30kHz SCS

| Number of |  | Propagation | Frequency | SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| TX antennas | Number of RX antennas | conditions and correlation matrix (Annex G) | offset | Burst format A2 | Burst format B4 | Burst format C2 |
| 1 | 2 | AWGN | 0Hz | -21.3 | -25.5 | -21.3 |
|  |  | TDLA30-10 Low | 400 Hz | -14.6 | -17.1 | -14.6 |

Table 2.1.5 Impairment simulation results for PRACH with 30kHz SCS

| Number of |  | Propagation | Frequency | SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| TX antennas | Number of RX antennas | conditions and correlation matrix (Annex G) | offset | Burst format A2 | Burst format B4 | Burst format C2 |
| 1 | 2 | AWGN | 0Hz | -18.8 | -23.0 | -18.8 |
|  |  | TDLA30-10 Low | 400 Hz | -12.1 | -14.6 | -12.1 |

# Conclusion

In this contribution, we deliver our simulation results based on the WF [1] in RAN4#98-e.

# References

[1]	R4-2103808, “Way forward on NR-U PRACH demodulation requirements”, RAN4#98-e, Nokia, Nokia Shanghai Bell