3GPP TSG RAN WG4 Meeting #99-e	R4-2109918

Electronic Meeting, 19th – 27th May, 2021

Agenda item:	8.17.2

Source:	LG Electronics

Title:	MSD results for PC3 NR inter-band DC band combinations

Document for:	Approval

1	Introduction

In the latest RAN Plenary, some new DC band are included to support PC3 NR inter-band DC band combinations in Rel-17. In here the DC_1A-11A_n41A has some self interference problem as follow

Table 1 coexistence analysis for NR DC band combinations in Rel-17

| DC band combinations | UL-Config | Victim DL | IMD | Additional Analysis |
| --- | --- | --- | --- | --- |
| DC_1A-11A_n41A | DC_1A_n41A | Band 11 | IMD4 | Need to define MSD in Band 11 by IMD4 |
|  | DC_11A_n41A | Band 1 | IMD4 | Need to define MSD in Band 1 by IMD4 |

Based on the coexistence analysis result in Table 1, we provide the MSD requiremeents for NR DC band combination with self interference issues.

2. MSD proposal

2.1 Inter-modulation problems by dual uplink

In this session, we analyze the required MSD values in the own Rx band by IMD product from dual uplink transmission. In table 2 and Table 3, we show the basic RF simulation assumptions.

Table 2: UE RF Front-end component parameters

| UE ref. architecture Component | Diplexer-Duplexer Architecture w/ single ant. |  |  |  |
| --- | --- | --- | --- | --- |
|  | DCA_1A-11A_n41A |  |  |  |
|  | IP2 (dBm) | IP3 (dBm) | IP4 (dBm) | IP5 (dBm) |
| Ant. Switch | 112 | 68 | 55 | 55 |
| Triplexer | 110 | 72 | 55 | 52 |
| Diplexer | 115 | 87 | 55 | 55 |
| Duplexer | 100 | 75 | 55 | 53 |
| Duplexer (n74) | 95 | 70 | 50 | 50 |
| PA Forward | 28.0 | 32 | 30 | 28 |
| PA Reversed | 40 | 30.5 | 30 | 30 |
| LNA | 10 | 0 | 0 | -10 |

Table 3 show the isolation levels according to the RF component.

Table 3: UE RF Front-end component isolation parameters

| Isolation Parameter | Value (dB) | Comment |
| --- | --- | --- |
| Antenna to Antenna | 10 | Main antenna to diversity antenna |
| PA (out) to PA (in) | 60 | PCB isolation (PA forward mixing) |
| Triplexer | 20 | High/low band isolation |
| Diplexer | 25 | High/low band isolation |
| PA (out) to PA (out) | 60 | L-H/H-L cross-band |
| PA (out) to PA (out) | 50 | H-H cross-band |
| LNA (in) to PA (out) | 60 | L-H/H-L cross-band |
| LNA (in) to PA (out) | 50 | H-H cross-band |
| Duplexer | 50 | Tx band rejection at Rx band |

Based on above assumptions and test configuration, we proposed the MSD levels as below.

Table 4: Proposed MSD test configuration and results for self desense problems

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-11A_n41A | B1 | IMD4 | ¦2*fB1 –2*fn41¦ | 1940 | 5 | 25 | 2130 | 5 | N/A |
|  | n41 |  |  | 2685 | 10 | 50 | 2685 | 10 | N/A |
|  | B11 |  |  | 1442 | 5 | 25 | 1490 | 5 | 10.6 |
|  | B11 | IMD4 | ¦2*fB11 –2*fn41¦ | 1442 | 5 | 25 | 1490 | 5 | N/A |
|  | n41 |  |  | 2520 | 10 | 50 | 2520 | 10 | N/A |
|  | B1 |  |  | 1966 | 5 | 25 | 2156 | 5 | 10.2 |

3	Conclusion

Above simulation results, we recommend to merged the other interested companies’ simulation results as average manner to derive MSD levels and propose as follow

Proposal 1: For IMD problem by dual uplink transmission of PC3 NR inter band DC UE, the proposed MSD values in Table 4 shall be considered in TS38.101-1.