3GPP TSG-RAN WG4 Meeting # 99-e		                       R4-2110175

Electronic Meeting, May. 19-27, 2021

Title: 	Discussion on channel bandwidth for newly introduced SL bands

Source: 	CATT

Agenda item:	9.14.3

Document for:	Approval

- Introduction

In RAN4#98-bis-e meeting, the CBW and system parameters for the licensed bands were preliminarily discussed. The general principle on CBW and system parameters was agreed as per the WF [1]. The approved TP [2] captures the system parameters for the licensed bands but without CBW. This TP will capture the CBW for NR SL enhancement.

***********************************Start of the TP*************************************

7	Operating bands and channel arrangement for SL enhancement

7.1	Operating bands

7.1.1	Operating bands

NR SL enhancement is designed to operate in the operating bands in FR1 defined in Table 7.1.1-1.

Table 7.1.1-1 operating bands in FR1 for NR SL enhancement

| V2X Operating Band | Sidelink (SL) Transmission operating band |  |  | Sidelink (SL) Reception operating band |  |  | Duplex Mode | Interface |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |  |
| n141 | 788 MHz | - | 798 MHz | 758 MHz | - | 768 MHz | FDD | PC5 |
| n792 | 4400 MHz | - | 5000 MHz | 4400 MHz | - | 5000 MHz | TDD | PC5 |
| Note 1: When this band is used for public safety service, the band is exclusively used for NR V2X in out-of-coverage scenario. Note 2: NR V2X service is partially operated in this band with NR Uu. |  |  |  |  |  |  |  |  |

7.1.2	Operating bands for inter-band con-current operation in FR1

The operating bands for inter-band V2X con-current operation specified in clause 5.2E.2 are applied for NR SL enhancement.

7.2	Channel bandwidth

7.2.1	Channel bandwidth

The operating bands and channel bandwidth for NR SL enhancement in FR1 are shown in Table 7.2.1-1. The same (symmetrical) channel bandwidth is specified for both the TX and RX path.

Table 7.2.1-1 V2X channel bandwidth

| V2X band / SCS/ V2X channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V2X Operating Band | SCS kHz | 5 MHz | 10 MHz | 20 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n14 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |
| n79 | 15 |  |  |  |  | Yes |  |  |  |  |  |
|  | 30 |  |  |  |  | Yes |  |  |  |  |  |
|  | 60 |  |  |  |  | Yes |  |  |  |  |  |

7.2.2	Channel bandwidth for inter-band con-current operation

The channel bandwidth for inter-band V2X con-current operation specified in clause 5.3E.2 in TS 38.101-1 is applied for NR SL enhancement.

***********************************End of the TP**************************************