3GPP TSG-RAN WG4 Meeting #99-e                                   R4-2110204

Electronic Meeting, May. 19-27, 2021

Agenda Item:	6.2.4.2.3

Source:	MediaTek Inc.

Title:	Simulation results for NR V2X PSBCH test case

Document for:		Discussion

- Introduction

In last RAN4 e-meeting, the simulation assumptions of PSBCH were agreed [1]. In the contribution, we will provide initial simulation results of PSBCH based on simulation assumption.

- Discussion

Simulation assumptions

The simulation assumptions are showed as following:

Table 3 Simulation assumption for PSBCH demodulation performance

| Parameters | Unit | Value |
| --- | --- | --- |
| Test |  | BCH_Test1 |
| SLID |  | 0 |
| Synchronization source |  | SLSS |
| Propagation condition |  | TDLA30-180 |
| Channel bandwidth | MHz | 20 |
| Number SSB per SL period |  | 1 |
| Allocated resource blocks |  | 11 |
| Subcarrier spacing | kHz | 30 |
| Timing offset (Note2) |  | 0 |
| Frequency offset (Note 3) | Hz | 0 |
| The number of symbols (Note 1) |  | 9 |
| Modulation order |  | QPSK |
| Transport Block Size (without CRC) | Bits | 32 |
| Transport block CRC | Bits | 24 |
| Binary Channel Bits | Bits | 1782 |
| Antenna configuration |  | 1x2 Low |
| Performance metric |  | SNR@1% BLER of PSBCH |
| Note 1: The first OFDM symbol is used for AGC, not included in PSBCH decoding. OFDM symbols used for PSS/SSS transmission and guard symbol are not included either. Note 2: Time offset of Sidelink UE receive signal with respect to GNSS reference timing. Note 3: Frequency offset of Sidelink UE with respect to GNSS reference frequency. |  |  |

Simulation results

Based on the simulation assumptions, we provide simulation results in Figure 1.

Figure 1 PSBCH simulation result

According to the simulation results, we can see that the corresponding SNR targeted 1% BLER is about -2.9dB.

Proposal 1: The required SNR targeted 1% BLER for PSBCH test case is about -2.9dB.

- Summary

In this contribution, we provide the PSBCH simulation result and make the following proposal:

Proposal 1: The required SNR targeted 1% BLER for PSBCH test case is about -2.9dB.

- Reference

- R4-2103991, “Updated simulation assumptions for V2X single link test cases”,  Huawei, HiSilicon