3GPP TSG-RAN WG4 Meeting # 99 	R4-2110416

# Electronic Meeting, May. 19-27, 2021

Source:	Ericsson

Title:	Timing requirements

Agenda item:	9.12.4.3

Document for:	Approval

# 1	Introduction

In RAN4#98bis-e a WF was approved [1]. We analyze the questions in the WF and continue to discuss timing requirements for NTN WI.

# 2	Discussion

## 2.1	UE initial transmit timing error

In the approved WF, it is stated [1]:

- The UE specific TA estimation accuracy is counted into the UE transmit timing error requirement - UE specific TA estimation accuracy is FFS - FFS whether the UE specific TA estimation accuracy shall be also defined as a separate accuracy requirement - Specify UE behaviour related to UE specific TA estimation and the detailed behaviour is FFS - FFS on the update periodicity of UE specific TA value - UE initial transmit timing error (Te) - Te requirement in NTN is consist of: - Same types of errors as terrestrial UE e.g. DL timing estimation accuracy and UL timing setting accuracy. and; - UE specific estimation accuracy; - FFS on whether and how much different relaxations are required for different sets of SCS of SSB and SCS of uplink signals - It is the total NTN UE Te error that decides UL performance, no matter the source of inaccuracy. Note: The original text in WF seem to be garbled. I have edited it according to original agreement, as per change marks.

Our main point is that the it is the total NTN UE Te error that decides UL performance, no matter the source of inaccuracy. If a UE is subject to a positioning error ΔUE-pos (in time), the error in satellite position is ΔSat-pos (in time) and the UE SSB timing estimation accuracy is ΔUE_timing_estimate then we get, where Te is the initial timing error:

ΔUE-pos + ΔSat-pos  + ΔUE_timing_estimate  < Te

Te is needed in existing specification to safeguard against Inter Symbol interference and maintain a desired UL throughput and capacity. The existing requirements can be found in Table 1.

Table 1: Te Timing Error Limit

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 12*64*Tc |
|  |  | 30 | 10*64*Tc |
|  |  | 60 | 10*64*Tc |
|  | 30 | 15 | 8*64*Tc |
|  |  | 30 | 8*64*Tc |
|  |  | 60 | 7*64*Tc |
| 2 | 120 | 60 | 3.5*64*Tc |
|  |  | 120 | 3.5*64*Tc |
|  | 240 | 60 | 3*64*Tc |
|  |  | 120 | 3*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

Observation 1: The UE initial transmit timing error is needed to make sure we avoid Inter Symbol Interference and loose UL throughput and capacity.

Observation 2: The existing requirements make sure we fulfil intended UL throughput and capacity in NR.

The existing requirements are the conclusion of analysis in 3GPP RAN4 for an NR network for a large set of time dispersive channels. This makes sure that minimum system performance is guaranteed also for difficult radio environments. We therefore propose to reuse existing UE initial transmit timing error. This is also the conclusion in [2], at least for SCS = 15 kHz. “NTN UL time synchronization requirements for the service link can be defined as the way of the UE transmit timing error ±Te = ± 0.39 μs (=12*64*Tc), as specified in 7.1.2 in TS 38.133, for PRACH transmission and the first transmission in a DRX cycle for PUCCH, PUSCH and SRS”.

Another way to analyse the question is to try and assess maximum allowed fraction of uncertainties of CP from UE to gNB. In [5], table 2 the existing uncertainties for a terrestrial UE transmitting to a gNB is summarised as:

Table 2: Relation between CP length and existing timing errors at the BS reception

| SSB SCS (kHz) | UL SCS (kHz) | UE timing error (Te) [Tc] | TA command resolution error (section 4.2, TS 38.213) [Tc] | TA adjustment accuracy (section 7.3.2, TS 38.133) [Tc] | BS error (TBSE) [Tc] | BS error (TBSE) as percentage of UL CP length (%) |
| --- | --- | --- | --- | --- | --- | --- |
| 15 | 15 | 768 | 512 | 256 | 1536 | 17 |
|  | 30 | 640 | 256 | 256 | 1152 | 25 |
|  | 60 | 640 | 128 | 128 | 896 | 39 |
| 30 | 15 | 512 | 512 | 256 | 1280 | 14 |
|  | 30 | 512 | 256 | 256 | 1024 | 22 |
|  | 60 | 448 | 128 | 128 | 704 | 31 |
| 120 | 60 | 224 | 128 | 128 | 480 | 21 |
|  | 120 | 224 | 64 | 32 | 320 | 28 |
| 240 | 60 | 192 | 128 | 128 | 448 | 19 |
|  | 120 | 192 | 64 | 32 | 288 | 25 |

In the same analysis the maximum allowed fraction of CP is show in the rightmost column of table 4 in [5]. Even though in some cases the error is still high but considering also UE implementation aspect, adding a UE margin of one Te , can be acceptable from the BS performance point of view.

Table 3: BS error (TBSE) + margin [5]

| SSB SCS (kHz) | UL SCS (kHz) | BS error (TBSE) [Tc] | UE margin [Tc] | BS error + margin [Tc] | BS error (TBSE) + margin as percentage of UL CP length (%) |
| --- | --- | --- | --- | --- | --- |
| 15 | 15 | 1536 | 768 | 2304 | 25 |
|  | 30 | 1152 | 640 | 1792 | 39 |
|  | 60 | 896 | 640 | 1536 | 67 |
| 30 | 15 | 1280 | 512 | 1792 | 19 |
|  | 30 | 1024 | 512 | 1536 | 33 |
|  | 60 | 704 | 448 | 1152 | 50 |
| 120 | 60 | 480 | 224 | 704 | 31 |
|  | 120 | 320 | 224 | 544 | 47 |
| 240 | 60 | 448 | 192 | 640 | 28 |
|  | 120 | 288 | 192 | 480 | 42 |

In the release 15 study report [6], section 6.7.2 the Delay Spread (DS) is listed as < 150 ns across NTN scenarios.

Observation 3: the Delay Spread (DS) is listed as < 150 ns NTN across scenarios in the release 15 study report

This could further motivate the allowed UE margin in Table 3.

Proposal 1: Use existing UE initial transmit timing error, Te also for NTN as UE specific estimation accuracy for initial access (or make Te_NTN = 2*Te)

## 2.2	TA adjustment accuracy

A TA adjustment accuracy requirement is needed to safeguard against Inter Symbol interference and maintain a desired UL throughput and capacity. The needed TA adjustment accuracy is a function of the step size. Based on the assumption that RAN1 keeps the existing TA step size of 16*64/2µ then we should maintain the existing step sizes in Table 2:

Table 2: UE Timing Advance adjustment accuracy

| UL Sub Carrier Spacing(kHz) | 15 | 30 | 60 | 120 |
| --- | --- | --- | --- | --- |
| UE Timing Advance adjustment accuracy | ±256 Tc | ±256 Tc | ±128 Tc | ±32 Tc |

Observation 4: The UE Timing Advance adjustment accuracy is needed to make sure we avoid Inter Symbol Interference and loose UL throughput and capacity.

Proposal 2: Use existing TA adjustment accuracy also for NTN.

## 2.3 	Gradual Timing Adjustment

The parameter Te has further use to regulate gradual timing adjustment. For the gradual timing adjustment, TS 38.133 offers the following definition:

When the transmission timing error between the UE and the reference timing exceeds ±Te then the UE is required to adjust its timing to within ±Te. The reference timing shall be  before the downlink timing of the reference cell. All adjustments made to the UE uplink timing shall follow these rules:

1)	The maximum amount of the magnitude of the timing change in one adjustment shall be Tq.

2)	The minimum aggregate adjustment rate shall be Tp per second.

3)	The maximum aggregate adjustment rate shall be Tq per 200 ms.

where the maximum autonomous time adjustment step Tq and the aggregate adjustment rate Tp are specified in Table 7.1.2.1-1.

Table 7.1.2.1-1 in TS 38.133 is repeated here as Table 1.

Table 1: Tq Maximum Autonomous Time Adjustment Step and Tp Minimum Aggregate Adjustment rate

| Frequency Range | SCS of uplink signals (kHz) | Tq | Tp |
| --- | --- | --- | --- |
| 1 | 15 | 5.5*64*Tc | 5.5*64*Tc |
|  | 30 | 5.5*64*Tc | 5.5*64*Tc |
|  | 60 | 5.5*64*Tc | 5.5*64*Tc |
| 2 | 60 | 2.5*64*Tc | 2.5*64*Tc |
|  | 120 | 2.5*64*Tc | 2.5*64*Tc |
| NOTE: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

The Tq and Tp requirements are essentially a function of Te, with an aim to make sure that the UL timing follow the DL timing reference. If we take the case of LEO, then the rate of change of round-trip time can be as high as dRTT/dt = 40 µs/s ≈ 79 Tc/ms [from Table 7.1-1 in [3]].

At higher speed there is larger time variation or time drift (Td) of the DL reference signal received at the UE. This will require the UE to apply larger maximum adjustment step or aggregated maximum adjustment over 200 ms period. The time drift over a given time period (T0) can be estimated as follows:

Td = dRTT/dt * T0

Where dRTT/dt is the maximum delay variation as seen by the UE.

Based on the above expression the speed dependent time drift (Td) at the TBD UE speed over a duration of 200 ms is about 246 * 64 Tc, if we assume a worst case dRTT/dt = ± 40 µs/s. from Table 7.1-1 in [3]. This is very high.

Observation 5: The parameter Tq will have to be modified. For a period of 200 ms we could have a worst case delay variation of 246 * 64 Tc.

This observation is similar to Table 1, Maximum timing drift in NTN scenario [4].

Observation: 6: Either the period has to be shortened from 200 ms to something smaller, or we need to increase Tq.

# 3	Summary

Observation 1: The UE initial transmit timing error is needed to make sure we avoid Inter Symbol Interference and loose UL throughput and capacity.

Observation 2: The existing requirements make sure we fulfil intended UL throughput and capacity in NR.

Observation 3: the Delay Spread (DS) is listed as < 150 ns NTN across scenarios in the release 15 study report

Proposal 1: Use existing UE initial transmit timing error, Te also for NTN as UE specific estimation accuracy for initial access (or make Te_NTN = 2*Te)

Observation 4: The UE Timing Advance adjustment accuracy is needed to make sure we avoid Inter Symbol Interference and loose UL throughput and capacity.

Proposal 2: Use existing TA adjustment accuracy also for NTN.

Observation 5: The parameter Tq will have to be modified. For a period of 200 ms we could have a worst case delay variation of 246 * 64 Tc.

Observation: 6: Either the period has to be shortened from 200 ms to something smaller, or we need to increase Tq.

# References

- R4-2105796, WF on timing requirements for NR NTN, Xiaomi

- R4-2106360, Discussion on timing requirements in NTN, MediaTek Inc

- TR 38.821, Solutions for NR to support non-terrestrial networks (NTN), V16.0.0

- R4-2104604, Discussion on NTN timing requirements, CMCC

- R4-1912197, Further analysis of one-shot timing adjustment requirements, Ericsson

- TR 38.811, Study on New Radio (NR) to support non-terrestrial networks (Release 15)