3GPP TSG-RAN WG4 Meeting # 99-e 	R4-2110492

# Electronic Meeting, May. 19-27, 2021

Title: 	Discussions on issues for FRCs of PUSCH requirements and UCI multiplexing on PUSCH requirements testing

Source: 	Huawei, HiSilicon

Agenda item:	4.1.9.3

Document for:	Discussion

Background

In this contribution, we give our discussions on issues for FRCs of PUSCH requirements and UCI multiplexing on PUSCH requirements testing.

Discussions

FRCs for UCI multiplexing on PUSCH

The FRCs such as G-FR1-A4-11 for FR1 and G-FR2-A4-3, G-FR2-A4-13 for FR2 which are reused from PUSCH requirements are defined for UCI multiplexing on PUSCH. The reason is that most parameters are same for UCI performance testing and PUSCH performance testing. But one problem is that overhead of UCI is ignored for calculation of channel bits denoted as “Total number of bits per slot” and total number of REs denoted as “Total symbols per slot”. As specified in TS38.212, data mapping occurs after UCI mapping. i.e. data is rate matched with UCI,  then overhead of UCI should be considered.

Taking G-FR1-A4-11 as an example, it is both used for PUSCH requirements and UCI multiplexing on PUSCH requirements, but channel bits is calculated based on conditions without consideration of UCI overhead.

Therefore, we propose to preparer new CRs to create the new FRCs for UCI multiplexing on PUSCH with recalculation of channel bits and total number of REs considering overhead of UCI.

Observation 1: Overhead of UCI is ignored for the calculation of channel bits and total number of REs for FRCs of UCI multiplexing on PUSCH requirements

Proposal 1: Preparer new CRs to create the new FRCs for UCI multiplexing on PUSCH with recalculation of channel bits and total number of REs considering overhead of UCI.

FRCs for FR2 PUSCH requirements with PTRS

Similar to UCI requirements, FRCs for PUSCH requirements with and without PTRS are the same without the consideration of PTRS overhead, the affected parameters are “Total number of bits per slot” and “Total symbols per slot”.

Taking G-FR2-A4-4 as an example,  it is both used for PUSCH with and without PTRS, but channel bits is calculated based on conditions without PTRS.( 9*66*12*4=28512 bits)

Other FRCs for FR2 PUSCH requirements with PTRS have the same problem.

Observation 2: Overhead of PTRS is ignored for the calculation of channel bits and total number of REs for FRCs of FR2 PUSCH requirements with PTRS

To solve the problem, we propose to prepare the new CRs to add the channel bits and number of REs to FRCs for PUSCH requirements with PTRS considering the overhead of PTRS like following:

| Reference channel | Value |
| --- | --- |
| Subcarrier spacing [kHz] | xx |
| Allocated resource blocks | xx |
| CP-OFDM Symbols per slot (Note 1) | xx |
| Modulation | xxx |
| Code rate (Note 2) | xxx |
| Payload size (bits) | xxxx |
| Transport block CRC (bits) | xx |
| Code block CRC size (bits) | xx |
| Number of code blocks - C | xx |
| Code block size including CRC (bits) (Note 2) | xxxx |
| Total number of bits per slot | X for case with PTRS Y for case without PTRS |
| Total symbols per slot | X for case with PTRS Y for case without PTRS |

Proposal 2: Prepare the new CRs to add the channel bits and number of REs to FRCs for PUSCH requirements with PTRS considering the overhead of PTRS

Conclusion

Observation 1: Overhead of UCI is ignored for the calculation of channel bits and total number of REs for FRCs of UCI multiplexing on PUSCH requirements

Proposal 1: Preparer new CRs to create the new FRCs for UCI multiplexing on PUSCH with recalculation of channel bits and total number of REs considering overhead of UCI.

Observation 2: Overhead of PTRS is ignored for the calculation of channel bits and total number of REs for FRCs of FR2 PUSCH requirements with PTRS

Proposal 2: Prepare the new CRs to add the channel bits and number of REs to FRCs for PUSCH requirements with PTRS considering the overhead of PTRS