3GPP TSG-RAN WG4 Meeting #99-e	 R4-2110569

# Electronic Meeting,  May. 19th – 27th, 2021

Title: 	Discussion on PUSCH demodulation requirements for FR1 256QAM

Source: 	Huawei, HiSilicon

Agenda item:	9.11.3.1

Document for:	Discussion

Background

In RAN4 #98bis-e meeting, the WF [1] for PUSCH 256QAM was agreed. Most of the test parameters were agreed as follows:

- Applicability for PUSCH 256QAM demodulation performance requirements: Only applied for BS manufacturers declaring that uplink 256QAM is supported.

- MCS: Evaluate {MCS24, MCS22} as starting point for next meeting, based on the simulation results to decide if MCS24 is feasible, otherwise to check lower MCS22 is feasible or not.

- TDD pattern:

  - 15kHz SCS: 3D1S1U, S=10D:2G:2U

  - 30kHz SCS: 7D1S2U, S=6D:4G:4U

- Applicability rules for TDD with different TDD patterns: Reuse the existing applicability rules defined in section 8.1.2.1.5 of TS 38.141-1

- PUSCH start symbol and symbol length: S = 0, L=14.

- Frequency domain allocation: Full bandwidth frequency domain resource allocation

- DM-RS type: Type 1 with single-symbol DM-RS.

- Additional DM-RS (dmrsAdditionalPosition): pos1, FFS pos2

- Decide based on interesting companies’ feedback for next meeting

- PT-RS configuration: FFS configure PT-RS.

- Further discuss and decide whether to configure PT-RS or not based on feedback from interesting companies in next meeting.

- Phase Noise modelling:

- Realistic phase noise modelling is left up to the contributing entities.

- Phase noise impact can be either considered in ideal simulations or included in impairment results based on further discussion and evaluations.

- Interesting company is welcome to do investigation on the PN impact on 256QAM performance for next meeting.

- Tx EVM:

- Interesting companies are welcome to check the performance difference with and without Tx EVM (3.5% as baseline) impact considered.

- RAN4 will discuss and decide whether additional margin should be considered in alignment results if no Tx EVM modelling in next meeting as per the evaluations results.

- Number of HARQ transmission and RV sequence: 4 HARQ transmission with RV sequence {0,2,3,1}.

- Test metric: 70% max throughput.

- Other test parameters:

- CP: Normal

- SRS: Not configured

- Number of DM-RS CDM groups with data: 2

- Ratio of PUSCH EPRE to DM-RS EPRE: -3dB

- DM-RS sequence generation: NID=0, nSCID=0

- Frequency hopping: Disabled

- CBG-based PUSCH transmission: Disabled

- Timing offset: 0

- Frequency offset: 0

- Limited buffer rate matching: Disabled.

- Waveform:

- CP-OFDM only.

- Propagation condition:

- TDLA30-10 Low

- PUSCH mapping type

- Type A and Type B with reusing the existing applicability rule defined in section 8.1.2.1.3 of TS 38.141-1

- FRC: Discuss the FRC after MCS, DM-RS pattern, BW and Rank are finalized.

Some open issues are remained:

- Number of Tx:

- Option 1: Only 1Tx

- Option 2: Both 1Tx and 2Tx

- Number of Rx:

- Option 1: 2/8 Rx

- Option 2: 2/4/8 Rx

- Number of layers:

- Option 1: Only 1 layer

- Option 2: Both of 1 and 2 layers

- SCS and bandwidth

- 15kHz SCS:

- Option 1:5MHz and 10MHz

- Option 2: 5MHz, 10MHz and 20MHz.

- 30kHz SCS

- Option 1:10MHz and 40MHz

- Option 2:10MHz, 20MHz, 40MHz and 100MHz.

- Applicability rule for different antenna configurations

- Option 1: Reuse the existing test applicability rule defined in clause 8.1.2.0 of TS38.141-1

- Other options

- Applicability rules for different SCS and CBW

- Option 1:  Reuse the existing applicability rules defined in sections 8.1.2.1.1 and 8.1.2.1.2 of TS 38.141-1

- Other options.

In this paper, the open issues will be discussed and views will be provided.

Discussion

Discussion:

In this section, the open issues will be discussed.

Number of Tx, Rx and layers:

For number of Tx, 1 Tx is preferred. 2Tx is not a typical schedule for PUSCH 256QAM.

For Rx, the applicability rule can cover all the possible configurations and to reduce the simulation efforts, we support to only define 2Rx/8Rx.

Proposal 1: We propose the following configurations:

Number of Tx: 1.

Number of Rx: 2/8Rx.

Number of layer: 1.

SCS and bandwidth:

For SCS and bandwidth, we prefer to define 10 MHz/15 kHz and 40 MHz/30 kHz as they are commonly used for PUSCH FR1 performance requirements. The test coverage of different BW have been verified in Rel-15. We prefer to define requirement with typical scenarios.

Proposal 2: We propose to define SCS and bandwidth of 10 MHz/15 kHz and 40 MHz/30 kHz for PUSCH 256QAM.

Applicability rule for different antenna configurations:

We prefer to reuse the existing applicability rule for different antenna configurations:

Unless otherwise stated, for a BS supporting more than 8 antenna connectors (for BS type 1-C) or TAB connectors (for BS type 1-H) (see D.37 in table 4.6-1), the performance requirement tests for 8 RX antennas shall apply, and the specific connectors used for testing are based on manufacturer declaration.

Unless otherwise stated, for a BS supporting different numbers of antenna connectors (for BS type 1-C) or TAB connectors (for BS type 1-H) (see D.37 in table 4.6-1), the tests with low MIMO correlation level shall apply only for the lowest and highest numbers of supported connectors, and the specific connectors used for testing are based on manufacturer declaration.

By using this applicability rule, the test can cover all the possible antenna configurations. We do not see the necessary to change it.

Proposal 3: We propose to reuse the existing test applicability rule for different antenna configurations.

Applicability rules for different SCS and CBW:

We support to reusing the existing applicability rule:

For each subcarrier spacing declared to be supported, the test requirements for a specific channel bandwidth shall apply only if the BS supports it (see D.14 in table 4.6-1).

Unless otherwise stated, for each subcarrier spacing declared to be supported, the tests shall be done only for the widest supported channel bandwidth. If performance requirement is not specified for this widest supported channel bandwidth, the tests shall be done by using performance requirement for the closest channel bandwidth lower than this widest supported bandwidth; the tested PRBs shall then be centered in this widest supported channel bandwidth.

To ensure the defined test cases can cover all the supported bandwidth, the applicability rule for different channel bandwidths is necessary. The existing applicability rule has been verified in Rel-15 and we prefer to reuse it.

Proposal 4: We propose to reuse the existing test applicability rule for different SCS and CBW.

Simulation:

Simulation assumption:

The agreed parameters are collected in the table below:

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1, FFS pos2 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | FFS |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain | PUSCH mapping type | A, B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
| assignment | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| Limited buffer rate matching |  | Disabled |
| SRS |  | Not configured |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL pattern. |  |  |

Simulation results:

Based on the parameters we proposed, the performance of PUSCH 256QAM with antenna configuration of 1x2 is simulated. The figure below shows the results:

Figure 1: Simulation results for PUSCH 256QAM with antenna configuration of 1x2

The SNR at 70% throughput of 15 kHz/10 MHz with MCS24 is about 19.8dB. As companies discussed during the last RAN4 meeting that the SNR value should be lower than 20dB and impairments values have not been added, thus, we propose to use MCS22.

Proposal 5: We propose to use MCS22.

Proposals

In this paper, we have the followings proposed:

P Proposal 1: We propose the following configurations:

Number of Tx: 1.

Number of Rx: 2/8Rx.

Number of layer: 1.

Proposal 2: We propose to define SCS and bandwidth of 10 MHz/15 kHz and 40 MHz/30 kHz for PUSCH 256QAM.

Proposal 3: We propose to reuse the existing test applicability rule for different antenna configurations.

Proposal 4: We propose to reuse the existing test applicability rule for different SCS and CBW.

Proposal 5: We propose to use MCS22.

Reference

[1] R4-2106120 “Way forward for PUSCH 256QAM performance requirements”, Huawei, RAN4 #98bis-e, April, 2021.