3GPP TSG-RAN WG4 Meeting#99-e                               R4-2110601

E-meeting,19th May– 27th May, 2021

Agenda item:	9.15.5.1

Source: 	ZTE Corporation

Title: 	Discussion on BS Tx requirements for 60GHz

Document for:	Approval

Introduction

In the previous RAN4 meetings, there were some initial discussions on BS Tx requirements for 60GHz and some high level agreement has been reached in WF [1]. In this contribution, we want to share some further considerations on each Tx requirements.

Discussion

## 2.1. TRP/EIRP accuracy

For TRP/EIRP accuracy requirement defined for mmWave BS in Rel-15, the following factors considered already could also apply for 60GHz BS. In addition, it should be noted that TRP/EIRP accuracy requirement is already relative large compared with declared maximum TRP and EIRP. If to further relax the TRP/EIRP requirements for 60GHz BS, the reliability of declared rated TRP/EIRP might be questionable, therefore we propose to reuse the existing FR2 TRP/EIRP accuracy requirements for 60GHz BS regardless of unlicensed or unlicensed operation.

1) transceiver unit accuracy;

2) phase shift accuracy among transceiver units;

3) phase shifter accuracy within analog phase shifters;

4) sub-array gain accuracy, mismatch errors and insertion loss variations.

Proposal 1: to reuse the existing FR2 TRP/EIRP accuracy requirements for 60GHz BS regardless of unlicensed or unlicensed operation.

## 2.2. ON-OFF mask

For ON-OFF/OFF-ON transition period, there were some concerns during 60GHz SI phase on its high overhead especially for 480kHz and 960kHz and some further improvement are encouraged to be investigated.

From the hardware feasibility perspective, with the increasing operation frequency for 60GHz, then settling time for PLL might be shorter compared with that of existing FR2. In addition, ramping up time and ramping down time of PA is highly related with the supported output power or electric current within the PA, if the output power for 60GHz is on the same level of  that for 45GHz and 28GHz and same PA manufacturing material (e.g. GaAs, Si), then similar ramping up time and ramping down time could be expected for 60GHz as the existing FR2.

Observation 1: if the output power for 60GHz is on the same level of that for the existing FR2 and same PA manufacturing material (e.g. GaAs, Si), then similar ramping up time and ramping down time could be expected for 60GHz.

In addition, the analysis on the GP overhead with the following TDD periodicity assumption is shown in Table 1 and Table 2. It could be found that GP overhead might be too high if the same ON-OFF transition time for the existing FR2 with the proposed TDD periodicity was reused for 60GHz. Alternatives to reduce the GP overhead could be either extend the TDD periodicity for 480kHz and 960kHz if to reduce ON-OFF transition time from BS and UE side to shorten the GP time is not feasible considering the hardware limitation.

Observation 2: to reduce the GP overhead for 480kHz and 960kHz of 60GHz, alternatives could be extend the TDD periodicity.

Table 1. cell coverage analysis

| Frequency range | Existing FR2 |  | 60GHz |  |
| --- | --- | --- | --- | --- |
| Numerology (kHz) | 60 | 120 | 480 | 960 |
| GP (symbol) | 1 | 2 | 8 | 16 |
| GP (us) | 16.67 | 16.67 | 16.67 | 16.67 |
| Sync error (us) | 3 | 3 | 3 | 3 |
| BS OFF-ON (us) | 3 | 3 | 3 | 3 |
| UE OFF-ON (us) | 5 | 5 | 5 | 5 |
| Tcell (us) | 1.3333 | 1.3333 | 1.3332 | 1.3328 |
| Cell coverage (m) | 399.99 | 399.99 | 399.96 | 399.84 |
| Periodicity of TDD (ms) | 0.5 | 0.25 | 0.0625 | 0.03125 |
| GP overhead (%) | 3.58 | 7.12 | 26.67 | 53.33 |

Table 2. BS to BS distance without interference

| Frequency range | Existing FR2 |  | 60GHz |  |
| --- | --- | --- | --- | --- |
| Numerology (kHz) | 60 | 120 | 480 | 960 |
| GP (symbol) | 1 | 2 | 8 | 16 |
| GP (us) | 16.67 | 16.67 | 16.67 | 16.67 |
| Sync error (us) | 3 | 3 | 3 | 3 |
| BS ON-OFF (us) | 3 | 3 | 3 | 3 |
| BS OFF-ON (us) | 3 | 3 | 3 | 3 |
| Tbs-bs (us) | 4.67 | 4.67 | 4.67 | 4.67 |
| Two BSs distance (m) | 1401 | 1401 | 1399.92 | 1399.68 |
| Periodicity of TDD (ms) | 0.5 | 0.25 | 0.0625 | 0.03125 |
| GP overhead (%) | 3.334 | 6.668 | 26.67 | 53.33 |

## 2.3. Tx OFF power

For Tx OFF power of 60GHz, if to follow the legacy approach as following adopted to derive FR2 BS TX OFF power, then -36dBm/MHz defined in Rel-15 could be reused since assumed typical deployment with ISD 100m between adjacent sites should be enough for 60GHz BS deployment and relative larger pathloss expected for 60GHz could also alleviate the performance impacts on victim BS.

TRPoff=-174+10*log10(B)+NF-M-10*log10(N)+Lp-GERX

Proposal 2: to reuse -36dBm/MHz for 60GHz BS regardless of licensed operation or unlicensed operation.

## 2.4.TAE requirements for MIMO, CA case

As discussed in 60GHz SI phase, it was recognized that TAE 65ns for MIMO transmission might be not valid anymore especially considering the CP length for 960kHz SCS as shown in Table 1. TAE 65ns is almost close to CP length which would result in serious performance degradation at receiver side, therefore some initial simulation results are provided in Figure 1 and Figure 2 to show the impacts of different TAE values on downlink performance. Detailed simulation assumptions are listed in Annex 5.

Table 1. Summary for CP and data length for different SCS

| u | SCS | CP[ns] | data[ns] | Sampling Rate [MHz] |
| --- | --- | --- | --- | --- |
| 2 | 60 | 1171.875 | 16666.6 | 245.76 |
| 3 | 120 | 585.9375 | 8333.3 | 491.52 |
| 4 | 240 | 292.96875 | 4166.6 | 983.04 |
| 5 | 480 | 146.484375 | 2083.3 | 1966.08 |
| 6 | 960 | 73.2421875 | 1041.6 | 3932.16 |

Figure 1. the impacts of TAE on downlink link performance [960kHz]

Figure 2. the impacts of TAE on downlink link performance [480kHz]

Based on the simulation results in Figure 1 and Figure 2, it could be easily found that 65ns TAE would cause severe performance degradation especially for 960kHz.

For 960kHz SCS, the acceptable TAE requirement should be around 10-20ns for 960kHz and 10-40ns for 480kHz.

Proposal 3: to discuss the simulation assumptions to further evaluate acceptable TAE requirements for 60GHz 480kHz and 960kHz SCS.

## 2.5. Unwanted emission requirements

In the last RAN4 meeting, there were some initial discussions how to further evaluate coexistence performance for 60GHz and whether coexistence work in TR 38.803 can be used as reference for the coexistence study for 60GHz. Based on our initial understandings, to increase the antenna array size for BS at 60GHz and lower the BS/UE maximum output power, this will only result in even lower ACIR values after the the coexistence study. Based on our understandings, this agreed value in Table 2 is also feasible even though this might be relative tighter than IEEE mask.

Table 2. Way forward on BS ACLR and ACS

| BS | 30 GHz | 45 GHz | 70 GHz |
| --- | --- | --- | --- |
| ACLR | 27.5 | 25.5 | 23.5 |
| ACS | 23.5 | 22.5 | 21.5 |

Proposal 4: to agree ACLR 24dBc for 60GHz.

In addition, for the OBUE limit for 60GHz, the corresponding mask could be defined as following Table 3:

Table 3: OBUE limits applicable in the frequency range 52.6 GHz-71 GHz

| Frequency offset of measurement filter -3B point, f | Frequency offset of measurement filter centre frequency, f_offset | Limit | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1*BWcontiguous | 0.5 MHz f_offset < 0.1* BWcontiguous +0.5 MHz | Min(-5 dBm, Max(Prated,t,TRP – 31 dB, -12 dBm)) | 1 MHz |
| 0.1*BWcontiguous f < fmax | 0.1* BWcontiguous +0.5 MHz f_offset < f_ offsetmax | Min(-13 dBm, Max(Prated,t,TRP – 39 dB, -20 dBm)) | 1 MHz |
| NOTE 1: For non-contiguous spectrum operation within any operating band the limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the sub-block gap. |  |  |  |

Proposal 5: to agree OBUE limit in Table 3 for 60GHz.

In addition, it could be noted that the proposed mask and ACLR value is tighter than IEEE mask as shown in Figure 3, in other words, this could also well coexist with the 802.11ad/ay system for unlicensed operation instead of licensed operation only.

Figure 3. Transmit mask for 802.11ad 2.16GHz

Conclusions

In this contribution, we want to further considerations on each BS Tx requirements for 60GHz and some proposals and observations are made as following:

Proposal 1: to reuse the existing FR2 TRP/EIRP accuracy requirements for 60GHz BS regardless of unlicensed or unlicensed operation.

Observation 1: if the output power for 60GHz is on the same level of that for the existing FR2 and same PA manufacturing material (e.g. GaAs, Si), then similar ramping up time and ramping down time could be expected for 60GHz.

Observation 2: to reduce the GP overhead for 480kHz and 960kHz of 60GHz, alternatives could be extend the TDD periodicity.

Proposal 2: to reuse -36dBm/MHz for 60GHz BS regardless of licensed operation or unlicensed operation.

Proposal 3: to discuss the simulation assumptions to further evaluate acceptable TAE requirements for 60GHz 480kHz and 960kHz SCS.

Proposal 4: to agree ACLR 24dBc for 60GHz.

Proposal 5: to agree OBUE limit in Table 3 for 60GHz.

References

- R4-2106113,WF on BS RF TX requirements for 52.6 – 71 GHz, approved.

- R4-210xxx, Discussion on switching delay for 52.6-71GHz, ZTE.

- R4-1700302,Way forward on ACLR & ACS for WP5D LS, approved.

Annex: simulation assumption

Table A1-1 Link level simulation assumption

| Parameters | Values or assumptions |
| --- | --- |
| Carrier Frequency | 60GHz |
| Waveform | CP-OFDM |
| System Bandwidth | 400MHz |
| Subcarrier spacing | 480kHz/960kHz |
| CP type | Normal CP |
| Channel Model | TDL-A 5ns |
| PN model | 3GPP TR38.803 example 2 |
| DMRS Configuration | 2 DMRS symbols per slot at (2,11) |
| PTRS Configuration | (K = 2, L = 1) |
| SLIV | (S=0, L=14) |
| Channel Estimation | Realistic |
| PN Estimation | Realistic |
| PN compensation | CPE & ICI |
| UE receiver | MMSE-IRC |
| BS antenna Array configuration | 2 |
| UE antenna Array configuration | 2 |