3GPP TSG-RAN WG4 Meeting #99-e		R4-2110837
Electronic Meeting, 19th May – 27th May, 2021

Agenda item:	9.1.3.1

Source:	OPPO

Title:	Channel model for FR1 4x4 MIMO OTA requirement

Document for:	Approval

1	Introduction

In the previous meeting of RAN4 #98-e, the channel model for FR1 4x4 MIMO OTA was concluded as CDL-C UMa [1].

During email discussion of RAN4 #98-bis-e, it is found that only one beam become effective when the centre frequency is lower than 2.5GHz for CDL-C UMa, which means 4x4 MIMO can not be applied on such channel model [2].

This paper presents our findings and proposal on channel model for FR1 4x4 MIMO OTA.

2	Discussion

Based on R4-2106097 [3], which presents the reference channel model curves, the gNB beams selected by BS antenna filtering effect can be derived according to TR 38.901. And for higher frequencies, they are illustrated in the following figure.

Referring to the response by TE vender, the cause of different beam selection result between lower (one strongest beam selected) and higher frequency (two strongest beam selected) is that different BS array assumptions are used, i.e. 4x8 array for frequencies lower than 2.5GHz and 8x8 array for frequencies above 2.5GHz. Compared with higher frequencies, BS array with lower frequencies has the same beam width on azimuth and wider beam width on zenith. From the above figure, ZODs of the strongest beam and the second strongest beam have difference of 0.7 degree, which can be distinguished by 8x8 array, while can not be distinguished by 4x8 array. This phenomenon needs more study before concluding on whether CDL-C is an appropriate channel model for frequencies lower than 2.5GHz. And TE venders are encouraged to provide solutions on channel models optimization to afford 4x4 MIMO.

Observation: for CDL-C, ZODs of the strongest beam and the second strongest beam have difference of 0.7 degree, which can be distinguished by 8x8 array, while can not be distinguished by 4x8 array.

Proposal: TE venders are encouraged to provide solutions on CDL-C optimization to afford 4x4 MIMO.

3	Conclusion

Observation: for CDL-C, ZODs of the strongest beam and the second strongest beam have difference of 0.7 degree, which can be distinguished by 8x8 array, while can not be distinguished by 4x8 array.

Proposal: TE venders are encouraged to provide solutions on CDL-C optimization to afford 4x4 MIMO.

References

- R4-2103913 WF on NR MIMO OTA_v3

- R4-2106144 Summary_326_after 2nd round

- R4-2106097 CM Reference Curves_v4