3GPP TSG-RAN WG4 #99-e	R4-2110937

Electronic Meeting, May 19th – May 27th, 2021

Agenda item:	6.1.7.2

Source:	MediaTek Inc.

Title:	Simulation Results for NR-U PDSCH requirements

Document for:	Discussion

- Introduction

In RAN4#98-bis-e, a way forward for UE demodulation requirements for NR-U was agreed [1]. We provide simulation results based on the agreed simulation assumptions.

- Simulation results

The provided simulation results are based in Table 1 in Annex.

2.1 Simulation results for 2Rx

Figure 2-1. Simulation results for 2Rx

Table 2-1: Summary of simulation results for 2Rx

| Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 20 / 30 | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 11.6 |
| 40 / 30 | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 11.9 |
| 60 / 30 | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 12 |
| 80 / 30 | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 12.2 |

2.2 Simulation results for 4Rx

Figure 2-2. Simulation results for 4Rx

Table 2-2: Summary of simulation results for 4Rx

| Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 20 / 30 | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 6.6 |
| 40 / 30 | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 6.7 |
| 60 / 30 | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 6.6 |
| 80 / 30 | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 6.8 |

- Conclusion

This contribution provides simulation results for the PDSCH demodulation requirements for NR-U.

- Reference

- R4-2106173, “Way Forward on NR-U UE demodulation requirements”, Qualcomm

Annex

The simulation assumptions are presented in Table 1.

Table 1. Simulation assumptions for PDSCH requirements in NR-U

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  |  | TDD |
| Active DL BWP index |  |  |  | 1 |
| LBT failure probability |  |  |  | 0.25 |
| Common serving cell parameters |  | Physical Cell ID |  | 0 |
|  |  | SSB position in burst |  | The first SSB |
|  |  | SSB Q factor |  | 8 |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | Starting symbol (S) |  |  | 2 |
|  | The number of slots set in a DL burst |  |  | {2,4,6,7} |
|  | Occupied OFDM symbols set in the last slot |  |  | {4,7,10,12} (Note 1) |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | 2 |
|  | Resource allocation type |  |  | Type 0 |
|  | RBG size |  |  | Config2 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | pos1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |
| CSI-RS for tracking | CSI-RS periodicity |  | Slots | 40 |
|  | CSI-RS offset |  | Slots | 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |
| Number of HARQ Processes |  |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | 8 if mod(i,10)=0 7 if mod(i,10)=1 6 if mod(i,10)=2 5 if mod(i,10)=3 4 if mod(i,10)=4 3 if mod(i,10)=5 2 if mod(i,10)=6 |
| TX EVM |  |  |  | 6% |
| Note 1: If DL Transmission duration is 2 Slot, the occupied OFDM symbols for PDSCH in the last slot is 12. |  |  |  |  |