3GPP TSG-RAN WG4 Meeting # 99-e	R4-2111003

# Electronic Meeting, 19 – 27 May 2021

Agenda item:	9.1.3.3

Source:	Keysight Technologies

Title:	Reference Channel Emulation Curves and a New Beam Selection for CDL-C UMa

Document for:	Approval

# Introduction

This contribution complements the reference curves for FR1 channel model validation purposes introduced in [1]. In addition, to decrease the BS correlation for CDL-C UMa channel model, a new beam direction for the second beam is introduced for ≤ 2.5 GHz. Since the CDL-A UMi model has been replaced by CDL-C UMi [2], this contribution takes the new CM into account.

# Discussion

As discussed and agreed in RAN4#98-e [3], reference curves need to be defined and agreed between CE vendors before the pass/fail limits for channel model validations can be agreed. This contribution complements the reference curves for FR1 channel model validation purposes introduced in [1]. The reference spatial correlation curves presented here are simulated curves to represent the actual correlation at the DUT position.

It was noticed the BS antenna correlation for CDL-C UMa (≤ 2.5 GHz) is too high (0.98) when using the beams with same azimuth direction. Therefore, a new second beam direction is needed to be defined. The beam directions for ≤ 2.5 GHz using 4x8 BS array are:

- Old beams:

  - Azimuth: -7.27, Elevation: -10

  - Azimuth: -7.27, Elevation: 0

- New beams:

  - Azimuth: -7.27, Elevation: -10

  - Azimuth: -21.82, Elevation: -10

Observation 1: BS correlation for CDL-C UMa at ≤ 2.5 GHz is too high with the old beam directions above.

Proposal 1: Use beam direction Az: -21.82, El: -10 for the second beam for CDL-C UMa at ≤ 2.5 GHz.

FR1 results for spatial correlation, PDP, and auto-correlation are shown in Figure 1 through Figure 9 and tabulated in Table 1 through Table 6; these results are based on the X polarized antenna model with 45˚ slant angle [2]. Tables for the FR1 spatial correlations will be added in a revision of this contribution. It should be noted that the spatial correlation curves for UMa CDL-C are shown per beam to simplify the alignment between CE vendors and that the combined beams will be addressed after successful alignment.

FR2 Channel model validation results are pending a definition of the BS antenna element polarization which is currently not defined [2].

Figure 1. Reference spatial correlation curves for strongest beam of CDL-C UMa channel model.

Figure 2. Reference spatial correlation curves for 2nd strongest beam of CDL-C UMa channel model.

Figure 3. Reference spatial correlation curves for CDL-C UMi channel model.

Figure 4. Reference PDP figure for CDL-C UMi channel model for fc ≤ 2.5 GHz.

Figure 5. Reference Temporal auto-correlation curves for CDL-C UMi channel model for fc ≤ 2.5 GHz.

Table 1. Reference PDP data for CDL-C UMi channel model for fc ≤ 2.5 GHz.

| Delay [ns] | Power [dB] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | X2V | H11 | H21 | H12 | H22 |
| 0.0 | -26.1 | -27.0 | -25.6 | -25.7 | -27.2 |
| 21.0 | -25.0 | -25.2 | -24.6 | -25.2 | -26.0 |
| 21.8 | -19.2 | -19.7 | -18.9 | -19.1 | -19.9 |
| 22.2 | -19.9 | -23.4 | -18.7 | -18.3 | -22.4 |
| 23.3 | -23.2 | -23.3 | -22.4 | -23.6 | -24.9 |
| 63.7 | 0.0 | 0.0 | 0.0 | -0.4 | -0.4 |
| 64.5 | -2.4 | -3.4 | -1.9 | -1.9 | -3.5 |
| 65.6 | -3.9 | -4.0 | -3.7 | -4.2 | -4.6 |
| 65.8 | -33.9 | -33.6 | -33.8 | -34.7 | -34.4 |
| 79.4 | -30.3 | -30.5 | -30.1 | -30.5 | -30.9 |
| 82.1 | -45.4 | -46.7 | -44.6 | -44.8 | -47.1 |
| 93.4 | -51.8 | -51.6 | -51.7 | -52.4 | -52.3 |
| 122.9 | -37.1 | -36.8 | -36.8 | -38.0 | -38.0 |
| 130.8 | -30.7 | -30.9 | -30.5 | -30.9 | -31.3 |
| 217.0 | -36.2 | -36.9 | -35.9 | -36.0 | -37.1 |
| 271.1 | -41.3 | -41.3 | -41.1 | -41.6 | -41.9 |
| 425.9 | -40.1 | -40.2 | -40.0 | -40.5 | -40.7 |
| 460.0 | -38.8 | -39.2 | -38.5 | -38.8 | -39.5 |
| 549.0 | -52.9 | -53.1 | -52.8 | -53.2 | -53.5 |
| 560.8 | -42.4 | -42.7 | -42.2 | -42.5 | -43.1 |
| 630.7 | -40.6 | -40.8 | -40.4 | -40.8 | -41.2 |
| 663.7 | -47.0 | -46.9 | -46.8 | -47.5 | -47.5 |
| 704.3 | -71.1 | -71.8 | -70.1 | -70.8 | -72.9 |
| 865.2 | -61.9 | -62.3 | -61.7 | -62.0 | -62.7 |

Table 2. Temporal auto-correlation data for CDL-C UMi channel model for fc ≤ 2.5 GHz.

| lambda | X2V | H11 | H21 | H12 | H22 |
| --- | --- | --- | --- | --- | --- |
| 0.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 |
| 0.16 | 0.99 | 0.99 | 0.99 | 0.99 | 0.99 |
| 0.31 | 0.97 | 0.98 | 0.96 | 0.96 | 0.98 |
| 0.47 | 0.94 | 0.96 | 0.92 | 0.92 | 0.95 |
| 0.63 | 0.90 | 0.93 | 0.88 | 0.87 | 0.93 |
| 0.78 | 0.86 | 0.90 | 0.82 | 0.81 | 0.89 |
| 0.94 | 0.81 | 0.87 | 0.76 | 0.74 | 0.86 |
| 1.09 | 0.75 | 0.84 | 0.70 | 0.67 | 0.82 |
| 1.25 | 0.70 | 0.80 | 0.63 | 0.60 | 0.77 |
| 1.41 | 0.64 | 0.76 | 0.57 | 0.53 | 0.72 |
| 1.56 | 0.59 | 0.72 | 0.52 | 0.47 | 0.67 |
| 1.72 | 0.54 | 0.68 | 0.48 | 0.41 | 0.62 |
| 1.88 | 0.50 | 0.64 | 0.44 | 0.37 | 0.57 |
| 2.03 | 0.46 | 0.60 | 0.42 | 0.34 | 0.53 |
| 2.19 | 0.43 | 0.56 | 0.40 | 0.32 | 0.48 |
| 2.34 | 0.40 | 0.53 | 0.39 | 0.31 | 0.44 |
| 2.50 | 0.38 | 0.50 | 0.38 | 0.31 | 0.39 |
| 2.66 | 0.36 | 0.46 | 0.36 | 0.30 | 0.36 |
| 2.81 | 0.33 | 0.43 | 0.34 | 0.28 | 0.32 |
| 2.97 | 0.30 | 0.39 | 0.31 | 0.27 | 0.29 |
| 3.13 | 0.27 | 0.35 | 0.27 | 0.24 | 0.26 |
| 3.28 | 0.24 | 0.32 | 0.24 | 0.22 | 0.23 |
| 3.44 | 0.21 | 0.28 | 0.20 | 0.19 | 0.21 |
| 3.59 | 0.17 | 0.25 | 0.16 | 0.16 | 0.20 |
| 3.75 | 0.14 | 0.21 | 0.13 | 0.13 | 0.19 |
| 3.91 | 0.11 | 0.18 | 0.10 | 0.10 | 0.18 |
| 4.06 | 0.08 | 0.15 | 0.08 | 0.09 | 0.18 |
| 4.22 | 0.06 | 0.12 | 0.08 | 0.09 | 0.18 |
| 4.38 | 0.04 | 0.10 | 0.08 | 0.09 | 0.18 |
| 4.53 | 0.04 | 0.08 | 0.08 | 0.09 | 0.18 |
| 4.69 | 0.03 | 0.07 | 0.09 | 0.09 | 0.18 |
| 4.84 | 0.03 | 0.06 | 0.09 | 0.09 | 0.18 |
| 5.00 | 0.03 | 0.05 | 0.09 | 0.08 | 0.18 |

Figure 6. Reference PDP figure for CDL-C UMi channel model for fc > 2.5 GHz.

Figure 7. Reference Temporal auto-correlation curves for CDL-C UMi channel model for fc > 2.5 GHz.

Table 3. Reference PDP data for CDL-C UMi channel model for fc > 2.5 GHz.

| Delay [ns] | Power [dB] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | X2V | H11 | H21 | H12 | H22 |
| 0.0 | -26.0 | -26.9 | -25.5 | -25.6 | -27.0 |
| 21.0 | -24.9 | -25.2 | -24.5 | -25.1 | -26.0 |
| 21.8 | -19.3 | -19.8 | -19.0 | -19.2 | -20.0 |
| 22.2 | -19.9 | -23.3 | -18.7 | -18.3 | -22.4 |
| 23.3 | -23.1 | -23.3 | -22.3 | -23.4 | -24.8 |
| 63.7 | 0.0 | 0.0 | 0.0 | -0.4 | -0.4 |
| 64.5 | -2.4 | -3.3 | -1.8 | -2.0 | -3.5 |
| 65.6 | -3.8 | -4.0 | -3.6 | -4.1 | -4.5 |
| 65.8 | -34.4 | -34.1 | -34.3 | -35.2 | -35.0 |
| 79.4 | -30.0 | -30.2 | -29.8 | -30.2 | -30.6 |
| 82.1 | -46.1 | -47.3 | -45.2 | -45.5 | -47.7 |
| 93.4 | -51.3 | -51.2 | -51.2 | -51.9 | -51.9 |
| 122.9 | -37.4 | -37.1 | -37.1 | -38.3 | -38.3 |
| 130.8 | -31.1 | -31.3 | -31.0 | -31.3 | -31.7 |
| 217.0 | -35.8 | -36.4 | -35.4 | -35.6 | -36.6 |
| 271.1 | -41.7 | -41.8 | -41.6 | -42.1 | -42.3 |
| 425.9 | -39.8 | -39.9 | -39.6 | -40.1 | -40.4 |
| 460.0 | -38.3 | -38.7 | -38.1 | -38.3 | -39.0 |
| 549.0 | -52.4 | -52.5 | -52.2 | -52.7 | -53.0 |
| 560.8 | -43.1 | -43.5 | -42.9 | -43.2 | -43.8 |
| 630.7 | -40.1 | -40.4 | -40.0 | -40.3 | -40.8 |
| 663.7 | -46.5 | -46.4 | -46.4 | -47.1 | -47.1 |
| 704.3 | -71.5 | -72.3 | -70.6 | -71.3 | -73.3 |
| 865.2 | -62.8 | -63.2 | -62.5 | -62.8 | -63.5 |

Table 4. Temporal auto-correlation data for CDL-C UMi channel model for fc > 2.5 GHz.

| lambda | X2V | H11 | H21 | H12 | H22 |
| --- | --- | --- | --- | --- | --- |
| 0.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 |
| 0.16 | 0.99 | 0.99 | 0.99 | 0.99 | 0.99 |
| 0.31 | 0.97 | 0.98 | 0.96 | 0.96 | 0.98 |
| 0.47 | 0.94 | 0.96 | 0.92 | 0.92 | 0.95 |
| 0.63 | 0.90 | 0.93 | 0.88 | 0.87 | 0.93 |
| 0.78 | 0.86 | 0.90 | 0.82 | 0.81 | 0.89 |
| 0.94 | 0.81 | 0.87 | 0.76 | 0.74 | 0.86 |
| 1.09 | 0.75 | 0.84 | 0.70 | 0.67 | 0.82 |
| 1.25 | 0.70 | 0.80 | 0.63 | 0.60 | 0.77 |
| 1.41 | 0.64 | 0.76 | 0.57 | 0.53 | 0.72 |
| 1.56 | 0.59 | 0.72 | 0.52 | 0.47 | 0.67 |
| 1.72 | 0.54 | 0.68 | 0.48 | 0.41 | 0.62 |
| 1.88 | 0.50 | 0.64 | 0.44 | 0.37 | 0.57 |
| 2.03 | 0.46 | 0.60 | 0.42 | 0.34 | 0.53 |
| 2.19 | 0.43 | 0.56 | 0.40 | 0.32 | 0.48 |
| 2.34 | 0.40 | 0.53 | 0.39 | 0.31 | 0.44 |
| 2.50 | 0.38 | 0.50 | 0.38 | 0.31 | 0.39 |
| 2.66 | 0.36 | 0.46 | 0.36 | 0.30 | 0.36 |
| 2.81 | 0.33 | 0.43 | 0.34 | 0.28 | 0.32 |
| 2.97 | 0.30 | 0.39 | 0.31 | 0.27 | 0.29 |
| 3.13 | 0.27 | 0.35 | 0.27 | 0.24 | 0.26 |
| 3.28 | 0.24 | 0.32 | 0.24 | 0.22 | 0.23 |
| 3.44 | 0.21 | 0.28 | 0.20 | 0.19 | 0.21 |
| 3.59 | 0.17 | 0.25 | 0.16 | 0.16 | 0.20 |
| 3.75 | 0.14 | 0.21 | 0.13 | 0.13 | 0.19 |
| 3.91 | 0.11 | 0.18 | 0.10 | 0.10 | 0.18 |
| 4.06 | 0.08 | 0.15 | 0.08 | 0.09 | 0.18 |
| 4.22 | 0.06 | 0.12 | 0.08 | 0.09 | 0.18 |
| 4.38 | 0.04 | 0.10 | 0.08 | 0.09 | 0.18 |
| 4.53 | 0.04 | 0.08 | 0.08 | 0.09 | 0.18 |
| 4.69 | 0.03 | 0.07 | 0.09 | 0.09 | 0.18 |
| 4.84 | 0.03 | 0.06 | 0.09 | 0.09 | 0.18 |
| 5.00 | 0.03 | 0.05 | 0.09 | 0.08 | 0.18 |

Figure 8. Reference PDP figure for 2nd beam of CDL-C UMa channel model for fc ≤ 2.5 GHz.

Figure 9. Reference Temporal auto-correlation curves for the 2nd beam of CDL-C UMa for fc ≤ 2.5 GHz.

Table 5. Reference PDP data for the 2nd beam of CDL-C UMa channel model for fc ≤ 2.5 GHz.

| Delay [ns] | Power [dB] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | X2V | H11 | H21 | H12 | H22 |
| 0.0 | -22.4 | -23.9 | -21.6 | -21.7 | -24.0 |
| 76.6 | 0.0 | -0.1 | 0.0 | -0.3 | -0.4 |
| 79.4 | -10.8 | -11.6 | -10.5 | -10.5 | -12.0 |
| 81.0 | -2.6 | -3.4 | -1.9 | -2.3 | -3.9 |
| 85.0 | -3.6 | -3.9 | -3.4 | -3.8 | -4.4 |
| 232.4 | -23.3 | -23.4 | -24.2 | -23.7 | -23.0 |
| 235.4 | -17.0 | -17.2 | -15.6 | -17.2 | -19.7 |
| 239.4 | -23.3 | -23.5 | -22.2 | -23.6 | -25.4 |
| 240.3 | -40.6 | -40.0 | -40.4 | -41.8 | -41.3 |
| 289.6 | -24.1 | -24.7 | -24.3 | -24.0 | -24.9 |
| 299.8 | -41.4 | -42.4 | -41.2 | -40.9 | -42.0 |
| 340.8 | -40.7 | -41.0 | -40.4 | -40.9 | -41.4 |
| 448.4 | -27.8 | -27.9 | -27.1 | -28.2 | -27.7 |
| 477.5 | -26.1 | -26.5 | -25.9 | -26.2 | -26.8 |
| 792.2 | -39.6 | -40.9 | -39.0 | -38.9 | -40.8 |
| 989.3 | -43.9 | -43.9 | -43.7 | -44.4 | -44.6 |
| 1554.5 | -43.8 | -43.6 | -43.8 | -44.5 | -44.3 |
| 1679.1 | -36.7 | -36.9 | -36.6 | -37.0 | -37.4 |
| 2003.9 | -45.5 | -45.9 | -45.2 | -45.6 | -46.3 |
| 2046.8 | -44.1 | -44.5 | -43.9 | -44.3 | -44.8 |
| 2301.9 | -41.1 | -41.1 | -41.0 | -41.6 | -41.7 |
| 2422.7 | -58.1 | -59.1 | -57.5 | -57.7 | -59.4 |
| 2570.6 | -65.3 | -65.7 | -65.8 | -65.3 | -65.3 |
| 3158.1 | -66.0 | -66.5 | -65.8 | -65.9 | -66.5 |

Table 6. Temporal auto-correlation data for the 2nd beam of CDL-C UMa channel model for fc ≤ 2.5 GHz.

| lambda | X2V | H11 | H21 | H12 | H22 |
| --- | --- | --- | --- | --- | --- |
| 0.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 |
| 0.16 | 0.94 | 0.95 | 0.93 | 0.93 | 0.95 |
| 0.31 | 0.82 | 0.85 | 0.79 | 0.78 | 0.85 |
| 0.47 | 0.75 | 0.82 | 0.70 | 0.69 | 0.81 |
| 0.63 | 0.69 | 0.78 | 0.62 | 0.60 | 0.78 |
| 0.78 | 0.56 | 0.68 | 0.48 | 0.45 | 0.67 |
| 0.94 | 0.41 | 0.57 | 0.33 | 0.27 | 0.53 |
| 1.09 | 0.36 | 0.52 | 0.30 | 0.22 | 0.46 |
| 1.25 | 0.34 | 0.50 | 0.30 | 0.22 | 0.42 |
| 1.41 | 0.27 | 0.41 | 0.24 | 0.16 | 0.32 |
| 1.56 | 0.19 | 0.31 | 0.17 | 0.12 | 0.21 |
| 1.72 | 0.18 | 0.27 | 0.17 | 0.13 | 0.19 |
| 1.88 | 0.19 | 0.26 | 0.20 | 0.13 | 0.19 |
| 2.03 | 0.16 | 0.23 | 0.22 | 0.10 | 0.13 |
| 2.19 | 0.11 | 0.17 | 0.22 | 0.06 | 0.10 |
| 2.34 | 0.10 | 0.15 | 0.21 | 0.05 | 0.15 |
| 2.50 | 0.13 | 0.17 | 0.23 | 0.09 | 0.15 |
| 2.66 | 0.12 | 0.17 | 0.24 | 0.08 | 0.11 |
| 2.81 | 0.08 | 0.13 | 0.18 | 0.04 | 0.10 |
| 2.97 | 0.10 | 0.13 | 0.15 | 0.11 | 0.14 |
| 3.13 | 0.16 | 0.18 | 0.21 | 0.17 | 0.15 |
| 3.28 | 0.17 | 0.20 | 0.23 | 0.15 | 0.11 |
| 3.44 | 0.14 | 0.18 | 0.20 | 0.10 | 0.07 |
| 3.59 | 0.13 | 0.20 | 0.19 | 0.08 | 0.08 |
| 3.75 | 0.14 | 0.23 | 0.18 | 0.06 | 0.09 |
| 3.91 | 0.10 | 0.20 | 0.12 | 0.04 | 0.07 |
| 4.06 | 0.03 | 0.15 | 0.02 | 0.12 | 0.05 |
| 4.22 | 0.03 | 0.14 | 0.06 | 0.17 | 0.03 |
| 4.38 | 0.04 | 0.14 | 0.11 | 0.16 | 0.03 |
| 4.53 | 0.04 | 0.12 | 0.12 | 0.14 | 0.06 |
| 4.69 | 0.07 | 0.11 | 0.14 | 0.13 | 0.06 |
| 4.84 | 0.09 | 0.15 | 0.16 | 0.08 | 0.03 |
| 5.00 | 0.09 | 0.14 | 0.16 | 0.05 | 0.03 |

# Conclusion

The following observations and proposals were made in this contribution:

Observation 1: BS correlation for CDL-C UMa at ≤ 2.5 GHz is too high with the old beam directions above.

Proposal 1: Use beam direction Az: -21.82, El: -10 for the second beam for CDL-C UMa at ≤ 2.5 GHz.

# References

- R4-2107127, Reference Channel Emulation Curves, Keysight Technologies, 3GPP TSG-RAN WG4 Meeting # 98bis-e, April 2021

- R4-2106092, WF on NR MIMO OTA, vivo, CAICT, 3GPP TSG-RAN WG4 Meeting #98-bis-e, April 2021

- R4-2103913, WF on NR MIMO OTA, vivo, CAICT, 3GPP TSG-RAN WG4 Meeting #98-e, February 2021