3GPP TSG-RAN4 Meeting #99-e	R4-2111075

# Online, 19 – 27 May, 2021

Agenda item:	9.12.4.3

Source:	Apple

Title:	On timing requirements for NR NTN

Document for:	Discussion

# Introduction

In last RAN4 meeting, the WF[1] on timing requirements for NR NTN was agreed as below,

WF on UE specific TA estimation error - The UE specific TA estimation accuracy is counted into the UE transmit timing error requirement - UE specific TA estimation accuracy is FFS - FFS whether the UE specific TA estimation accuracy shall be also defined as a separate accuracy requirement - Specify UE behavior related to UE specific TA estimation and the detailed behavior is FFS - FFS on the update periodicity of UE specific TA value WF on timing requirement - UE initial transmit timing error (Te) - Te requirement in NTN is consist of: - Same types of errors as terrestrial UE e.g. DL timing estimation accuracy and UL timing setting accuracy. and; - UE specific estimation accuracy; - FFS on whether and how much different relaxations are required for different sets of SCS of SSB and SCS of uplink signals - It is the total NTN UE Te error that decides UL performance, no matter the source of inaccuracy. - N_TA_offset - The existing N_TA offset value defined in Table 7.1.2-2 in TS38.133 can be reused in NTN - Gradual timing adjustment - FFS whether to define new gradual timing adjustment requirements for NTN UE - FFS whether and how to count the maximum delay variation for the round trip delay; - FFS: whether define different requirements for different NTN topologies in terms of, e.g. GEO, MEO, LEO, HAPS, HIBS, altitude, elevation angles for feeder/service links, UE speed, etc; - FFS the reference timing for the Gradual timing adjustment in NTN - One shot timing adjustment - Not introduce one shot timing adjustment requirement for NTN UE WF on TA adjustment accuracy requirement - In RRC_idle mode - FFS whether to define TA adjustment accuracy requirement; - In RRC_CONNECTED mode - Option 1: Reuse the existing TA adjustment accuracy requirement defined in TS 38.133 with considering of UL timing quantization accuracy. - Option 2: FFS on whether relax the TA adjustment accuracy requirement. - FFS on UE position and satellite position estimation error; - FFS on propagation delay change from a slot when UE received timing advance command to a slot when the indicated TA.

In this contribution, we discuss timing requirements for NR NTN.

# UE specific TA estimation error

Regarding the UE specific TA estimation, it was agreed that UE specific TA estimation accuracy is counted into the UE transmit timing error requirement. Since the UE specific TA estimation is based on the RTT derived from UE position and satellite position, the UE specific TA estimation accuracy is up to the GNSS positioning accuracy and ephemeris accuracy. In TS38.171, the worst case of minimum requirement for GNSS positioning accuracy has been defined as below,

Since different UE has different GNSS implementations and this NTN WI is not targeted to specify the UE GNSS implementation, we propose to use the worst case of GNSS positioning accuracy requirement in TS38.171 as baseline to define the UE timing requirement in NTN.

Proposal 1: use the worst case of GNSS positioning accuracy requirement (i.e., 2-D position error = 100m) in TS38.171 as baseline to define the UE timing requirement in NTN.

As agreed in RAN1, the ephemeris information could be:

Agreement: Support serving-satellite ephemeris broadcast based on one or more of the following: - Set 1: Satellite position and velocity state vectors: - position X,Y,Z in ECEF (m) - velocity VX,VY,VZ in ECEF (m/s) - Set 2: At least the following parameters in orbital parameter ephemeris format: - Semi-major axis α [m] - Eccentricity e - Argument of periapsis ω [rad] - Longitude of ascending node Ω [rad] - Inclination i [rad] - Mean anomaly M [rad] at epoch time to - FFS: Whether pre-provisioned ephemeris based on orbital elements can be used as reference. Thereby, only delta corrections can be broadcast in order to reduce the overhead - FFS: The field size for each parameter - FFS: The impact on signaling due to the required accuracy of serving-satellite ephemeris - FFS: Whether down-selection is needed or both sets are supported Conclusion: The orbital propagator model to be used at UE side can be left to implementation.

The ephemeris information is broadcasted by network and therefore the ephemeris accuracy information is also up to network implementation. UE could use the obtained ephemeris information to calculate the satellite position, but RAN1 agreed that orbital propagator model to be used at UE side can be left to implementation.

With 100m UE GNSS positioning error, the RTT error in the worst case could be 2*100m/c = 20.48*64*Tc, and the ephemeris information for satellite position is up to both network implementation and UE implementation.

Proposal 2: UE specific TA estimation accuracy is 20.5*64*Tc + Tephemeris_uncertainty. Tephemeris_uncertainty is the satellite position error due to ephemeris information and UE calculation.

As in proposal 2, the UE specific TA estimation is up to UE/network implementation. However, RAN2 had following agreement in the last meeting that,

Agreements: 1. At least for uplink scheduling adaptations, the UE may report information about the UE specific TA pre-compensation. The exact information and frequency of reports depend on RAN1 outcome. FFS on when/how to report. 2. The UE reports the UE specific TA pre-compensation during RACH procedure using MAC CE (FFS if this needs to be configured). Actual content is FFS and also depends on further RAN1 input.

So, if eventually RAN2 or RAN1 has such reporting mechanism for UE specific TA pre-compensation, we could consider to have such separated requirement for UE specific TA estimation. But at this stage we could directly define the RRM requirement which contains UE specific TA estimation.

Proposal 3: wait RAN1/RAN2 conclusions on UE specific TA pre-compensation reporting to determine whether we need to define separate UE specific TA estimation requirement or not.

The UE behavior related to UE specific TA estimation, e.g., estimation periodicity, is up to couple factors, i.e., GNSS measurement periodicity, ephemeris update rate, and UE calculation scheme for satellite position. All of those factors are up to UE and network implementation, we do not think it’s necessary to define UE behavior for UE specific TA estimation as a requirement as long as UE can meet the timing requirement, i.e., Te/Tq/Tp.

Proposal 4: No need to define UE behavior for UE specific TA estimation (e.g., estimation periodicity) as a requirement, as long as UE can meet the timing requirement, i.e., Te/Tq/Tp.

# Timing requirement for NTN

Regarding the Te requirement, it was agreed in last RAN1 meeting that,

Agreement: The Timing Advance applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED is given by: TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc Where: - NTA is defined as 0 for PRACH and updated based on TA Command field in msg2/msgB and MAC CE TA command. - FFS: details of NTA update/accumulation. - NTA,UE-specific is UE self-estimated TA to pre-compensate for the service link delay. - NTA,common is network-controlled common TA, and may include any timing offset considered necessary by the network. - NTA,common with value of 0 is supported. - FFS: details of signaling including granularity. - NTA,offset is a fixed offset used to calculate the timing advance. Note-1: Definition of NTA is different from that in RAN1#103-e agreement. Note-2: UE might not assume that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A. Note-3: NTA,common is the common timing offset X as agreed in RAN1 #103-e.

To RAN4, the uplink frame transmission takes place NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell. The legacy Te error mainly consists of DL timing estimation accuracy and UL timing setting accuracy. Now UE specific TA is another error source for Te requirement, as discussed in section 2, UE specific TA estimation accuracy is 20.5*64*Tc + Tephemeris_uncertainty. Tephemeris_uncertainty is the satellite position error due to ephemeris information and UE calculation. From our perspective, the half CP could be max tolerance for multiple path reception or multiple UE reception on UL to minimize the interference (ISI, ICI). However, the delay spread may impact the real arrival time on the UL as well. In TR38.811, the NTN channel delay spread has been discussed as below,

Here we use 250ns delay spread for FR1 in minimum requirement while we do not need delay spread for FR2 case. Thus, the max tolerance for multiple path reception or multiple UE reception on UL to minimize the interference (ISI, ICI) could be (half CP – 250ns) for FR1 and half CP for FR2. The NTN Te requirement with relaxation shall not exceed (half CP – 250ns (i.e., 8*64*Tc)) for FR1 and half CP for FR2 on UL.

Proposal 5: The NTN Te requirement with relaxation shall not exceed (half CP – 8*64*Tc) for FR1 and half CP for FR2 on UL.

The Tephemeris_uncertainty in UE specific TA estimation accuracy is up to network and UE implementation, however, if the ephemeris information is ideal based on network broadcast, the UE mathematical calculation would not have additional error. The common TA is also broadcasted by network, but we did not take into account the common TA error; so similarly here we do not need to consider the Tephemeris_uncertainty in the timing requirement.

Proposal 6: when ephemeris information is used to derive UE specific TA in Te requirement, the error due to ephemeris uncertainty shall not be considered.

With proposal 6, the Te requirement for NTN shall be equivalent to: legacy Te + UE specific TA estimation error (without ephemeris uncertainty), and that is: legacy Te + 20.5*64*Tc. So, based on proposal 5, the Te requirement for NTN shall be defined as:

- FR1 NTN Te requirement: min{(legacy Te + 20.5*64*Tc), (half CP – 8*64*Tc)}

- FR2 NTN Te requirement: min{(legacy Te + 20.5*64*Tc), half CP }

The cap of the Te requirement is summarized as in following table 1.

Table 1. Cap of NTN Te

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Legacy Te | Half CP minus 8*64*Tc for FR1 | Half CP for FR2 |
| --- | --- | --- | --- | --- | --- |
| 1 | 15 | 15 | 12*64*Tc | 64*64*Tc | - |
|  |  | 30 | 10*64*Tc | 28*64*Tc | - |
|  |  | 60 | 10*64*Tc | 10*64*Tc | - |
|  | 30 | 15 | 8*64*Tc | 64*64*Tc | - |
|  |  | 30 | 8*64*Tc | 28*64*Tc | - |
|  |  | 60 | 7*64*Tc | 10*64*Tc | - |
| 2 | 120 | 60 | 3.5*64*Tc | - | 18*64*Tc |
|  |  | 120 | 3.5*64*Tc | - | 9*64*Tc |
|  | 240 | 60 | 3*64*Tc | - | 18*64*Tc |
|  |  | 120 | 3*64*Tc | - | 9*64*Tc |

Besides the above analysis, we shall also follow that FR1 Te cannot be smaller than FR2 Te as in legacy NR. So we have the following proposal.

Proposal 7: the Te requirement for NTN is defined by:

- FR1 NTN Te requirement: min{(legacy Te + 20.5*64*Tc), (half CP – 8*64*Tc)}

- FR2 NTN Te requirement: min{(legacy Te + 20.5*64*Tc), half CP }

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te for NTN | Note |
| --- | --- | --- | --- | --- |
| 1 | 15 | 15 | 32.5*64*Tc | min{(legacy Te + 20.5*64*Tc), (half CP – 8*64*Tc)} note: 60kHz FR1 Te is not smaller than FR2 60kHz Te |
|  |  | 30 | 28*64*Tc |  |
|  |  | 60 | 18*64*Tc |  |
|  | 30 | 15 | 32.5*64*Tc |  |
|  |  | 30 | 28*64*Tc |  |
|  |  | 60 | 18*64*Tc |  |
| 2 | 120 | 60 | 18*64*Tc | min{(legacy Te + 20.5*64*Tc), half CP } |
|  |  | 120 | 9*64*Tc |  |
|  | 240 | 60 | 18*64*Tc |  |
|  |  | 120 | 9*64*Tc |  |

Regarding the gradual timing adjustment, since the NTN frequency/time drifting and relative speed between UE and satellite could be significantly different from legacy TN scenario, it’s necessary to define new gradual timing adjustment (Tp/Tq) for NTN.

Proposal 8: RAN4 to define new gradual timing adjustment (Tp/Tq) for NTN.

Since the frequency/time drifting and relative speed between UE and satellite is unclear so far, we may need to wait for more conclusions from RF and RAN1. However, we propose to discuss the methodology of designing Tp/Tq requirement first. In order to define Tp and Tq, we need to understand the frequency/time drifting rate and then the time drifting value within 200ms could be derived. Secondly, the timing change due to UE and satellite position change could be derived from relative speed between UE and satellite. And thirdly, the digital RF margin shall be considered as well, i.e., 1.5*64*Tc. We did not see any motivation to make Tp and Tq different, and we prefer to follow the same way as in legacy NR, i.e., Tp=Tq.

Proposal 9: the design principle for gradual timing adjustment requirement is:

Tq=ceiling (Tdrift + 200ms*Vrelativect_granularity)*t_granularity + digRF_margin

Tp=Tq

Where,

Tdrift is the UE time drifting during 200ms;

Vrelative is the relative speed between UE and satellite

T_granularity is the UE UL timing granularity

digRF_margin is the margin for digital RF, i.e., 1.5*64*Tc.

# TA adjustment error:

In R15/16, TA adjustment is defined for the case when UE received the MAC CE command of TA. However, in IDLE mode it’s not possible for UE to receive MAC CE command, and therefore it’s not necessary to define TA adjustment accuracy requirement for IDLE mode.

Proposal 10: Not define TA adjustment accuracy requirement for NTN UE in IDLE mode.

In legacy NR connected mode, the TA adjustment error is up to the UE UL timing granularity, as summarized as below,

| UL Sub Carrier Spacing(kHz) | 15 | 30 | 60 | 120 |
| --- | --- | --- | --- | --- |
| UE Timing Advance adjustment accuracy | ±256 Tc | ±256 Tc | ±128 Tc | ±32 Tc |
| UL timing granularity | 4Ts | 4Ts | 2Ts for FR1 0.5Ts for FR2 | 0.5Ts |

Thus, the legacy NR TA adjustment accuracy requirement could be reused for NTN case.

Proposal 11:in RRC connected mode, the legacy NR TA adjustment accuracy requirement could be reused for NTN case.

# Conclusion

In this contribution, we discuss timing requirements for NR NTN.

Proposal 1: use the worst case of GNSS positioning accuracy requirement (i.e., 2-D position error = 100m) in TS38.171 as baseline to define the UE timing requirement in NTN.

Proposal 2: UE specific TA estimation accuracy is 20.5*64*Tc + Tephemeris_uncertainty. Tephemeris_uncertainty is the satellite position error due to ephemeris information and UE calculation.

Proposal 3: wait RAN1/RAN2 conclusions on UE specific TA pre-compensation reporting to determine whether we need to define separate UE specific TA estimation requirement or not.

Proposal 4: No need to define UE behavior for UE specific TA estimation (e.g., estimation periodicity) as a requirement, as long as UE can meet the timing requirement, i.e., Te/Tq/Tp.

Proposal 5: The NTN Te requirement with relaxation shall not exceed (half CP – 8*64*Tc) for FR1 and half CP for FR2 on UL.

Proposal 6: when ephemeris information is used to derive UE specific TA in Te requirement, the error due to ephemeris uncertainty shall not be considered.

Proposal 7: the Te requirement for NTN is defined by:

- FR1 NTN Te requirement: min{(legacy Te + 20.5*64*Tc), (half CP – 8*64*Tc)}

- FR2 NTN Te requirement: min{(legacy Te + 20.5*64*Tc), half CP }

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te for NTN | Note |
| --- | --- | --- | --- | --- |
| 1 | 15 | 15 | 32.5*64*Tc | min{(legacy Te + 20.5*64*Tc), (half CP – 8*64*Tc)} note: 60kHz FR1 Te is not smaller than FR2 60kHz Te |
|  |  | 30 | 28*64*Tc |  |
|  |  | 60 | 18*64*Tc |  |
|  | 30 | 15 | 32.5*64*Tc |  |
|  |  | 30 | 28*64*Tc |  |
|  |  | 60 | 18*64*Tc |  |
| 2 | 120 | 60 | 18*64*Tc | min{(legacy Te + 20.5*64*Tc), half CP } |
|  |  | 120 | 9*64*Tc |  |
|  | 240 | 60 | 18*64*Tc |  |
|  |  | 120 | 9*64*Tc |  |

Proposal 8: RAN4 to define new gradual timing adjustment (Tp/Tq) for NTN.

Proposal 9: the design principle for gradual timing adjustment requirement is:

Tq=ceiling (Tdrift + 200ms*Vrelativect_granularity)*t_granularity + digRF_margin

Tp=Tq

Where,

Tdrift is the UE time drifting during 200ms;

Vrelative is the relative speed between UE and satellite

T_granularity is the UE UL timing granularity

digRF_margin is the margin for digital RF, i.e., 1.5*64*Tc.

Proposal 10: Not define TA adjustment accuracy requirement for NTN UE in IDLE mode.

Proposal 11:in RRC connected mode, the legacy NR TA adjustment accuracy requirement could be reused for NTN case.

# References

[1] R4-2105796, WF on timing requirements for NR NTN, Xiaomi, RAN4 #98bis-e