3GPP TSG-RAN WG4 meeting #99-e	R4-2111251

Electronic Meeting, 19th – 27th May 2021

Agenda Item:	5.2.2.3.1

Source:	Ericsson

Title:	Discussions on RRM maintenance issues for Rel-16 eMTC UE

Document for:	Discussion

# 1	Introduction

In this contribution we discuss the RSS based RSRQ measurement for release 16 eMTC based on the incoming LSs.

# 2	Discussions

RAN4 has received three LS responses related to RSS measurement [1, 2]. The first LS response [1] contains RAN2 replies to the questions about neighbor cell assumptions with regarding to the timing. It does not require any further action except that RAN4 needs to take them into account when defining the neighbor cell measurements.

The second LS is related to the RSS based RSRQ measurement. On this issue, RAN2 has replied as follows [2]:

The options proposed by RAN4 are: - Option 1: Remove RSRQ from the cell selection and cell re-selection criterion when a cell is measured using RSS. - Option 2: Define RSRQ for RSS measurements RAN2 has discussed the options listed in the LS and concluded that from RAN2 perspective option 1 is not preferred because it may have an impact on cell (re)-selection performance and behaviour, but it is up to RAN1 and RAN4 to decide.

Firstly, it is noted that the work item objective only targets improving of the DL RSRP measurement accuracy through use of RSS, see below [3]:

Mobility enhancement: - Improve the DL RSRP measurement accuracy through use of RSS [RAN1, RAN4, RAN2] - Specify relaxation of RRM measurements for serving cell for UEs using WUS for at least low mobility UEs [RAN4, RAN2]

- Observation #1: Rel-16 eMTC WI objective only includes improving the DL RSRP measurement accuracy through use of RSS, i.e. RSRQ is not included.

Secondly, developing the RSS based RSRQ measurement requirements requires RAN4 to study whether RSRQ is feasible given that RSS is transmitted only on 2 PRBs. This can be compared to the CRS based RSRQ measurement which is measured over the measurement bandwidth of 6 PRBs. For example, RAN4 needs to study whether RSRQ over 2 PRBs can correctly reflect actual channel quality, and also look into the risk of overestimating or underestimating the channel quality given the very short measurement bandwidth. Such study needs to be done using simulation campaign and may require several meetings. Given that release 16 WI is already completed and the work of introducing RSRQ needs to be carry out under TEI work item.

- Observation #2: Rel-16 WI on eMTC is completed and thus RSRQ measurement support through use of RSS (if introduced) has be done under TEI work item.

Nevertheless, only small technical enhancements and improvements can be done TEI work item according to the new TEI rules agreed in [4], see below:

A. TEI Work Item codes shall only be used for small technical enhancements and improvements. B. A TEI CR set shall be fully completed within one TSG cycle/quarter in all affected WGs.

- Observation #3: Only small technical enhancement work is allowed under TEI work item.

As explained above, introducing of RSRQ requires a lot more work and cannot be considered as a small technical enhancements since RAN4 has to first agree on the simulation assumptions, evaluate the feasibility of RSRQ and thereafter (if found feasible) develop the detailed requirements. All this work cannot be considered as a small technical enhancement and cannot be done within one TSG cycle/quarter following the MCC guidelines. Therefore, our view is to not introduce RSS based RSRQ measurement under TEI work item, instead the corresponding release 17 work item can be updated to include RSRQ (if needed), and this requires RAN plenary decision.

- Observation #4: The work of introducing RSS based RSRQ can be divided into following:

  - Agreeing on simulation assumptions

  - Evaluating the feasibility of RSRQ

  - If found feasible, develop detailed requirements

Based on the discussions above, following is proposed:

- Proposal: RSS based RSRQ shall not be introduced under TEI work for release 16 eMTC.

RAN4 shall send LS to RAN2 informing about the RAN4 agreement to no introduce RSS based RSRQ measurement in release 16. A draft LS is provided in the Appendix.

# 3		Summary

In this contribution we have discussed RSS based RSRQ measurement for release 16 eMTC. Based on the discussions, we have made following observations and proposal:

- Observation #1: Rel-16 eMTC WI objective only includes improving the DL RSRP measurement accuracy through use of RSS, i.e. RSRQ is not included.

- Observation #2: Rel-16 WI on eMTC is completed and thus RSRQ measurement support through use of RSS (if introduced) has be done under TEI work item.

- Observation #3: Only small technical enhancement work is allowed under TEI work item.

- Observation #4: The work of introducing RSS based RSRQ can be divided into following:

  - Agreeing on simulation assumptions

  - Evaluating the feasibility of RSRQ

  - If found feasible, develop detailed requirements

- Proposal: RSS based RSRQ shall not be introduced under TEI work for release 16 eMTC.

# References

- R2-2104391, "Reply LS on timing of neighbor cell RSS-based measurements", RAN2

- R2-2104392, "Reply LS related to RSS based RSRQ for LTE-MTC", RAN2

- RP-192875, "Revised WID: Additional MTC enhancements for LTE", Ericsson

- RP-210826, "Handling of TEI CRs", ETSI MCC

# Appendix

3GPP TSG-RAN WG4 Meeting # 99-e    				  R4-210xxxx

Electronic Meeting, 19th – 27th May 2021

Title:	LS on RAN4 agreement on RSS based RSRQ measurement for cat-M

Response to:

Release:	Rel-16

Work Item:	LTE_eMTC5

Source:	RAN WG4

To:	RAN WG2

Cc:

Contact Person:

#### Name:	Santhan Thangarasa

#### E-mail Address:	santhan.thangarasa@ericsson.com

Attachments:

1. Overall Description:

RAN4 would like to inform RAN2 about the RAN4 agreement on whether to introduce RSS based RSRQ measurement for cat-M in release 16. RAN4 has discussed and reached the conclusion to not include RSS based RSRQ measurement for cat-M in release 16.

2. To RAN WG2 group.

ACTION: 	RAN4 respectfully asks RAN2 to take the above agreements into account.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN4 Meeting #100-e		Electronic Meeting

TSG-RAN4 Meeting #101-e		Electronic Meeting