3GPP TSG-RAN WG4 Meeting#98                	R4-2111274

E-meeting, May 19th – 27th, 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	Direct SCell activation delay

Agenda item:	6.4.2.1

Document for:	Agreement

Introduction

In last meeting RAN4#98bis meeting the final core requirements related to the direct activation delay were agreed. Agreement related to the FR1 SCell known condition and measurement condition for direct SCell activation at SCell addition and Handover.

Two number were left in [] which we discuss in this paper.

Discussion

SCell known condition for direct activated NR FR1 SCell

Firstly, we discuss the time delay condition for the known SCell in FR1:

The SCell is known provided the following conditions are met for the SCell:

- During the last [5] seconds before the reception of the direct SCell configuration command:

- the UE has sent a valid measurement report for the SCell being directly activated, and

- the SSB measured remains detectable according to the cell identification conditions specified in sections 9.2 and 9.3,

- the SSB measured during the period equal to [5] seconds also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3.

Otherwise, the SCell is unknown.

In LTE the core requirements are defined for direct SCell activation. LTE use 5 seconds as condition as being the delay between UE sending the measurement report for the SCell being activated and the receiving the configuration including the direct SCell activation.

As FR1 requirements are to a great extend based on the LTE requirements we believe the existing 5 seconds as defined in LTE can be readily applied also to NR.

The 5 seconds used as condition and defined in LTE can be readily applied also to NR

Remove the [] around [5] in 38.133.

Tactivation_time for Direct activated NR FR1 SCell

In last meeting RAN4 agreed:

If the SCell is known and belongs to FR1, TCSI_Reporting is specified in clause 8.3.2 and Tactivation_time is defined as:

-	TFirstSSB+ 5ms, if the measurement period is equal to or smaller than [1280ms].

-	TFirstSSB_MAX + Trs + 5ms, if measurement period is larger than [1280]ms.

RAN4 should consider two cases:

- Target SCell has been measured with inter-frequency measurement and measurement period

- Target SCell has been measured by a deactivated SCell and measurement period

If looking at normal inter-frequency measurement period for FR1:

| Condition NOTE1,2 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600ms, 8 Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(600ms, Ceil(8*1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | 8 DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. |  |

Assuming typical SMTC of 20ms, and MGRP of 40ms this would for the best case when CSSF=1 in No DRX give a measurement period of 600ms assuming 1 carrier is measured.

In case DRX is in use the delay in many cases increases. Assuming same setting as in No DRX case and DRX of 80ms the measurement period equals 960ms.

Any larger DRX cycle, e.g. 160ms, would lead to a measurement period larger than 1280ms after which the activation delay is increased. This means, in order to keep the direct SCell activation delay short the network would have to use optimize the UE configuration such the inter-frequency measurement period of the target cell is below 1280ms. This can of course have negative impact on the overall UE power consumption. Hence, we suggest increasing the 1280ms to match the number used for known SCell (i.e. 5 seconds).

Looking at the measurement periods for a deactivated SCell are:

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | 5 x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | 5 x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | 5 x max(measCycleSCell, DRX cycle) x CSSFintra |

where measCycleSCell can be configured as:

measCycleSCell ENUMERATED {sf160, sf256, sf320, sf512, sf640, sf1024, sf1280}

for this case, if the network would want to keep the measurement period below the 1280ms to ensure short cirect activation delay, the network would not be able to use a measCycleSCell larger than 256ms for when no DRX is in use. If DRX is used it is most likely the used DRX cycle cannot be larger than 160ms as it would otherwise lead to that the measurement period would be larger than 1280ms (1.5*160*5). This is of course a network configuration option – however, also for this case it can have negative impact on the UE power consumption.

Based on above we in general propose:

Increase the measurement period threshold used for Tactivation_time for direct NR FR1 SCell activation from 1280ms.

RAN4 to define the measurement period threshold equal 5 seconds.

Conclusion

In this paper we discuss the final core requirements related to the direct activation delay. We discuss the numbers related to the FR1 SCell known condition and measurement period condition for direct SCell activation at SCell addition and Handover.

- The 5 seconds used as condition and defined in LTE can be readily applied also to NR

Remove the [] around [5] in 38.133.

Increase the measurement period threshold used for Tactivation_time for direct NR FR1 SCell activation from 1280ms.

RAN4 to define the measurement period threshold equal 5 seconds.

We have also provided a CR capturing the proposals.

References

- R4-2105739, CR for core requirement maintenance on direct SCell activation, Apple

- R4-2105740, Draft CR Correction of activation delay for Direct activated SCell, Nokia, Nokia Shanghai Bell