# 3GPP TSG-RAN WG4 Meeting # 98-bis-e 	R4-2111466

# Electronic Meeting, Apr. 12-20, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.175 | CR |  | rev |  | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.175: IAB EMC test configurations and performance requirements (updated) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Perf |  |  |  |  |  | Date: |  |  | 2021-05-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Last meeting the IAB EMC test configurations and performance requirements were discussed, based on R4-2106511 from Ericsson. During the second round, all the comments shared were not addressed. For sake of progress, we re-submit (part of the) CR version which was commented during the second round last meeting, with some additional text clarifications introduced in section 6.2. Please note that there are still some unresolved [] which may require coordination with the RF session, e.g. interpretation of the “uplink” and “downlink” for IAB node. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of test configurations and performance requirements in IAB EMC specification TS 38.175. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of tests configurations and performance requirements to proceed with IAB EMC testing. |  |  |  |  |  |  |  |  |
|  |  | 6.1, 6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

- ------------------------------ Start of new section ------------------------------

## 6.1	Performance criteria for continuous phenomena for IAB

The test should, where possible, be performed using a bearer with the characteristics of data rate and throughput defined in table 6.1-1 and table 6.1-2. If the test is not performed using one of these bearers (for example, none of them are supported by the IAB node), the characteristics of the bearer used shall be recorded in the test report.

The throughput in table 6.1-1 and table 6.1-2 is stated relative to the maximum throughput of the FRC.

The IAB node [uplink and downlink] paths shall each meet the performance criteria defined in table 6.1-1 and table 6.1-2 during the test. If the uplink and downlink paths are evaluated as a one loop then the criteria is two times the throughput reduction shown in table 6.1-1 and table 6.1-2 (i.e. throughput > 90 % instead of throughput > 95 %). After each test case IAB node shall operate as intended with no loss of user control function, stored data and the communication link to [both UE and donor test equipments] shall be maintained.

Table 6.1-1: FR1 performance criteria for continuous phenomena for IAB

| NR channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Bearer information data rate for IAB-MT | Bearer information data rate for IAB-DU | Performance criteria (Note 1, Note 2) |
| --- | --- | --- | --- | --- |
| 10, 15 | 30 | G-FR1-A1-22 in annex A.1 in TS 38.174 [2] | G-FR1-A1-2 in annex A.1 in TS 38.104 [22] | Throughput > 95 %, no loss of service |
| 10, 15 | 60 | G-FR1-A1-23 in annex A.1 in TS 38.174 [2] | G-FR1-A1-3 in annex A.1 in TS 38.104 [22] |  |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-25 in annex A.1 in TS 38.174 [2] | G-FR1-A1-5 in annex A.1 in TS 38.104 [22] |  |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-26 in annex A.1 in TS 38.174 [2] | G-FR1-A1-6 in annex A.1 in TS 38.104 [22] |  |
| NOTE 1: The performance criteria, throughput > 95 %, no loss of service, applies also if a bearer with another characteristics is used in the test. NOTE 2: The performance criteria, throughput > 90 %, no loss of service, applies instead if the uplink and downlink paths are evaluated as a one loop. |  |  |  |  |

Table 6.1-2: FR2 performance criteria for continuous phenomena for IAB

| NR channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Bearer information data rate for IAB-MT | Bearer information data rate for IAB-DU | Performance criteria (Note 1, Note 2) |
| --- | --- | --- | --- | --- |
| 50, 100, 200 | 60 | G-FR2-A1-21 in annex A.1 in TS 38.174 [2] | G-FR2-A1-1 in annex A.1 in TS 38.104 [22] | Throughput > 95 %, no loss of service |
| 50 | 120 | G-FR2-A1-22 in annex A.1 in TS 38.174 [2] | G-FR2-A1-2 in annex A.1 in TS 38.104 [22] |  |
| 100, 200, 400 | 120 | G-FR2-A1-23 in annex A.1 in TS 38.174 [2] | G-FR2-A1-3 in annex A.1 in TS 38.104 [22] |  |
| NOTE 1: The performance criteria, throughput > 95 %, no loss of service, applies also if a bearer with another characteristics is used in the test. NOTE 2: The performance criteria, throughput > 90 %, no loss of service, applies instead if the uplink and downlink paths are evaluated as a one loop. |  |  |  |  |

## 6.2	Performance criteria for transient phenomena for IAB

[At the conclusion of each exposure the IAB node shall operate with no user noticeable loss of the communication link.

At the conclusion of the total test (comprising the series of individual exposures to transient phenomena) the IAB node shall operate as intended with no user noticeable loss of control functions or stored data. At the conclusion of each exposure the IAB node shall operate with no user noticeable loss of

the communication link to [both UE and donor test equipments].

The channel bandwidth, sub-carrier spacing and bearer information data rate should be chosen based on manufacturer’s declaration and defined according to table 6.1-1 and table 6.1-2.

------------------------------ End of section ------------------------------

----------------------------- End of proposed text ------------------------------