3GPP TSG-RAN WG4 Meeting#99-e             	R4-2111530

E-meeting, 19th – 27th May 2021

Title:	Remaining n8 n71 Asymmetric Uplink Downlink Requirements

Source:	Skyworks Solutions Inc.

Agenda item:	8.28.2 UE RF requirements

Document for:	Approval

# 1	Introduction

This document makes proposals on the remaining missing requirements for bands n8 and n71 asymmetric Uplink (UL) 20MHz / Downlink (DL) 35MHz channel bandwidth (CBW) operation based on last meeting’s WF agreements [1]. We also discuss associated parameter “ΔFTX-RX” and propose minor changes to remove any ambiguity of its applicability.

# 2	Discussion

### 2.1	REFSENS Test Point Selection

In WF [1], the “mid-case” REFSENS test points have been agreed for n8 and n71 UL 20MHz/ DL 35MHz operation. The “mid-case” terminology refers to the scenario where the UL modulated carrier is configured at the default Tx-Rx frequency separation distance from their respective DL carrier frequencies as shown in Table 1 below.

Table 1 Two Antenna Port asymmetric 20MHz Uplink / 35MHz Downlink Reference Sensitivity QPSK PREFSENS and Uplink configuration table for n8 and n71 from WF [1] (with footnote 2 addition).

| Operating band / SCS / Channel bandwidth / Channel allocations / REFSENS/ Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS (kHz) | Channel bandwidth (DL) (MHz) | Channel bandwidth (UL) (MHz) | FC (DL) (MHz) | FC (UL) (MHz) | UL allocation (LCRB) | REFSENS (dBm) | Duplex Mode |
| n8 | 15 | 35 | 20 | 942.5 | 897.5 | 252 | [-78.4] | FDD |
|  | 30 |  |  |  |  | 102 | [-78.5] |  |
|  | 60 |  |  |  |  |  |  |  |
| n71 | 15 | 35 | 20 | 634.5 | 680.5 | 252 | [-80.7] | FDD |
|  | 30 |  |  |  |  | 102 | [-80.8] |  |
|  | 60 |  |  |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 2: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1) |  |  |  |  |  |  |  |  |

In this contribution we bring our views on how to address the remaining open points that we brought during last RAN4 #98-bis-e meeting and captured in WF [1].

### 2.1	Impact on Asymmetric Channel Bandwidths

The impact of UL 20MHz/DL 35MHz introduction in TS 38.101-1 on subclause “5.3.6 Asymmetric channel bandwidths” has not been previously considered. Also, since 35MHz CBW is only applicable to downlink operation, subclause “5.3.5 UE channel bandwidth per operating band” needs to be updated accordingly. We make proposal 1 in Table 2 and Table 3 for “Table 5.3.5-1 Channel bandwidths for each NR band” and “Table 5.3.6-1: FDD asymmetric UL and DL channel bandwidth combinations” respectively.

Proposal 1: Adopt changes highlighted in yellow in Table 2 and Table 3 for introduction of n8 and n71 asymmetric UL 20MHz/DL 35MHz operation.

Table 2: Proposal for Table 5.3.5-1 - Channel bandwidths for NR band n8 and n71.

| NR band / SCS / UE Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band |  | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n8 |  | 15 | Yes | Yes | Yes | Yes |  |  | Yes3,4 |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  | Yes3,4 |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n71 |  | 15 | Yes | Yes | Yes | Yes |  |  | Yes3,4 |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  | Yes3,4 |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NOTE 1: Void. NOTE 2: Void. NOTE 3: This UE channel bandwidth is applicable only to downlink. NOTE 4: This UE channel bandwidth is optional in this release of the specification. NOTE 5: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of DC or CA configuration. NOTE 6: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as a downlink SCell part of CA configuration. NOTE 7: For the 20 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 713-723 MHz or 728-738 MHz. For the 30MHz bandwidth, the minimum. requirements are specified for NR UL transmission bandwidth configuration confined to either 703-733 or 718-748 MHz. NOTE 8: This UE channel bandwidth is applicable only to uplink. NOTE 9: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of DC or CA configuration. NOTE 10: These UE channel bandwidths are applicable to sidelink operation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 3: Proposal “Table 5.3.6-1 FDD asymmetric UL and DL channel bandwidth combinations” for n8 and n71.

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) | Asymmetric channel bandwidth combination set |
| --- | --- | --- | --- |
| n8 | 20 | 35 | 0 |
| n71 | 5 | 10 | 0 |
|  | 10 | 15 |  |
|  | 15 | 20 |  |
|  | 5 | 10 | 1 |
|  | 10 | 15 |  |
|  | 15 | 20 |  |
|  | 20 | 35 |  |
| NOTE 1: The assignment of the paired UL and DL channels are subject to a TX-RX separation as specified in clause 5.4.4. |  |  |  |

About parameter ΔFTX-RX: it is defined as a deviation to the Tx-Rx carrier center frequency separation and is specified as ΔFTX-RX = | (BWDL – BWUL)/2 |. We observe that for the example of n71 UL 20MHz/ DL 35MHz, ΔFTX-RX is equal to 7.5MHz and the UE shall support the following deviations (all expressed in MHz units):

- “Best-case” UL 20/ DL 35: UL Fc = 688, DL Fc=634.5, deviation from n71 TX-RX separation = + ΔFTX-RX;

- “Mid-case” UL 20/ DL 35: UL Fc = 680.5, DL Fc=634.5, deviation from n71 TX-RX separation = 0, i.e. no deviation;

- “Worst-case” UL 20/ DL 35: UL Fc = 673.0, DL Fc=634.5, deviation from n71 TX-RX separation = - ΔFTX-RX.

With the adoption of “mid-case” (Table 1 – WF [1]) for which there is no deviation, there might be a risk of misinterpretation as this UL/DL configuration set may be considered contradictory to the ΔFTX-RX requirements. It is our understanding that parameter ΔFTX-RX was introduced in the early phases of Release 15 to ensure that a UE can be operated with asymmetric UL / DL CBW as long as the deviation from the default frequency separation is constrained and remains relatively small. As such, ΔFTX-RX should be considered as a maximum “range of deviation” from the band Tx-Rx frequency separation, a nuance that may not be explicit from the current specifications.

To remove any ambiguity when UL and DL are configured at the exact default Tx-Rx frequency separation, we propose to clarify the applicability of ΔFTX-RX in 38.101-1 by mentioning that ΔFTX-RX is a maximum deviation to the Tx-Rx carrier center frequency separation (as defined in table 5.4.4-1). By introducing “maximum deviation,” we believe it is clearer that the UE shall support a range of   ΔFTX-RX. We invite companies to provide their views on this proposal 2. If acceptable, it may also be worth adding this parameter to sub-clause 3.2 (symbols) – refer to proposal 3.

Proposal 2: Include the word “maximum” in the following text in sub-clause 5.3.6, as highlighted in yellow:

The UE channel bandwidth can be asymmetric in downlink and uplink. In asymmetric channel bandwidth operation, the narrower carrier shall be confined within the frequency range of the wider channel bandwidth.

In FDD, the confinement is defined as a maximum deviation to the Tx-Rx carrier center frequency separation (defined in table 5.4.4-1) as following:

ΔFTX-RX = | (BWDL – BWUL)/2 |

Proposal 3: Add text highlighted in yellow to sub-clause 3.2

ΔFTX-RX	Maximum deviation to the Tx-Rx carrier center frequency separation for asymmetric uplink/downlink channel bandwidth operation

# 3	Conclusion

In this contribution, we propose to introduce n8 and n71 35MHz DL BW operation as:

Proposal 1: Adopt changes highlighted in yellow in Table 2 and Table 3 for introduction of n8 and n71 asymmetric UL 20MHz/DL 35MHz operation.

Table 2: Proposal for Table 5.3.5-1 - Channel bandwidths for NR band n8 and n71.

| NR band / SCS / UE Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band |  | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n8 |  | 15 | Yes | Yes | Yes | Yes |  |  | Yes3,4 |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  | Yes3,4 |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n71 |  | 15 | Yes | Yes | Yes | Yes |  |  | Yes3,4 |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  | Yes3,4 |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NOTE 1: Void. NOTE 2: Void. NOTE 3: This UE channel bandwidth is applicable only to downlink. NOTE 4: This UE channel bandwidth is optional in this release of the specification. NOTE 5: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of DC or CA configuration. NOTE 6: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as a downlink SCell part of CA configuration. NOTE 7: For the 20 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 713-723 MHz or 728-738 MHz. For the 30MHz bandwidth, the minimum requirements are specified for NR UL transmission bandwidth configuration confined to either 703-733 or 718-748 MHz. NOTE 8: This UE channel bandwidth is applicable only to uplink. NOTE 9: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of DC or CA configuration. NOTE 10: These UE channel bandwidths are applicable to sidelink operation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 3: Proposal “Table 5.3.6-1 FDD asymmetric UL and DL channel bandwidth combinations” for n8 and n71.

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) | Asymmetric channel bandwidth combination set |
| --- | --- | --- | --- |
| n8 | 20 | 35 | 0 |
| n71 | 5 | 10 | 0 |
|  | 10 | 15 |  |
|  | 15 | 20 |  |
|  | 5 | 10 | 1 |
|  | 10 | 15 |  |
|  | 15 | 20 |  |
|  | 20 | 35 |  |
| NOTE 1: The assignment of the paired UL and DL channels are subject to a TX-RX separation as specified in clause 5.4.4. |  |  |  |

To remove any contradictory interpretation of parameter ΔFTX-RX specifications and proposed UL carrier frequency configuration, we propose the following change:

Proposal 2: Include the word “maximum” in the following text in sub-clause 5.3.6, as highlighted in yellow:

The UE channel bandwidth can be asymmetric in downlink and uplink. In asymmetric channel bandwidth operation, the narrower carrier shall be confined within the frequency range of the wider channel bandwidth.

In FDD, the confinement is defined as a maximum deviation to the Tx-Rx carrier center frequency separation (defined in Table 5.4.4-1) as follows:

ΔFTX-RX = | (BWDL – BWUL)/2 |

Proposal 3: Add text highlighted in yellow to sub-clause 3.2

ΔFTX-RX	Maximum deviation to the Tx-Rx carrier center frequency separation for asymmetric uplink/downlink channel bandwidth operation

# References

[1]	R4-2105364, WF on UE REFSENS for 35 MHz and 45 MHz, 3GPP TSG-RAN WG4 #98bis-e, Electronic Meeting, 12th – 20th Apr. 2021, Huawei, HiSilicon.