3GPP TSG-RAN WG4 Meeting # 99-e		R4-2107880

Electronic Meeting, May, 2021

Title:	Reply LS on PUCCH and PUSCH repetition

Response to:	LS R1-2104119 on PUCCH and PUSCH repetition

from RAN1

Release:	Rel-17

Work Item:	NR_cov_enh-Core

Source:	RAN4

To:	RAN1

Cc:

Contact person:	Ville Vintola

vvintola { a } qti.qualcomm.com

858- three six one - 8110

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

R4-2103393 Reply on LS on PUCCH and PUSCH repetition

R4-2105417	Reply LS  on PUCCH and PUSCH repetition

# 1	Overall description

RAN4 has continued discussing the gap between two consecutive repetitions and reached a conclusion that it is feasible for UE to maintain phase continuity when the gap is 13 symbols or less.  If new RF requirements for UE during the gap are needed is under discussion in RAN4.

Regarding questions from RAN1 in R1-2104119

- RAN1 Question 1:

  - In addition to the conditions provided in R4-2103393, can RAN4 please confirm that “Applying the same TPMI precoder across PUSCH transmissions” is also a necessary condition to keep phase continuity across PUSCH transmissions?

RAN4 Answer is that applying the same TPMI precoder across PUSCH transmissions is necessary condition for maintaining phase continuity.

- RAN1 Question 2:

  - Whether “no TA adjustment in between PUCCH transmissions or PUSCH transmissions” is another necessary condition to keep phase continuity across PUCCH repetitions or PUSCH transmissions?

RAN4 Answer is that

  - For network commanded TA adjustments:

    - It is known by both gNB side and UE side, and probably can be compensated by UE/BS.

    - FFS how to handle the TA inaccuracy.

  - For UE autonomous adjustments:

    - FFS how to handle autonomous adjustments, e.g., whether it can be compensated by UE.

- RAN1 Question 3:

  - There are two different interpretation in RAN1 regarding the “downlink reception” in “No downlink reception in-between the PUSCH or PUCCH repetition in the same band for TDD case” (in R4-2103393)

- “downlink reception” refers to downlink symbols with actual DL transmission from gNB to UE.

- “downlink reception” refers to downlink symbols with actual DL transmission from gNB to UE and/or downlink symbols without actual DL transmission from gNB to UE and/or no DL monitoring occasions configured.

Can RAN4 please confirm which interpretation is correct?

RAN4 Answer is that

- In general, in-between PUSCH/PUCCH transmissions, the UE shall not be required to receive any DL signal, meaning that the UE shall not be required to receive any DL transmission and shall not be required to perform any DL monitoring.

- The “downlink reception” at least includes downlink symbols with actual DL transmission from gNB to UE and/or DL monitoring.

- Regarding whether “downlink reception” include downlink symbols without actual DL transmission from gNB to UE and without DL monitoring, it would be helpful if RAN1 could provide more information on the exact scenario.

# 2	Actions

To RAN1

ACTION:

RAN4 asks RAN1 to take above information in to account in their work.

RAN4 also asks RAN1 to provide more information on the scenario when “downlink reception” from UE point of view includes downlink symbols without actual DL transmission from gNB to UE and UE is not assumed to do DL monitoring?

# 3	Dates of next WG4 meetings

TSG-RAN WG4 Meeting #100-e				Aug 2021		E-meeting

TSG-RAN WG4 Meeting #101-e				Nov 2021		E-meeting