3GPP TSG RAN WG4 Meeting #63 UE performance AH           R4-63AH-0186

Oulu, Finland, June 26th – 28th , 2012

Source: 	Broadcom Corporation

Title: 	Link Level Simulation Results for Evaluating Advanced Receivers

Agenda Item: 	7.2

Document for: 	Discussion

# Introduction

In RAN4 #61, link level assumptions of enhanced performance requirement for LTE UE study item were agreed in which the advanced receiver is an IRC receiver, based on correlation matrix estimation approximation [1]. The results of this study item were captured in [2]. During the RAN#55 meeting the work item for interference rejection was approved [3].

The following transmission modes were agreed as simulation assumptions [4]:

- CRS based transmission: TM2 as serving cell and TM3 as interfering cells

- CRS based transmission: TM6 as serving cell and TM4 as interfering cells

- CSI-RS based transmission: TM9 with single-layer as serving cell and TM9 with single/dual-layer as interfering cells

This contribution presents link level simulation results characterizing the gains of advanced receivers over MMSE implementations for Scenario 1 (TM2) and 2 (TM6), based on the agreed simulation assumptions in [5].

# Simulation Assumptions

Table 1 summarizes the different simulation assumptions which were agreed in [5] for different scenarios.

Table 1: Simulation assumptions for link level performance evaluation

| Parameter | Test 1 (TM2) | Test 2 (TM6) | Test 3 (TM9) |
| --- | --- | --- | --- |
| Carrier frequency | 2 GHz |  |  |
| System bandwidth | 10 MHz |  |  |
| Transmission mode in serving cell | TM2 | TM6 | TM9 with 1-layer |
| Transmission mode in interfering cells | TM3 | TM4 | TM9 |
| MIMO configuration | 2x2, [low] correlation See Note 1 | 2x2, low correlation | 4x2, low correlation |
| Channel model and Doppler frequency for target and interfering cells | [EVA70] | EVA5 | EVA5 |
|  | Use different channel seed for between cells |  |  |
| Number of interfering cells | 2 interfering cells | 2 interfering cells | Option 1: 2 interfering cells Option 2: 1 interfering cell (DIP1 is the same as option 1) |
| Geometry | Geometry range: [-8:1:6] dB |  |  |
| Simulation output for alignment | Sweep throughput vs. geometry (SINR), keeping DIP(s) fixed to agreed values |  |  |
| DIP values | DIP1=-1.73dB, DIP2=-8.66dB |  |  |
| CRS configuration | 2 CRS ports per cell with planning (non-colliding CRS between cells) |  |  |
| CSI reference signals | N/A | N/A | Antenna ports 15,…,18 |
| CSI-RS periodicity and subframe offset (TCSI-RS / ICSI-RS) | N/A | N/A | 5 / 2 |
| CSI reference signal configuration | N/A | N/A | 0 |
| Resource allocation | 50 PRBs | 50 PRBs | 50 PRBs |
|  |  |  | 41 PRBs in subfr.#0 (skip center 6 PRBs, allocated PRBs: RB0–RB20 and RB30–RB49) |
| Subframes for demodulation | All subframes scheduled for demodulation except subframe #5 |  |  |
| MSC and TBS options | Refer to [5] | Refer to [5] | Refer to [5] |
| HARQ | 8 HARQ processes and max 4 transmissions |  |  |
| Feedback periodicity for target signal | Feedback periodicity: 5 msec Feedback delay: 8 msec |  |  |
| PMI granularity and rank of interfering signals (% of rank-1 and % of rank-2) | Randomly changing per sub-band from subframe to subframe as baseline. Randomly changing per sub-band per 10 msec periodicity by interested companies Frequency granularity is 6 PRBs |  |  |
|  | 80% rank-1,20% rank-2 | 80% rank-1, 20% rank-2 | 70% rank-1, 30% rank-2 |
| PMI for target signal | N/A | Follow wideband PMI | Follow wideband PMI |
| Channel and interference estimation at UE | Practical and realizable channel and interference covariance estimates with no a-priori knowledge of the channel state information |  |  |
| Physical channels transmitted in serving cell | PSS/SSS/PBCH |  |  |
| PCFICH | CFI = 2 |  |  |
| PCFICH/PDCCH detection | Not considered |  |  |
| Physical channels transmitted in interfering cells | PDCCH PDSCH: 16QAM modulation is agreed to be used in interfering cells PSS/SSS/PBCH |  |  |
| Cyclic prefix | Normal |  |  |
| Simulation length | 20000 sub-frames at minimum |  |  |

In addition to the configuration described in Table 1, the following assumptions were used for the link level performance simulations presented in this paper:

- Network configuration: Synchronous

# UE Receivers

The following two UE receivers can be defined for evaluation:

- Release 8 Baseline MMSE Receiver:  For Release 8 baseline receiver, the MMSE receiver can be expressed as follow:

WMMSE=H*R-1,

where

R=HH*+σI+N2I,

where σI+N2 denotes the interference plus noise power.

- RAN4 Enhanced MMSE-IRC Receiver:

MMSE-IRC has been the baseline assumption for the enhanced performance requirement for LTE UE receiver study item. The MMSE-IRC receiver for the above channel model can be written as

WMMSE-IRC=H*R-1,

where

R=Eyy*,

denotes the spatial correlation matrix of the total received signal.

# Simulation Results for Test 1

The TM2/TM3 results for the different values of G are given in Table 2.

Table 2: Link level throughput (Mbps) in TM2 scenario

| G (dB) | MCS 6 | MCS 7 |
| --- | --- | --- |
| -8.0 | 1427000 | 1219000 |
| -7.0 | 1763000 | 1728000 |
| -6.0 | 2100000 | 2111000 |
| -5.0 | 2414000 | 2486000 |
| -4.0 | 2797000 | 2809000 |
| -3.0 | 3360000 | 3149000 |
| -2.5 | 3680000 | 3410500 |
| -2.0 | 4000000 | 3672000 |
| -1.0 | 4426000 | 4424000 |
| 0.0 | 4595000 | 5088000 |
| 1.0 | 4637000 | 5436000 |
| 2.0 | 4644000 | 5550000 |
| 3.0 | 4644000 | 5577000 |
| 4.0 | 4644000 | 5580000 |
| 5.0 | 4644000 | 5580000 |
| 6.0 | 4644000 | 5580000 |

# Simulation Results for Test 2

TBD.

# Conclusion

This contribution shows the link simulation results for Scenario 1 and 2 for testing MMSE-IRC receiver.

# References

- R4-116304, “Evaluation methodologies and Simulation assumptions for Enhanced performance requirements for LTE UE SI”, NTT DOCOMO.

- TR 36.829 v1.0.0, “Enhanced performance requirement for LTE UE Study Item”

- RP-120382, “New WI: Improved minimum performance requirements for E-UTRA: interference rejection”

- R4-123638, “Framework document for advanced receivers work item”, Renesas Mobile Europe Ltd.

- R4-123639, “Simulation assumptions for enhanced receiver”, Renesas Mobile Europe Ltd.