3GPP TSG-RAN WG4 (Radio) Meeting #73-AH-UE-RF	R4-73AH-0014

Oulu, Finland, 13 – 16 January 2015

Agenda Item:	4.1.1

Source: 	Alcatel-Lucent, Sprint

Title: 	TP for Rel-13 2DL TR 36.8xx: TIB and RIB values of LTE-A Inter-band Carrier Aggregation of Band 25A and Band 26A (1UL)

Document for:	Approval

# 1.	Introduction

The WI proposal to support the LTE Advanced Carrier Aggregation (CA) of Band Combination (25 + 26) was approved in RAN#66 [1]. In consequence of this WI approval, RAN4 should define the relevant requirements of the maximum output power relaxation and the reference sensitivity relaxation to support inter-band CA of Band 25 and Band 26. In this contribution, both ∆TIB and ∆RIB values are proposed and the relaxation requirements are based on the RAN4 approved UE RF requirements for the inter-band Carrier Aggregation scenario of Class A1 in Release 11.

# 2.	Discussion

Based on the defined inter-band carrier aggregation scenario of Class A1, the Inter-band Carrier Aggregation of Band 25 and Band 26 is a low-high pair spectrum without harmonics relationship. The relaxations for Class A1, i.e. ΔTIB,c = 0.3dB and ΔRIB = 0dB, should be hired in support of inter-band Carrier Aggregation Band 25 and Band 26.

# 3.	Conclusion

Based on the UE RF requirements for the inter-band Carrier Aggregation scenario of Class A1, we propose the following requirements for the inter-band Carrier Aggregation Band 25 and Band 26,

Proposal: ΔTIB,c = 0.3dB and ΔRIB = 0dB should be applied to both bands for CA of Band 25 + Band 26

This proposal should be applied to the TR36.8xx [2] and cover the requirements of Inter-band Carrier Aggregation Band 25 and Band 26.

# References

[1]	RP-142020, “New WID: LTE Advanced inter-band Carrier Aggregation of Band 25 and Band 26”, Sprint.

[2]	R4-147551, “Rel-13 2DL TR 36.8xx: LTE-A Inter-band Carrier Aggregation v0.2.0”, Ericsson.

# Text Proposal

<Start of change>

# 1	Scope

The present document is a technical report for Inter-band Carrier Aggregation for 2DL/1UL under Rel-13 time frame. The purpose is to gather the relevant background information and studies in order to address Inter-band Carrier Aggregation requirements.

This TR covers relevant background information and studies in order to address Inter-band Carrier Aggregation (2DL/1UL) requirements for the Rel-13 band combinations in table 1-1.

Table 1-1: Release 13 inter-band carrier aggregation combinations (2DL/1UL)

| WI code | WI title | Class |
| --- | --- | --- |
| LTE_CA_B2_B28 | LTE Advanced inter-band Carrier Aggregation of Band 2 and Band 28 | A1 |
| LTE_CA_B3_B31 | LTE Advanced inter-band Carrier Aggregation of Band 3 and Band 31 | A2 |
| LTE_CA_B4_B28 | LTE Advanced inter-band Carrier Aggregation of Band 4 and Band 28 | A2 |
| LTE_CA_B20_B31 | LTE Advanced inter-band Carrier Aggregation of Band 20 and Band 31 | A3 |
| LTE_CA_B25_B26 | LTE Advanced inter-band Carrier Aggregation of Band 25 and Band 26 | A1 |

The scope of the report also covers the 2 DL fallback modes for the 3 DL Carrier Aggregation combinations with single UL configuration [2]. These 3 DL/1UL combinations (with 2 DL fallback modes) are listed in table 1-2. In addition, the report also covers the LTE TDD-FDD joint operation including Carrier Aggregation combinations [3], captured in Table 1-3.

Table 1-2: Release 13 inter-band carrier aggregation combinations (3 DL/1UL)

| WI code | WI title |
| --- | --- |
| LTE_CA_B2_B5_B29 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 2, Band 5 and Band 29 |
| LTE_CA_B38_B40_B40 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 38, Band 40 and Band 40 |

Table 1-3: Release 13 inter-band carrier aggregation combinations (LTE TDD-FDD joint operation)

| WI code | WI title | Class |
| --- | --- | --- |
| LTE_CA_B1_B40 | LTE Advanced inter-band Carrier Aggregation of Band 1 and Band 40 | - |
| LTE_CA_B3_B40 | LTE Advanced inter-band Carrier Aggregation of Band 3 and Band 40 | A3 |
| LTE_CA_B25_B41_B41 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 25, Band 41 and Band 41 | - |

This TR contains a general part and band specific combination part. The actual requirements are added to the corresponding technical specifications.

<New section>

### 6.1.X	LTE Advanced Carrier Aggregation of Band 25 and Band 26 (1 UL)

CA_25-26 is designed to operate in the operating bands defined in table 6.1.X-1.

Table 6.1.X-1: Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_25-26 | 25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | 26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz |  |

#### 6.1.X.1	List of specific combination issues

##### 6.1.X.1.1	Channel bandwidths per operating band for CA

Table 6.1.X.1.1-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| CA operating / channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_25A-26A | 25 | - | Yes | Yes | Yes | Yes | Yes | 35 | 0 |
|  | 26 | Yes | Yes | Yes | Yes | Yes | - |  |  |

##### 6.1.X.1.2	Co-existence studies for CA_25-26

The UL frequencies and their harmonics for Bands 25 and 26 are listed in Table 6.1.X.1.2-1 below. It can be seen that the UL harmonics cannot interfere with the DL of Band 25 or 26. Hence CA_25A-26A belongs to inter-band carrier aggregation scenario of Class A1.

Table 6.1.X.1.2-1: Band 25 and Band 26 DL harmonics

| BS DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 814 | 849 | 1850 | 1915 |
| 2nd harmonics frequency limits (MHz) | 1628 | 1698 | 3700 | 3830 |
| 3rd harmonics frequency limits (MHz) | 2442 | 2547 | 5550 | 5745 |

##### 6.1.X.1.3		ΔTIB,c and ΔRIB (1 UL)

For the UE which supports CA_25A-26A the ΔTIB,c is defined for applicable bands in table 6.1.X.1.3-1.

Table 6.1.X.1.3-1: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_25A-26A | 25 | 0.3 |
|  | 26 | 0.3 |

For the UE which supports CA_25A-26A the ΔRIB is defined for applicable bands in table 6.1.X.1.3-2.

Table 6.1.X.1.3-2: ΔRIB

| Inter-band CA Configuration | E-UTRA Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_25A-26A | 25 | 0 |
|  | 26 | 0 |

<End of change>