3GPP TSG RAN WG4 Meeting #73 UE RF AH	                   R4-73AH-0042

Oulu, Finland, 13th – 16th Jan., 2015

- Agenda Item:	4.6

- Source:	LG Electronics

- Title:	Work plan for 3DLs/2ULs inter-band CA

Document for:	Approval

- Introduction

In the last RAN Plenary #66, 3DLs/2ULs inter-band CA band combination related way forward was proposed [1]. Related new WID can be provided in Feb. RAN4 #74. In this contribution, we provide an estimated work plan for a new 3DLs/2ULs WI. RAN4 can use the similar class based approach used in Rel-12 for dual uplink inter-band CA.

- Open issues for 3DLs/2ULs inter-band CA

Dual uplink CA is completed in Rel-12 time frame. And spectrum issues are defined as release independent issues, so it is necessary to determine which release can support the 3DLs/2ULs CA UE. And also RAN4 need to decide whether or not to reuse the CA UE RF architecture for dual uplink CA according to the CA class.

The Table 1 shows possible discussion points for 3DLs/2ULs CA WI.

Table 1: Estimated discussion points in UE aspect for 3DLs/2ULs CA WI

|  | Conventional 2ULs CA WI | Conventional 3DLs CA WI | New 2ULs/3DLs WI |
| --- | --- | --- | --- |
| Release independent issues | From REl-11 | 1. From rel-10 2. From rel-11 for over 40MHz aggregated CH BW | Can be support from Rel-11 |
| Additional RF component | Harmonics filter, Quadplexer | Triplexer, Hexaplexer | Harmonic filter, Triplexer, Hexaplexer |
| Reference architecture | TR36.860 [3] | R4-69AH-0015[4] | Need further discussion |
| Cross region issue for coexistence | Choose subset of protected frequency ranges/bands where both operating bands deployed in country [3] | Not necessary | Need further discussion for new band combinations. Can reuse the same approach used in 2ULs CA |
| Harmonic/IMD | Consider up to 5th order IMD products | Not necessary | Can consider up to 5th order IMD products |
| GNSS/ISM band protection | Only consider GNSS | Not necessary | Can reuse the same principle in 2ULs CA |
| Fallback mode | 2ULs/2DLs 1UL/2DLs CA fallback | 3DLs/1UL 2DLs/1UL CA fallback | Need further discussion 3DLs/2ULs CA 3DLs/1UL or 2ULs/2DLs CA fallback |
| General TX/RX requirements | TS36.101 and TS36.307 [5] | TS36.101 and TS36.307 [5] | Need further discussion |

- Time schedule for 3DLs/2ULs CA

The new work item can be studied in Rel-13 time frame and should be supported as release independent manner. Interested operator would like to launch a commercial service on 2016. So the new WI should be completed by November 2015 which is RAN4#77 and should be closed in RAN Plenary #70 meeting.

In Figure1, we showed the expected time schedule.

Figure1. Work plan for 3DLs/2ULs inter-band CA frame-work

- RAN4 # 73 UE RF AH

- Discussion

- Work plan

- 2ULs/3DLs pairing

- Agreement

- 2ULs/3DLs pairing

- RAN4 # 74

- Discussion

- Work plan

- Potential REL-13 TR Skeleton

- Work item description

- Consensus

- Work plan

- Potential REL-13 TR Skeleton

- RAN4 # 74bis

- Discussion

- Reference UE RF architecture

- Cross region coexistence analysis

- Harmonic/IMD analysis for  each CA band combination

- Agreement

- Work plan

- Potential REL-13 TR Skeleton

- Reference UE RF architecture

- Cross region coexistence requirements

- RAN4 # 75& 76

- Discussion

- General Tx/Rx requirements

- Additional ILs for each CA band combination

- GNSS/ISM band protection

- Harmonic/IMD analysis for  each CA band combination

- Fallback CA mode

- Agreement

- General Tx requirements

- Additional ILs for each CA band combination

- GNSS/ISM band protection method

- How to treat Harmonic/IMD problems for each CA band combination

- RAN4 # 76BIS& 77

- Discussion

- General Rx requirements

- Fallback CA mode

- Agreement

- General Rx requirements

- Fallback CA mode

- Agreed all related CRs

- Conclusions

In this contribution, we provided work plan for 3DLs/2ULs new WI. Our proposal as follow

Proposal 1: For the 3DLs/2ULs inter-band CA WI, RAN4 can follow similar procedure used in dual uplink CA with class-based approach

Proposal 2: Work plan is proposed in section 3 to complete the WI in December 2015.

- References

- RP-141885, “Way Forward for 3DL/2UL CA”, LG Uplus

- R4-148107, “Future CA WI naming, WI acronym, TR handling and WID content”, Ericsson, Nokia Corporation, ALU, Sprint

- TR36.860 v1.0.0

- R4-69AH-0015, “Open issues related to TX and RX relaxations in 3DL inter-band CA”, Broadcom

- TS36.307 v12.6.0