3GPP TSG-RAN WG4 (Radio) Meeting #73-AH-UE-RF	R4-73AH-0110

Oulu, Finland, 13 – 16 January 2015

Agenda Item:	4.5.3

Source: 	Alcatel-Lucent, Sprint

Title: 	TP for Rel-13 3DL TR 36.8xx: LTE Advanced Carrier Aggregation of Band Combination (25 + 41 + 41)

Document for:	Approval

# 1.	Introduction

The WI to support the LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 25, Band 41 and Band 41 was completed in Rel-12 [1-3]. But the related text proposals [4-5] were approved into the Rel-13 TR [6]. In this contribution, we provide a text proposal to remove the corresponding UE materials into the Rel-13 TR to avoid unnecessary confusion. Note that the removed materials are put into the Rel-12 TR in [7].

# 2.	Text Proposal

<Start of change>

## 6.6	LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 25, Band 41 and Band 41

### 6.6.1	Co-existence studies for 1 UL/2 DL

The 2nd and 3rd order harmonics and IMD products caused in the BS by transmitting of contiguous Band 41 DL carriers can be calculated as shown in Table 6.6.1-1 below:

Table 6.6.1-1: Band 41 DL harmonics and IMD products

| BS DL carriers | f-low | f-high |
| --- | --- | --- |
| DL frequency (MHz) | 2496 | 2690 |
| 2nd harmonics frequency limits (MHz) | 4992 | 5380 |
| 3rd harmonics frequency limits (MHz) | 7488 | 8070 |
| 2nd order IMD products | (f-low – f-high) | (f-high – f-low) |
| IMD frequency limits (MHz) | -194 | 194 |
| 3rd order IMD products | (2*f-low – f-high) | (2*f-high – f-low) |
| IMD frequency limits (MHz) | 2302 | 2884 |
| 3rd order IMD products (with maximum channel bandwidth) | (f-low – BWmax) | (f-high + BWmax) |
| IMD frequency limits (MHz) | 2456 | 2730 |

It can be seen from Table 6.6.1-1 that the 3rd order IMD products of BS transmitting contiguous carriers in Band 41 may fall into the BS receive band of Bands 7, 30, 38, 40 and 41. Note that the calculation in Table 6.6.1-1 (except the last row) assumes the BS is transmitting with the whole 194 MHz DL frequency of Band 41. If the BS is only transmitting an up to 40 MHz DL in Band 41 as stated in the WIDS, then the 3rd IMD products will not fall into the BS receive band of the Band 30 or 40 as shown in the last row in Table 6.6.1-1.

Note that Bands 7 and 38 are not intended for use in the same geographical area as Band 41. Moreover, the Band 41 BS deployed in the same geographical area are assumed to be synchronized so that the Band 41 BS receivers will not be interfered by the Band 41 BS transmission. Therefore, the 3rd IMD products of BS transmitting two contiguous carriers in Band 41will not fall into the BS receive band of any operating band currently defined in the 3GPP standards for use in the same geographical area.

### 6.6.2	Co-existence studies for 1 UL/3 DL

The 3rd order IMD products caused in the BS by transmitting of Band 25, Band 41 and Band 41 DL carriers (i.e. 3DL CA with one component carrier in each band) can be calculated as shown in Table 6.6.2-1 below:

Table 6.6.2-1: Band 25, Band 41 and Band 41 DL 3rd order IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high | f3-low | f3-high |
| --- | --- | --- | --- | --- | --- | --- |
| DL frequency (MHz) | 1930 | 1995 | 2496 | 2690 | 2496 | 2690 |
| 3rd order IMD products | (f1-low + f2-low – f3-high) |  |  | (f1-high + f2-high – f3-low) |  |  |
| IMD frequency limits (MHz) | 1736 |  |  | 2189 |  |  |
| 3rd order IMD products | (f1-low + f3-low – f2-high) |  |  | (f1-high + f3-high – f2-low) |  |  |
| IMD frequency limits (MHz) | 1736 |  |  | 2189 |  |  |
| 3rd order IMD products | (f2-low + f3-low – f1-high) |  |  | (f2-high + f3-high – f1-low) |  |  |
| IMD frequency limits (MHz) | 2997 |  |  | 3450 |  |  |
| 3rd order IMD products | (f1-low + f2-low + f3-low) |  |  | (f1-high + f2-high + f3-high) |  |  |
| IMD frequency limits (MHz) | 6922 |  |  | 7375 |  |  |

It can be seen from Table 6.6.2-1 that in addition to the impacts of harmonics and IMD products from the constituent 2DL CA configurations, the 3rd order IMD products supporting 3 DL CA of Band (25 + 41 + 41) may fall into the BS receive band of Bands 1, 2, 3, 4, 9, 10, 22, 23, 25, 33, 34, 35, 36, 37, 39 and 42. However, Bands 1, 3, 9, 10, 33, 34 and 39 are not intended for use in the same geographical area as Bands 25 and 41. Moreover, co-location of Band 25 transmitter and Band 35, 36 or 37 transceiver implies FDD/TDD co-location on adjacent frequencies which requires the use of certain site-engineering solutions to avoid mutual interference. Therefore, the focus here is on the 3rd order IMD products falling into Bands 2, 4, 22, 23, 25 and 42. Note that the 3rd order IMD products will not fall into the BS own (up to 20 MHz) receive block within Band 25.

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the IMD interference generated within the Band 4, 22 or 42 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Band (25 + 41 + 41) BS transmitter does not share the same antenna with Band 4, 22 or 42 BS receiver. But there is not a large frequency gap between Band 25 DL and Bands 2, 23 and 25 UL, and hence Bands 2, 23 and 25 BS receiver desensitization may still be an issue.

Therefore, it is recommended that Band (25 + 41 + 41) BS transmitter should not share the same antenna with Band 2, 23 or 25 BS receiver to prevent BS receiver desensitization, or with Band 4, 22 or 42 BS receiver unless the antenna path meets very stringent 3rd order PIM specification so that the PIM will not cause Band 4, 22 or 42 BS receiver desensitization. Note that antenna sharing may be allowed as the state-of-the-art continues to evolve in the future.

<End of change>

# References

[1]	R4-148124, “Introduction of additional band combinations for 3DL inter-band CA”, Ericsson, CATT, CMCC.

[2]	R4-148121, “Introduction of 3DL CA combinations”, Nokia Networks.

[3]	R4-148122, “Introduction of 3DL CA combinations”, Nokia Networks.

[4]	R4-146170, “TP for Rel-13 2DL TR 36.8xx: Coexistence Studies of Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (25 + 41)”, Alcatel-Lucent, Sprint.

[5]	R4-146840, “TP for Rel-13 2DL TR 36.8xx: TIB and RIB values of LTE-A Inter-band Carrier Aggregation of Band 25A and Band 41A (1UL)”, Alcatel-Lucent, Sprint.

[6]	R4-73AH-0074, “Rel-13 3DL CA TR 36.8xx version 0.2.0”, ZTE.

[7]	R4-73AH-0019, “Addition of 3 Band Carrier Aggregation (3DL/1UL) of Band 25, Band 41 and Band 41”, Alcatel-Lucent, Sprint.