3GPP TSG-WG4 Meeting #79-AH	R4-79AH-0035

# Hong Kong, China, 28th – 30th, June 2016

Agenda item: 	2.4.1

Source: 	Qualcomm Incorporated

Title: 	NPBCH Demodulation Performance

Document for:	Discussion

# 1		Introduction

For NPBCH, RAN4 agreed with the following simulation assumption for alignment, and related main configurations are listed as in Table 1 in [1]. RMC table is shown in Table 2 in [1].

Companies are encouraged to provide the simulation result for SNR satisfying 1% Pm-bch.

Table: RMC table for NPBCH

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | R.NB1.1 | R.NB1.2 |
| Number of transmitter antennas |  | 1 | 2 |
| Channel bandwidth | kHz | 180 | 180 |
| Payload (without CRC) | Bits | 34 | 34 |
| CRC | Bits | 16 | 16 |

# 2	NPBCH Simulation Results

1% BLER is achieved at SNR=--16.8dB for the 1Tx antenna case.

# 3	Conclusion

This paper has presented simulation results for NPBCH and observed the following performance:

Table 1: Minimum performance for NPBCH

| Simulation number | Bandwidth | Reference Channel | Propagation Condition | Antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch (%) | SNR (dB) |
| 1 | 180 KHz | R.NB1.1 | ETU5 | 1 x 1 | 1 | -16.8 |

# 4	References

- R4-164479, “Way Forward and simulation assumptions for NB-IoT UE demodulation”, Huawei, HiSilicon, Ericsson, Qualcomm, [NTT DOCOMO], [Intel]