3GPP TSG-RAN WG4 #79AH	R4-79AH-0050

Hong Kong, China, 28 - 30 June, 2016

Source: 	Ericsson

Title:	General test setup and requirements applicability for LAA demodulation

Agenda Item:	3.2.2

Document for:	Discussion

# Introduction

In this paper, the general setup for LAA demodulation is further discussed, and our view on some open issues are also shared.

# System setup for LAA demodulation

## Bandwidth combination

For the bandwidth combination, in RF, the following combinations have been approved for LAA in RAN4#77 meeting:

- Some CA combinations including B46 up to 2CC (e.g. CA_1A-46A) [1]

- CA_1A-46C [2]

- CA_1A-46D [3]

- CA_1A-46E [4]

- CA_3A-46C [5]

- CA_3A-46D [6]

- CA_3A-46E [7]

Based on this approved combination, we can see that up to 4 LAA Scells can be configured. In principle, the performance requirements shall cover up to 4 LAA Scells. However, due to time reason, in RF part, only one primary cell and one LAA Scell can be configured in Rel-13. In order to resolve this issue, we can reuse the traditional way for the band which is release independent. For release independent requirements, performance requirements for multiple LAA Scells can be specified in Rel-14 but are defined as release-independent performance requirements in 36.307 [11]. It means UEs that conform to Release 13 and support multiple Scells shall support the requirements defined in Rel-14 for multiple LAA Scells. Hence, we prefer to have the following proposal regarding the bandwidth configuration:

Proposal 1:

- The performance requirements are defined based on one primary cell and one LAA Scells in Rel-13, and

- The performance requirements for multiple LAA Scells (at least up to 4 LAA Scells) will be defined in Rel-14, and

- UEs that conform to Release 13 and support multiple LAA Scells shall support the corresponding requirements defined in Rel-14, which will be captured in 36.307.

## CRS structure configuration

According to the agreements in [14], we have the following agreements:

- MBSFN subframe configuration

  - Option1: MBSFN is configured in subframes 4 and 9 for DMRS-based transmission mode

  - Option 2: Not configure MBSFN

Since MBSFN configuration is more challenge for UE implementation, in Rel-13, we are fine to start from non-MBSFN configuration for both CRS-based transmission scheme and DMRS-based transmission scheme.

## PDCCH and ePDCCH test

In [12], technical analyses are provided for the necessity of explicit PDCCH and ePDCCH test. The observations are recapped here:

Observation 1:  AGC/FTL/TTL problem or other receiver problem for (e)PDCCH can be explicitly reflected in explicit (e)PDCCH performance test and it may not be  fully reflected in PDSCH performance test.

Observation 2:  The MCS of PDSCH and the number of CCEs, the number of OFDM symbols for control channel shall be tuned carefully for the implicit (e) PDCCH performance test if implicit (e)PDCCH performance verification  is used. Otherwise, the bad implementation can easily pass the test.

Observation 3: (e)PDCCH performance shall be explicitly specified based on general RAN4 principle

Observation 4:  The test number of LAA is reasonable when the explicit test is considered.

Based on these observations, we propose to have:

Proposal 2: Explicitly specify the PDCCH and EPDCCH performance requirements

The majority companies share the same view for this issue [13].

## Transmission mode and antenna configuration

For the transmission mode, we have two options:

- Option 1: TM4 4x2, TM9 2x2

- Option 2: TM3 2x2, TM4 4x2, TM9 2x2

For both options, TM4 4x2 and TM9 2x2 are included. In order to reduce the test case and cover TM3 2x2 case, we can combine TM3 2x2 case with PDCCH test.

Propose 3: TM3 2x2 PDSCH test can be combined with explicitly PDCCH test in order to reduce the test case number

## Frequency offset and timing error, and synchronization

For frequency offset and timing error, we have two options:

- Option 1: [750]Hz,  [30.26]μs

- Option 2: 200 Hz, 3 us

In legacy CA performance requirements, the time offset between PCell and any SCell is 30usec, as shown in Table 8.2.1.1.1-6 of 36.101 and which is captured as Table 1. As a start point, we can reuse the time error in LAA test.

Table 1: Minimum performance (FRC) based on single carrier performance for CA with 3DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 3x20MHz | As specified in Table 8.2.1.1.1-5 per CC | ≥5 |
| 2 | 20MHz+20MHz+15MHz | As specified in Table 8.2.1.1.1-5 per CC | ≥5 |
| 3 | 20MHz+20MHz+10MHz | As specified in Table 8.2.1.1.1-5 per CC | ≥5 |
| 4 | 20MHz+15MHz+15MHz | As specified in Table 8.2.1.1.1-5 per CC | ≥5 |
| 5 | 20MHz+15MHz+10MHz | As specified in Table 8.2.1.1.1-5 per CC | ≥5 |
| 6 | 20MHz+10MHz+10MHz | As specified in Table 8.2.1.1.1-5 per CC | ≥5 |
| 7 | 15MHz+15MHz+10MHz | As specified in Table 8.2.1.1.1-5 per CC | ≥5 |
| 8 | 20MHz+10MHz+5MHz | As specified in Table 8.2.1.1.1-5 per CC | ≥5 |
| 9 | 20MHz+15MHz+5MHz | As specified in Table 8.2.1.1.1-5 per CC | ≥5 |
| NOTE 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3 NOTE 2: 30usec timing difference between PCell and any SCell is applied in inter-band CA case, where PCell can be assigned on any CC. |  |  |  |

For the frequency error, according to RF requirements, the frequency error for each CC is +/-0.1ppm for local area BS, hence, we can use +/-0.2 ppm as the frequency offset between different carriers. Hence, it is reasonable to use Option 1 for the frequency offset and time error.

Proposal 4: The frequency offset between carriers can be 750 Hz and the time offset between PCcell and LAA Scells is 30.26 usec.

## Handling for UE capability for partial subframe

For the partial subframe, we agreed to have the following test scenarios:

- Test Scenario 1: full subframe only;

- Test Scenario 2:  full subframe + ending partial subframe;

- Test Scenario 3: Initial partial subframe + full subframe;

- Test Scenario 4: Initial partial subframe + full subframe + ending partial subframe.

RAN4 will define different tests for different capabilities. UEs will be tested based on its capability with the understanding that one test scenario is applied for a specific UE.It is also subject to RAN1 decision whether ending partial subframe will be mandatory or optional.

From RAN4 point of view, if the normalized throughput is used, the target SNR scenario 1, 2, 3 and 4 would be very close. Hence, we can run simulation based on scenario 4 for the first round alignment. The specification can be updated based on latest RAN agreements on whether ending partial subframe is mandatory or not.

Proposal 5: Test scenario 4 will be used for simulation results alignment and the specification will be updated based on latest RAN agreements on whether ending partial subframe is mandatory or not.

# Conclusion

In this paper, we give our view on general test setup and also discuss the requirements applicability for LAA demodulation. We have the following proposals:

Proposal 1:

- The performance requirements are defined based on one primary cell and one LAA Scells in Rel-13, and

- The performance requirements for multiple LAA Scells (at least up to 4 LAA Scells) will be defined in Rel-14, and

- UEs that conform to Release 13 and support multiple LAA Scells shall support the corresponding requirements defined in Rel-14, which will be captured in 36.307.

Proposal 2: Explicitly specify the PDCCH and EPDCCH performance requirements

Propose 3: TM3 2x2 PDSCH test can be combined with explicitly PDCCH test in order to reduce the test case number

Proposal 4: The frequency offset between carriers can be 750 Hz and the time offset between PCcell and LAA Scells is 30.26 usec.

Proposal 5: Test scenario 4 will be used for simulation results alignment and the specification will be updated based on latest RAN agreements on whether ending partial subframe is mandatory or not.

# Reference

- R4-158443, Introduction of RF requirements for LAA operation, Ericsson, Nov. 2015

- R4-158270, TP for TR 36.853-13: LTE Advanced Carrier Aggregation of Band Combination (1A+LAA_C), NTT DOCOMO, INC. Nov, 2015

- R4-158271, TP for TR 36.854-13: LTE Advanced Carrier Aggregation of Band Combination (1A+LAA_D), NTT DOCOMO, INC. Nov, 2015

- R4-158272, TP for TR 36.857-13: LTE Advanced Carrier Aggregation of Band Combination (1A+LAA_E), NTT DOCOMO, INC. Nov, 2015

- R4-158412, TP for TR 36.853-13: LTE Advanced Carrier Aggregation of Band Combination (3A+LAA_C), NTT DOCOMO, INC. Nov, 2015

- R4-158267, TP for TR 36.854-13: LTE Advanced Carrier Aggregation of Band Combination (3A+LAA_D), NTT DOCOMO, INC. Nov, 2015

- R4-158268, TP for TR 36.857-13: LTE Advanced Carrier Aggregation of Band Combination (3A+LAA_E), NTT DOCOMO, INC. Nov, 2015

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #83, 15th-22th Nov. 2015.

- R4-161182, Way forward on demodulation performance and CSI requirements for LAA, Huawei, HiSilicon, Feb. 2016.

- R1-161547, RAN1 UE feature list on Rel-13 LTE, RAN1, Feb. 2016.

- 3GPP 36.307, Requirements on User Equipments (UEs) Supporting a release-independent frequency band, v10.17.0, 2016.

- R4-163927, Further discussion on the control channel performance requirements, Ericsson, May 2016

- R4-79AH-0055, Ericsson, LGE, Nokia, Verizon, Vodafone, LG Uplus, ZTE, May 2016.

- R4-164749, Way forward on LAA demodulation performance, Huawei, Ericsson, Intel and Qualcomm, May 2016.