3GPP TSG-RAN WG4 #AH-1807 Meeting                                R4-1808617

Montreal, Canada, 2 to 6 July, 2018

Agenda item:	5.4.2.2

Source: 	ZTE

Title: 	On NR BS PUSCH demodulation performance requirements

Document for:	Discussion

Introduction

During RAN4#87 meeting in May two WFs [1][2] relevant for NR BS PUSCH demodulation performance requirements and simulation assumptions for alignment purpose have been approved. This contribution focuses on the initial simulation results of NR PUSCH for FR1 conducted case. For FR2 case, the need of PTRS is discussed.

Discussions

- Simulation assumptions and initial results for FR1 conducted

Here are the simulation assumptions for FR1 conducted:

| Parameter | Value |  | Remarks |
| --- | --- | --- | --- |
|  | FR1 |  |  |
| Transform precoding | Disabled |  |  |
| Number of Tx | 1 |  | No transparent Tx diversity considered |
| Number of Rx | 2 |  | Number of diversity branches in baseband |
| Number of layers | 1 |  |  |
| DMRS type | Type-1 |  |  |
| Number of DMRS symbols | Case 2: 1 front-loaded |  |  |
| Time domain resource allocation type | Type A |  |  |
| PUSCH symbol length | 14 OFDM symbols |  |  |
| MCS index | QPSK: 4 16QAM: 16 64QAM: 25 |  |  |
| Carrier frequency (GHz) | 4 |  |  |
| Propagation channel | AWGN |  |  |
| Time offset | Not modelled |  |  |
| Freq_offset | Not modelled |  |  |
| Waveform | CP-OFDM |  |  |
| SCS@CBW | 30 kHz@40 MHz (Only used part of the RBs in simulations) |  |  |
| Maximum number of HARQ transmissions | 4 |  |  |

The initial simulation results for FR1 conducted are summarized in Table 1 below.

Table 1: Initial simulation results for FR1 conducted: 1Tx, SCS 30 kHz @ CBW 40 MHz (Only used part of the RBs in simulations)

| Number of TX | Number of RX (Number of diversity branches in baseband) | Cyclic prefix | Propagation conditions | Modulation, MCS | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | AWGN | QPSK, MCS 4 | 70% | -0.9 |
|  |  |  |  | 16QAM, MCS 16 | 70% | 11.0 |
|  |  |  |  | 64QAM, MCS 25 | 70% | 19.0 |

- PTRS in FR2 tests

PTRS is introduced in NR to enable compensation of phase noise, which typically increases as a function of carrier frequency. It should be used in FR2 tests in Rel-15.

Proposal 1: PTRS is used in FR2 tests in Rel-15.

Conclusions

Based on the discussions above, we have the following proposal:

Proposal 1: PTRS is used in FR2 tests in Rel-15.

# References

- R4-1808022 Way forward on General part of NR BS demodulation performance, May 2018.

- R4-1808024 WF on NR PUSCH demodulation requirements in Rel-15, May 2018.