TSG-RAN Working Group 4 (Radio) meeting AH1807  	R4-1808655

Montreal, Canada, 2nd July – 6th July 2018

Source:	Ericsson

Title:	Necessity of PA for OTA Adjacent channel selectivity and in-band blocking measurement for FR2

Agenda item:	5.1.4.3.2.1

Document for:	Discussion

# Introduction

This contribution presents a discussion about the necessity of using PA in OTA in-band blocking, ACS measurement for FR2. This contribution is based on our similar discussion paper on link budget for FR1 [1]. We are glad to encourage feedback from other companies on this topic.

# Discussion

- Indoor Anechoic Chamber

The setup of OTA in-band blocking, ACS measurement in IAC is showed as Figure 2.1 below. This figure is taken from R4-1808368 [2] which is endorsed in the Busan meeting. The setup of these measurements in FR2 is the same as in FR1.

Figure 2.1  In-door Anechoic Chamber measurement system setup for adjacent channel selectivity, in-bands blocking

We can calculate the link budget based on this setup and get the power level at point A. Typically we use 28GHz wide area BS as an example to show the calculation procedure.

NOTE:

- As the wanted signal requirement at RIB is much lower than interfering signal, so here we only discuss about the interferer signal link budget.

- We assume that antenna array of all DUTs take the same deployment showed in Figure 2.2 and take the diagonal length of antenna array as the DUT size in the calculation (It may be discussed further whether the assumed aperture size is large enough, or the calculations might be based on a larger aperture).

- We take 1.2 times of far field distance boundary as test distance and assume the chamber is big enough.

Figure 2.2  64 TRX Antenna elements deployment

The link budget example is listed in Table below.

Table 2.1 Interferer signal Link budget example of OTA general blocking, ACS measurement for IAC

| Link items | Value | Note |
| --- | --- | --- |
| BS type | Wide area |  |
| Frequency: f (MHz) | 28000 |  |
| DUT size: D (m) | 0.07 | Diagonal length of antenna array |
| test distance: d (m) | 1 | d=(2D2λ)*1.2 |
| Signal Generator Power: PSG (dBm) | 11 | In dynamic range |
| Cable1 gain: Gcable1 (dB) | -2.1 | Cable length is 1m |
| 20dB Directional coupler gain: GComb (dB) | -3 | assumption |
| Cable2 gain: GCable2 (dB) | -8 | Cable length is 5m |
| Test Antenna Gain: GTA (dB) | 20 |  |
| Free space gain: GFSG (dB) | -61.43 | GFSG=32.44+20lgd1000+20lgf |
| RIB power level: PRIB (dBm) | -43.53 | PRIB=PSG+Gcable1+GComb+Gcable2+GTA+GFSG |
| OTA ACS core requirement at point A: Pcore (dBm) | -63 ~ -83 | General blocking: EISREFSENS+33dB [3] |
|  | -67.3 ~ -87.3 | ACS: EISREFSENS+[28.7 - SINR-IM]dB [3] Make SINR-IM = 0 |
| Power margin: Gmargin (dB) Gmargin=PRIB- Pcore | 19.47 ~ 39.47 | In-band blocking |
|  | 23.77 ~ 43.77 | ACS |

Based on the margin in this budget table, PA isn’t needed during the measurement.

We can also calculate the similar link budgets of 39GHz measurements. The results are listed in Table 2.2.

Table 2.2  Link budget comparison at different frequency in IAC

| f (MHz) | D (m) | d (m) | PSG (dBm) | Gcable1 (dB) | GComb (dB) | GCable2 (dB) | GTA (dB) | GFSG (dB) | PRIB (dBm) | Pcore (dBm) | Gmargin_min(dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  | IBB | ACS |
| 28000 | 0.07 | 1 | 11 | -2.1 | -3 | -8 | 20 | -61.4 | -43.5 | -55 | 19.5 | 23.8 |
| 39000 | 0.05 | 0.72 | 11 | -2.1 | -3 | -8 | 20 | -61.4 | -43.5 | -55 | 19.5 | 23.8 |

From the analysis above, we can see that the margins between the power levels at RIB and interferer power level requirement at different frequency are more than 19dB.

It might not need any PA in OTA in-band blocking and ACS measurements for FR2 for indoor anechoic chamber.

# 3.  Conclusion

It might not need PA for nowadays products in the OTA in-band blocking and ACS measurement for FR2 for IAC since the margin between the power level at RIB and core requirement is more than 19dB and should be large enough. The assumption on the aperture may be discussed further.

# 4.  Reference

[1]	R4-180xxxx, ” Discussion - necessity of PA for OTA ACS, general blocking and narrow band blocking for FR1”, Ericsson

[2]	R4-1808368, “Draft CR for TR37.843 -Indoor anechoic chamber test method procedure for OTA adjacent channel selectivity, narrow-band blocking, and in-band blocking”, NTT DOCOMO

[3]	3GPP TS 38.104 V15.1.0 (2018-03)