3GPP TSG-RAN WG4 Meeting AH-1807 LTE/NR perf	R4-1808663

Montreal, Canada, 2th – 6th July 2018

Agenda Item:	4.2.1.1.3.1

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Proposals on MU for OTA receiver intermodulation requirements

Document for:	Approval

# 1.	Introduction

The MU and TT for OTA receiver directional requirements were discussed in RAN4#87 and the way forward was agreed [1], with two open issues relating to the MU. Our proposals to conclude on the two open issues relating to the MU are provided in [2].

This contribution provides our proposals on the MU for the OTA receiver intermodulation requirements, according to the agreed way forward in [1] and proposals in [2].

# 2.	Discussion

It was agreed in [1] that the MU for OTA receiver directional requirements should be calculated as:

MU=MUconductedwanted2+MUconductedint2+MUEIS2-MUTestEquipment2-2*MUmatching2+ACLR effect

And we propose in [2] to use the EIS measurement uncertainty from the source ‘Impedance mismatch in the transmitting chain’ recorded in TR 37.842 [3] as the mismatch uncertainty for each of the radiated measurement methods for the frequency range below 4.2 GHz, and PA is not needed for frequency range below 6GHz for high-end signal generators.

There are one wanted signals and two interfering signals (one modulated and one CW) in the for the OTA receiver intermodulation requirements. Hence, the equation above should be modified for the additional CW interfering signal as:

MU=MUconductedwanted2+MUconductedmodint2+MUconductedCWint2+MUEIS2-MUTestEquipment2-3*MUmatching2+ACLR effect

Therefore, the MU for the OTA receiver intermodulation requirements can be calculated as shown in Tables 1 to 4 below for each of the radiated measurement methods, respectively.

Table 1: MU for Indoor Anechoic Chamber

| Test System Uncertainty | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- |
| MUconductedwanted (Wanted signal level error in [4]) | 0.7 | 1.0 |
| MUconductedmodint (Modulated interferer signal level error in [4]) | 0.7 | 1.0 |
| MUconductedCWint (CW interferer signal level error in [4]) | 0.5 | 0.7 |
| MUEIS (Combined standard uncertainty [3]) | 0.62 | 0.64 |
| MUTestEquipment (Uncertainty of the RF signal generator in [3]) | 0.46 | 0.46 |
| MUmatching (Impedance mismatch in the transmitting chain in [3]) | 0.10 | 0.16 |
| ACLR effect (Impact of interferer leakage [4]) | 0.4 | 0.4 |
| Combined standard uncertainty (1σ) | 1.57 | 2.02 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) | 3.08 | 3.95 |

Table 2: MU for Compact Antenna Test Range

| Test System Uncertainty | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- |
| MUconductedwanted (Wanted signal level error in [4]) | 0.7 | 1.0 |
| MUconductedmodint (Modulated interferer signal level error in [4]) | 0.7 | 1.0 |
| MUconductedCWint (CW interferer signal level error in [4]) | 0.5 | 0.7 |
| MUEIS (Combined standard uncertainty [3]) | 0.68 | 0.71 |
| MUTestEquipment (Uncertainty of the RF signal generator in [3]) | 0.46 | 0.46 |
| MUmatching (Impedance mismatch in the transmitting chain in [3]) | 0.09 | 0.23 |
| ACLR effect (Impact of interferer leakage [4]) | 0.4 | 0.4 |
| Combined standard uncertainty (1σ) | 1.61 | 2.02 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) | 3.15 | 3.96 |

Table 3: MU for One Dimensional Compact Range Chamber

| Test System Uncertainty | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- |
| MUconductedwanted (Wanted signal level error in [4]) | 0.7 | 1.0 |
| MUconductedmodint (Modulated interferer signal level error in [4]) | 0.7 | 1.0 |
| MUconductedCWint (CW interferer signal level error in [4]) | 0.5 | 0.7 |
| MUEIS (Combined standard uncertainty [3]) | 0.66 | 0.73 |
| MUTestEquipment (Uncertainty of the RF signal generator in [3]) | 0.46 | 0.46 |
| MUmatching (Impedance mismatch in the transmitting chain in [3]) | 0 | 0.01 |
| ACLR effect (Impact of interferer leakage [4]) | 0.4 | 0.4 |
| Combined standard uncertainty (1σ) | 1.61 | 2.08 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) | 3.15 | 4.07 |

Table 4: MU for Near Field Test Range

| Test System Uncertainty | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- |
| MUconductedwanted (Wanted signal level error in [4]) | 0.7 | 1.0 |
| MUconductedmodint (Modulated interferer signal level error in [4]) | 0.7 | 1.0 |
| MUconductedCWint (CW interferer signal level error in [4]) | 0.5 | 0.7 |
| MUEIS (Combined standard uncertainty [3]) | 0.63 | 0.63 |
| MUTestEquipment (Uncertainty of the RF signal generator in [3]) | 0.46 | 0.46 |
| MUmatching (Impedance mismatch in the transmitting chain in [3]) | 0.20 | 0.20 |
| ACLR effect (Impact of interferer leakage [4]) | 0.4 | 0.4 |
| Combined standard uncertainty (1σ) | 1.54 | 2.00 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) | 3.01 | 3.92 |

Consequently, the test system specific measurement uncertainty values for the OTA receiver intermodulation tests can be obtained in Table 5 below.

# Table 5: Test system specific measurement uncertainty values for the OTA receiver intermodulation tests

|  | Expanded uncertainty ue [dB] |  |
| --- | --- | --- |
|  | f ≦ 3GHz | 3GHz < f ≦ 4.2 GHz |
| Indoor Anechoic Chamber | 3.08 | 3.95 |
| Compact Antenna Test Range | 3.15 | 3.96 |
| One Dimensional Compact Range Chamber | 3.15 | 4.07 |
| Near Field Test Range | 3.01 | 3.92 |
| Common maximum accepted test system uncertainty | 3.2 | 4.0 |

# 3.	Conclusion

This contribution has provided our proposals on the MU for the OTA receiver intermodulation requirements, according to the agreed way forward in [1] and proposals in [2].

Proposal:

Adopt the following table as the MU for the OTA receiver intermodulation tests.

# Table 5: Test system specific measurement uncertainty values for the OTA receiver intermodulation tests

|  | Expanded uncertainty ue [dB] |  |
| --- | --- | --- |
|  | f ≦ 3GHz | 3GHz < f ≦ 4.2 GHz |
| Indoor Anechoic Chamber | 3.08 | 3.95 |
| Compact Antenna Test Range | 3.15 | 3.96 |
| One Dimensional Compact Range Chamber | 3.15 | 4.07 |
| Near Field Test Range | 3.01 | 3.92 |
| Common maximum accepted test system uncertainty | 3.2 | 4.0 |

# References

[1]	R4-1808366, “WF on MU and TT for receiver requirements”, Ericsson.

[2]	R4-1808658, “Proposals on MU and TT for OTA receiver directional requirements”, Nokia, Nokia Shanghai Bell.

[3]		3GPP TR 37.842 v13.2.0, “E-UTRA and UTRA; Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS)”.

[4]	3GPP TR 37.145-1 v15.2.0, “Active Antenna System (AAS) Base Station (BS) conformance testing; Part 1: Conducted conformance testing”.