3GPP TSG-RAN WG4 Meeting #AH07                                                                   R4-1808720

# Montreal, Canada, 2th – 6th July, 2018

Agenda Item:	5.2.8.1

Source:	Intel Corporation

Title:	Discussion about evaluation time for CSI-RS based NR RLM

Document for:	Discussion

# Introduction

In last meeting, there is agreed CR about CSI-RS RLM[1]. The evaluation time for INS and OOS are defined for CSI-RS with density=3.

The values of Mout and Min used in Table 8.1.3.2-1 and Table 8.1.3.2-2 are defined as: - Mout = 20 and Min = 10, if the CSI-RS resource configured for RLM is transmitted with Density =3. Table 8.1.3.2-1: Evaluation period TEvaluate_out and TEvaluate_in for FR1 Configuration; TEvaluate_out (ms); TEvaluate_in (ms). non-DRX; max(200, ceil(Mout×P)×TCSI-RS); max(100, ceil(Min×P) × TCSI-RS). DRX ≤ 320ms; max(200, ceil(1.5×Mout×P)× max(TDRX, TCSI-RS)); max(100, ceil(1.5×Mout×P)× max(TDRX, TCSI-RS)). DRX > 320ms; ceil(Mout×P) × TDRX; ceil(Mout×P) × TDRX. Note: TCSI-RS is the periodicity of CSI-RS resource configured for RLM. TDRX is the DRX cycle length.. Table 8.1.3.2-2: Evaluation period TEvaluate_out and TEvaluate_in for FR2 Configuration; TEvaluate_out (ms); TEvaluate_in (ms). non-DRX; max(200, ceil(Mout×P×N)×TCSI-RS); max(100, ceil(Min×P×N) × TCSI-RS). DRX ≤ 320ms; max(200, ceil(1.5×Mout×P×N)× max(TDRX, TCSI-RS)); max(100, ceil(1.5×Mout×P×N)× max(TDRX, TCSI-RS)). DRX > 320ms; ceil(Mout×P×N) × TDRX; ceil(Mout×P×N) × TDRX. Note: TCSI-RS is the periodicity of CSI-RS resource configured for RLM. TDRX is the DRX cycle length..

However, the evaluation time is not defined for CSI-RS with D=1. In this contribution, we provide our simulation results for CSI-RS with D=1 for 96RB.

# Simulation results

The agreed simulation parameters can be found in [1]. The detailed simulation parameters and assumptions are given in Table 1 and Table 2.

Table 1: General parameters

| Simulation parameters | Comments/values |
| --- | --- |
| Carrier frequency | 4 GHz; 30 GHz |
| DRX | No |
| BS transmit antennas for SS blocks | 1 tx or single layer transmissions |
| UE receive antennas | Case A: 2 rx (uncorrelated with equal gain, no rx beamforming); Case B: 2 rx with rx beamforming Note |
| Number of samples | 10,20,40 |
| Slot length | 14 symbols |
| CP Length | Normal |
| Propagation conditions | For 4 GHz: AWGN, EPA5, ETU30 (for 15 kHz SCS), ETU70 (for 15 kHz SCS); For 30 GHz: AWGN, CDL-C, optional TDL-A/B/C Note (delay spread: 30 ns and 300 ns; UE speed: 3 km/h and 30 km/h) |
| SINR | [-14, -13, ..., 1, 0] dB |
| NOTE: companies are encouraged to state the rx beamforming assumptions |  |

Table 2: CSI-RS configuration parameters

| Parameter | 4 GHz | 30 GHz |
| --- | --- | --- |
| CSI-RS bandwidth | 24 PRBs, 96 PRBs; optional: 48 PRBs, 192 PRBs (not for 60 kHz), 264 PRBs (not for 60 kHz) | 24 PRBs, 96 PRBs; optional: 48 PRBs, 192 PRBs, and 264 PRBs |
| CSI-RS SCS | 15 kHz, 30 kHz, 60 kHz | 60 kHz; 120 kHz |
| Periodicity | 5 ms (optional); 40 ms | 5 ms (optional); 40 ms |
| CSI-RS configuration: <X,D,N,(Y,Z),CDM>, where: X=number of CSI-RS ports, D=density [RE/RB/port], N=number of OFDM symbols in the same slot, Y=adjacent in frequency REs in PRB, Z=adjacent in time REs in PRB | <1,1,1,N/A,noCDM> <1,3,1,N/A,noCDM> | <1,1,1,N/A,noCDM> <1,3,1,N/A,noCDM> |

For Legacy LTE, there are 8 CRS in each RB. The frequency density is every 6 subcarriers. If the bandwidth is 96RB, there are totally 8*96=768 RE. For CSI-RS with D=1, there are 96RE for bandwidth of 96RB. The CSI-RS RE number is 1/8 compared with legacy LTE. The estimation performance will grade a lot especially in relative large delay spread case. Here, we provide our simulation results for ETU30 for SCS=15K Hz and SCS=30K Hz in Fig.1 and Fig.2 respectively.

Fig.1 SNR estimation error for SCS=15K Hz for ETU30

Fig.2 SNR estimation error for SCS=30K Hz for ETU30

For 15K Hz case, suppose the SNR point is -13dB for OOS and 40 samples are used. The estimation error is around 2.5dB. Suppose that the SNR point is -6dB for INS and 20 samples are used. The estimation error is around 2dB. Then the total error is 2.5+2=4.5dB. When 1.5dB RF margin is considered, the total error is 7dB which is very close to the SNR gap of 7dB between -13dB and -6 dB.  Therefore, it can’t achieve good evaluation performance when D=1 is used. For 30KHz case, the estimation error will increase to 7.5dB, where the performance is worse.

Proposal 1: Don’t define RLM evaluation time for CSI-RS with D=1 for FR1.

For FR2, we also provide our simulation results in Fig.3 and Fig.4.

Fig.3 SNR estimation error for SCS=60K Hz for CDL

Fig.3 SNR estimation error for SCS=120K Hz for CDL

With similar analysis for FR1, the SNR estimation error will be 7dB and 10dB for SCS = 60KHz and SCS=120K Hz case, which can’t achieve the estimation accuracy either.

Proposal 2: Don’t define RLM evaluation time for CSI-RS with D=1 for FR2.

# Conclusion

In this contribution some NR link level simulation results for CSI-RS based SINR estimation with density = 1 was provided. The following conclusion can be drawn:

Proposal 1: Don’t define RLM evaluation time for CSI-RS with D=1 for FR1.

Proposal 2: Don’t define RLM evaluation time for CSI-RS with D=1 for FR2.

# References

[1] R4-1808521, “CR on TS38.133 for CSI-RS based RLM requirements”, Huawei.