3GPP TSG-RAN WG4 Meeting AH-1807 LTE/NR perf	R4-1808772

Montreal, Canada, 2th – 6th July, 2018

Agenda item:	5.4.1.5

Source: 		NTT DOCOMO, INC.

Title: 	Views on UE requirements for CSI reporting

Document for:	Approval

1. Introduction

At the previous meetings, following agreements were reached regarding UE requirements on CSI reporting [1]. In this contribution, we present our further views on that.

Agreements (at #86bis on Test scope):

- CQI reporting

- PMI reporting

- RI reporting

- FFS for:

  - CRI reporting

  - L1-RSRP reporting

  - LI reporting

Agreement (#87):

- Overall Test Configuration & Scope

  - First priority focused on:

    - NZP CSI-RS for channel measurement
CSI-IM for interference measurement

    - CSI report types: periodic CSI report and aperiodic CSI report

    - CSI-RS resources types: periodic and aperiodic

- PMI reporting

  - In Rel-15 focused on Type-I single panel codebook

2. Discussion

NR DL supports codebook based closed-loop transmission that is analogous to LTE TM9/10 and doesn’t support other transmissions schemes such as diversity transmission. In this sense, we can generally follow test design for LTE TM9/10. Followings table shows major parameters and candidate values that should be selected to determine CSI performance test. It should be noted this table lists only major CSI-related parameters and we need to specify these test cases for different non-CSI-related and detailed parameters, e.g., CBW, SCS and FDD/TDD configurations, channel models, N1/N2 for codebook. Considering possible combinations of test cases, we need to first focus on limited test cases with basic parameters that is not indicated as square bracket in the table.

Table I. Parameters for CSI performance tests (TBD or non-high priority parameter is indicated as [square bracket])

| Parameters | FR1 | FR2 |
| --- | --- | --- |
| Modulation | QPSK, 16QAM, 64QAM, 256QAM | QPSK, 16QAM, 64QAM |
| Rank | 1, 2, 3, 4 [, etc.] | 1, 2 [, etc.] |
| Precoding | Follow PMI [, etc.] |  |
| CSI-RS transmission | Periodic [, semi-persistent] and aperiodic |  |
| Channel measurement restriction | [On, off] |  |
| Interference measurement restriction | [On, off] |  |
| Codebook | Type I single-panel [, etc.] |  |
| PMI, CQI frequency granularity | Wideband [and subband] |  |
| CQI frequency granularity | Wideband [and subband] |  |
| Number of Tx CSI-RS ports | 2, 4, 8 [, etc.] | 2, 4 [, etc.] |
| Number of Rx ports | 2, 4 | 2, 4 |

Proposal 1: CSI test reuses basic CSI test design from LTE TM 9/10.

Proposal 2: Following parameters should be included in the CSI test.

Table II. Proposed parameters for basic CSI performance tests

| Parameters | FR1 | FR2 |
| --- | --- | --- |
| Modulation | QPSK, 16QAM, 64QAM, 256QAM | QPSK, 16QAM, 64QAM |
| Rank | 1, 2, 3, 4 | 1, 2 |
| Precoding | Follow PMI |  |
| CSI-RS transmission | Periodic and aperiodic |  |
| PMI frequency granularity | Wideband |  |
| CQI frequency granularity | Wideband |  |
| Number of Tx CSI-RS ports | 2, 4, 8 | 2, 4 |
| Number of Rx ports | 2, 4 | 2, 4 |

3. Conclusion

In this contribution, we presented our further views on UE requirements for CSI reporting.

Proposal 1: CSI test reuses basic CSI test design from LTE TM 9/10.

Proposal 2: Following parameters should be included in the CSI test.

Table. Proposed parameters for basic CSI performance tests

| Parameters | FR1 | FR2 |
| --- | --- | --- |
| Modulation | QPSK, 16QAM, 64QAM, 256QAM | QPSK, 16QAM, 64QAM |
| Rank | 1, 2, 3, 4 | 1, 2 |
| Precoding | Follow PMI |  |
| CSI-RS transmission | Periodic and aperiodic |  |
| PMI frequency granularity | Wideband |  |
| CQI frequency granularity | Wideband |  |
| Number of Tx CSI-RS ports | 2, 4, 8 | 2, 4 |
| Number of Rx ports | 2, 4 | 2, 4 |

References

[1] R4-1808027, Ericsson, “WF on simulation assumptions for NR UE performance tests,” RAN4 #87.