3GPP TSG-RAN WG4 Meeting AH1807	R4-1808809

Montreal, Canada, 2 – 6 July 2018

Agenda Item:	4.1.2.1.2

Source:	Ericsson

Title:	Simulation results of SPDCCH for sTTI UE demodulation requirements

Document for:	Discussion

# 1	Introduction

RAN4#87 agreed with the way forward on UE demodulation requirements for sTTI [1].

- Consider EVA30 for CRS-based sPDCCH and sPDSCH performance requirements. - Consider dual layer for DMRS-based sPDSCH performance requirements. - Companies are encouraged to provide the simulation results of sPDCCH and Slot/Subslot-PDSCH based on the agreement - Simulation assumption: R4-1805493

In this contribution we provide the ideal simulation results for alignment.

# 2	Simulation results

## 2.1	CRS-based results

Figure 1 is the simulation results for DMRS-based SPDCCH and Table 1 is the required SNR for 1% BLER.

Figure 1	CRS-based SPDCCH results.

Table 1	SNR [dB] for 1% BLER (CRS-based SPDCCH).

|  | SNR |
| --- | --- |
| Slot-based SPDCCH | 0.8 |
| Subslot-based SPDCCH | 0.1 |

## 2.2	DMRS-based results

Figure 2 is the simulation results for DMRS-based SPDCCH and Table 2 is the required SNR for 1% BLER.

Figure 2	DMRS-based SPDCCH.

Table 2	SNR [dB] for 1% BLER (DMRS-based SPDCCH).

|  | SNR |
| --- | --- |
| Slot-based SPDCCH | -0.4 |
| Subslot-based SPDCCH | -0.9 |

# 3	Conclusion

Proposal: We propose to consider our simulation results for SPDCCH demodulation requirements.

# References

- R4-1808475, “Way forward on sTTI UE demodulation and CSI requirements”, Qualcomm, Ericsson, Huawei.