3GPP TSG-RAN WG4 Meeting #1807	R4-1809002

## Montreal, Canada, 2 - 6 July, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.843 | CR | CRNum | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TR 37.843: Corrections on co-location conformance requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  |  | Date: |  | 2018-06-21 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | An error exist in antenna gain assumptions |  |  |  |  |  |  |  |  |
| Summary of change: |  | Co-location reference antenna gain towards DUT is corrected to match the core specification |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Error in the antenna gain assumption remains |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

##### 5.6.6.2.3	Conformance requirements

During conformance testing the co-location reference antenna may be replaced with a measurement antenna with a known performance. The core requirement level is translated to a conformance requirement level considering the difference in OTA performance between the co-location reference antenna and the measurement antenna.

The translation from the co-location reference antenna to the measurement antenna are FFS.

{editors note: to be completed as part of conformance work}

Measurements are done with test antennas which are co-location reference antenna as described in subclause 5.1.2. These antennas will effectively be passive single column BS antenna suitable for the specified frequency range. As such the signal level measured during the test will be the same as the core requirement, which is very low:

- Tx OFF level (E-UTRA only),	-106 dBm/MHz

- Protection of own other Rx,	-119dBm/100kHz (E-UTRA BC2), -120 dBm/100kHz (UTRA)

- Co-location with other BS, -129dBm/100kHz (E-UTRA, GSM bands),  -122dBm/100kHz (UTRA, GSM bands)

The worst case is the UTRA co-location requirement which is only 2dB above the thermal noise floor.

As the emission received at the measurement receiver PUEM for co-location requirements are very low, it is suggested to measure the noise change. The relations between measure noise change 1, noise floor N0 and the relation to PUEM with respect to the noise floor denoted 2 is visualized in Figure 5.6.6.2.3-1.

Figure 5.6.6.2.3-1: Relative noise measurement

The absolute emission level is calculated as:

, where N0o is the noise floor of the measurement receiver and 2 is found in Figure 5.6.6.2.3-2.

Figure 5.6.6.2.3-2: Measured noise change to relative noise level

The absolute noise floor of the measurement receiver, including probe antenna, cables, filter and LNA is determined by a calibration procedure. The calibration will determine the absolute accuracy of measuring out-of-band unwanted emission close to the thermal noise floor.

### 5.7	Transmitter intermodulation

------------------------------ End of modified sections ------------------------------