3GPP TSG-RAN WG4 Meeting AH-1807	R4-1809163

# Montreal, CA, 2nd – 6th July, 2018

Source:	NTT DOCOMO, INC.

Title:	TT of OTA ACLR relative requirement

Agenda item:	4.2.1.1.4.1

Document for:	Approval

- Introduction

This contribution discusses the TT of OTA ACLR relative test.

2. TT of conductive OTA ACLR relative test in NR 1-O

In the last RAN4#87 meeting, the test tolerance (TT) of OTA ACLR relative requirement for NR 1-O is agreed as shown in Table 1 [1]. Here, the TT of the ACLR relative requirement is 0.8 dB for BW of below and equal to 20 MHz.

Table1.  Summary of current agreement of MU/TT, general time plan and open issues for BS type 1-O [1]

|  | Package | Sub-clauses | Req | Req Type | Current agreement (TS 38.141-2 BS type 1-O) |  | Target complete date |  |  | Priority | Open issue |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | MU | TT | Technical solution | Draft CR text/MU agreed | CR agreed |  |  |
| TX | Tx output power | 6.2 | Radiated transmit power | EIRP/ directional | Normal: ±1.0 dB, f ≤ 3.0 GHz ±1.2 dB, 3.0 GHz < f ≤ 4.2 GHz FFS, 4.2 GHz < f ≤ 6.0 GHz Extreme: FFS | Normal: ±1.0 dB, f ≤ 3.0 GHz ±1.2 dB, 3.0 GHz < f ≤ 4.2 GHz FFS, 4.2 GHz < f ≤ 6.0 GHz Extreme: FFS | RAN4 AH-1807 | RAN4 AH-1807 | RAN4#88 | 1st | - MU for 4.2-6GHz - Transform from MU to TT |
|  |  | 6.3 | OTA Base Station output power (accuracy) | TRP (in-band) | FFS | FFS | RAN4 AH-1807 | RAN4 AH-1807 | RAN4#88 | 1st | Regulation - TRP systematic error - MU - Transform from MU to TT |
|  | Transmit power dynamics | 6.4 | OTA output power dynamics | EIRP/ directional | FFS | FFS | RAN4 AH-1807 | RAN4 AH-1807 | RAN4#88 |  | - MU for 4.2-6GHz - Transform from MU to TT |
|  | ON/OFF power | 6.5 | OTA transmit ON/OFF power | co-location | FFS | FFS | RAN4 AH-1807 | RAN4 AH-1807 | RAN4#88 |  | - MU - Transform from MU to TT |
|  | Transmit signal quality | 6.6 | OTA frequency Error | EIRP/ directional | [±12Hz] | ±12Hz | RAN4 AH-1807 | RAN4 AH-1807 | RAN4#88 | 1st | Regulation - MU - Transform from MU to TT |
|  |  |  | OTA time alignment error | EIRP/ directional | FFS | FFS | RAN4 AH-1807 | RAN4 AH-1807 | RAN4#88 |  |  |
|  |  |  |  |  |  |  |  |  |  |  | - MU - Transform from MU to TT |
|  |  |  | OTA Modulation quality (EVM) | EIRP/ directional | FFS | FFS | RAN4 AH-1807 | RAN4 AH-1807 | RAN4#88 |  |  |
|  |  |  |  |  |  |  |  |  |  |  | - MU - Transform from MU to TT |
|  | Occupied BW | 6.7.2, | Occupied bandwidth | EIRP/ directional | FFS | 0Hz | RAN4 AH-1807 | RAN4 AH-1807 | RAN4#88 | 1st | Regulation - MU - Transform from MU to TT |
|  | ACLR , | 6.7.3 | OTA Adjacent Channel Leakage power Ratio (ACLR/CACLR) | TRP (in-band) | Relative (<=20MHz): [0.8dB] Relative (>20MHz): FFS Absolute: FFS | Relative (<=20MHz): 0.8dB Relative (>20MHz): FFS Absolute:0dB | RAN4 AH-1807 | RAN4 AH-1807 | RAN4#88 | 1st | Regulation - TRP systematic error - MU - Transform from MU to TT |
|  | In-band UEM | 6.7.4 | Operating band unwanted emission | TRP (in-band) | FFS | Offsets < 10MHz FFS Offsets ≥ 10MHz 0dB | RAN4 AH-1807 | RAN4 AH-1807 | RAN4#88 | 1st | Regulation - TRP systematic error - MU - Transform from MU to TT |
|  | Out of band spurious emissions (Tx and Rx) | 6.7.5 | Spurious emission (general) | TRP (out-of-band) | FFS | 0dB | RAN4 AH-1807 | RAN4 AH-1807 | RAN4#88 | 1st | Regulation - TRP systematic error - MU - Transform from MU to TT |
|  |  |  | co-existence (same geographical area) | TRP (out-of-band) | FFS | FFS |  |  |  |  | - TRP systematic error - MU - Transform from MU to TT |
|  |  |  | Protection of own BS Co-location | co-location | FFS | FFS | RAN4 AH-1807 | RAN4 AH-1807 | RAN4#88 | 1st | Regulation - MU - Transform from MU to TT |
|  | Tx IMD | 6.8 | Transmitter intermodulation | co-location | FFS | FFS | RAN4 AH-1807 | RAN4 AH-1807 | RAN4#88 | 1st | Regulation - MU - Transform from MU to TT |
| RX | Rx sensitivity | 7.2 | OTA sensitivity | EIS/ directional | ±1.3 dB, f ≤ 3.0 GHz ±1.4 dB, 3.0 GHz < f ≤ 4.2 GHz FFS, 4.2 GHz < f ≤ 6.0 GHz | ±1.3 dB, f ≤ 3.0 GHz ±1.4 dB, 3.0 GHz < f ≤ 4.2 GHz FFS, 4.2 GHz < f ≤ 6.0 GHz | RAN4 AH-1807 | RAN4 AH-1807 | RAN4#88 |  | Regulation - MU for 4.2-6GHz - Transform from MU to TT |
|  |  | 7.3 | OTA Reference sensitivity level | EIS/ directional | ±1.3 dB, f ≤ 3.0 GHz ±1.4 dB, 3.0 GHz < f ≤ 4.2 GHz FFS, 4.2 GHz < f ≤ 6.0 GHz | ±1.3 dB, f ≤ 3.0 GHz ±1.4 dB, 3.0 GHz < f ≤ 4.2 GHz FFS, 4.2 GHz < f ≤ 6.0 GHz | RAN4 AH-1807 | RAN4 AH-1807 | RAN4#88 |  | Regulation - MU for 4.2-6GHz - Transform from MU to TT |
|  | Rx inband performance | 7.4 | OTA Dynamic range | EIS/ directional | FFS | FFS | RAN4 AH-1807 | RAN4 AH-1807 | RAN4#88 |  | - MU - Transform from MU to TT |
|  |  | 7.5 | OTA Adjacent channel selectivity | EIS/ directional | FFS | 0dB | RAN4 AH-1807 | RAN4 AH-1807 | RAN4#88 |  | Regulation - MU - Transform from MU to TT |
|  |  |  | , OTA in-band blocking | EIS/ directional | FFS | [0dB (general blocking)] FFS (Narrowband blocking) | RAN4 AH-1807 | RAN4 AH-1807 | RAN4#88 |  | Regulation - MU - Transform from MU to TT |
|  |  | 7.8 | OTA receiver intermodulation | EIS/ directional | FFS | 0dB | RAN4 AH-1807 | RAN4 AH-1807 | RAN4#88 |  | Regulation - MU - Transform from MU to TT |
|  |  | 7.9 | OTA in-channel selectivity | EIS/ directional | FFS | FFS | RAN4 AH-1807 | RAN4 AH-1807 | RAN4#88 |  | - MU - Transform from MU to TT |
|  | Rx blocking (oob) | 7.6 | OTA out of band Blocking | EIS/ directional | FFS | FFS | RAN4 AH-1807 | RAN4 AH-1807 | RAN4#88 |  | - MU - Transform from MU to TT |
|  | part of "Out of band spurious emissions (Tx and Rx)" above | 7.7 | OTA Receiver spurious emissions | TRP (out-of-band) | FFS | 0dB | RAN4 AH-1807 | RAN4 AH-1807 | RAN4#88 | 1st | Regulation - TRP systematic error - MU - Transform from MU to TT |
|  | OTA Perf | 8 | Radiated Performance characteristics | EIS/ directional | FFS | FFS | RAN4 #88 | RAN4 #88bis | RAN4 #89 |  | - Core requirement - MU - Transform from MU to TT etc |

Observation1: TT of the OTA ACLR relative requirement for NR 1-O is 0.8.

Since the BW <= 20 MHz is the same as eAAS, TT for OTA ACLR relative requirement for eAAS can be aligned to that for NR 1-O. We propose TT of OTA ACLR relative requirement for AAS and eAAS is the same as that for NR 1-O, 0.8.

Proposal: TT for OTA ACLR relative requirement for AAS and eAAS is 0.8

# 4	Summary

In this document, we have discussed the TT for OTA ACLR relative requirement. We made the following proposals based on the following observations.

Observation1: TT of the OTA ACLR relative requirement for NR 1-O is 0.8.

Proposal: TT for OTA ACLR relative requirement is 0.8

References

[1] R4-1808514, “WF on test tolerance for the NR requirement,” NTT DOCOMO, RAN4#87, May, 2018