3GPP TSG-RAN WG4 Meeting AH-1807                	R4-1809174

Montreal, Canada, 2 – 6 July 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR for 38.133 on Interruptions due to measurements on deactivated NR SCells |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2017-06-25 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Requirements for EN-DC and SA interruptions when NR cell is the victim are incomplete in 38.133. Requirements for interruptions due to measurements on deactivated NR SCells are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Requirements for interruptions during measurements on deactivated NR SCells are introduced in section 8.2.2. Section 8.2.1 for EN-DC is referring to this section already. Some editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 8.2.2	SA: Interruptions with Standalone NR Carrier Aggregation

#### 8.2.2.1	Introduction

This section contains the requirements related to the interruptions on PCell and activated SCell if configured, when up to TBD SCells are configured, deconfigured, activated or deactivated.

Note:	interruptions at SCell addition/release, activation/deactivation and during measurements on SCC may not be required by all UEs.

Editor’s Note:	The interruptions shall not interrupt RRC signalling or ACK/NACKs related to RRC reconfiguration procedure [2] for SCell addition/release or MAC control signalling [17] for SCell activation/deactivation command. How to specify this is FFS.

#### 8.2.2.2	Requirements

##### 8.2.2.2.1	Interruptions at SCell addition/release

When any number of SCells between one and TBD is added or released using the same RRCConnectionReconfiguration message as defined in [2], the UE is allowed an interruption on PCell and on any activated SCell during the RRC reconfiguration procedure as follows:

-	an interruption on PCell:

-	of up to the duration shown in table 8.2.2.2.1-1, if the PCell is not in the same band as any of the SCells being added or released, or

-	of up to the duration shown in table 8.2.2.2.1-2, if the PCell is in the same band as any of the SCells being added or released, provided the cell specific reference signals from the PCell and the SCells being added or released are available in the same slot;

-	an interruption on any activated SCell:

-	of up to to the duration shown in table 8.2.2.2.1-1, if the activated SCell is not in the same band as any of the SCells being added or released, or

-	of up to the duration shown in table 8.2.2.2.1-2, if the activated SCell is in the same band as any of the SCells being added or released, provided the cell specific reference signals from the activated SCell and the SCells being added or released are available in the same slot.

Table 8.2.2.2.1-1: Interruption duration for SCell addition/release for inter-band CA

|  | NR Slot length (ms) | Interruption length (slot) |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 2 |
| 2 | 0.25 | 4 |
| 3 | 0.125 | 8 |

Table 8.2.2.2.1-2: Interruption duration for SCell addition/release for intra-band CA

|  | NR Slot length (ms) | Interruption length (slot) |
| --- | --- | --- |
| 0 | 1 | 1 + SMTC duration |
| 1 | 0.5 | 2 + SMTC duration |
| 2 | 0.25 | 4 + SMTC duration |
| 3 | 0.125 | 8 + SMTC duration |

##### 8.2.2.2.2	Interruptions at SCell activation/deactivation

When an intra-band SCell is activated or deactivated as defined in [17] the UE is allowed

-	an interruption on PCell:

-	of up to the duration shown in table 8.2.2.2.2-1, if the PCell is not in the same band as any of the SCells being activated or deactivated, or

-	of up to the duration shown in table 8.2.2.2.2-2 if the PCell is in the same band as any of the SCells being activated or deactivated provided the cell specific reference signals from the PCell and the SCells being activated or deactivated are available in the same slot;

-	an interruption on any activated SCell:

-	of up to the duration shown in table 8.2.2.2.2-1, if the activated SCell is not in the same band as any of the SCells being activated or deactivated, or

-	of up to the duration shown in table 8.2.2.2.2-2, if the activated SCell is in the same band as any of the SCells being activated or deactivated provided the cell specific reference signals from the PCell and the SCells being activated or deactivated are available in the same slot.

Table 8.2.2.2.2-1: Interruption duration for SCell activation/deactivation for inter-band CA

|  | NR Slot length (ms) | Interruption length (slot) |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 1 |
| 2 | 0.25 | 2 |
| 3 | 0.125 | 4 |

Table 8.2.2.2.2-2: Interruption duration for SCell activation/deactivation for intra-band CA

|  | NR Slot length (ms) | Interruption length (slot) |
| --- | --- | --- |
| 0 | 1 | 1 + SMTC duration |
| 1 | 0.5 | 2 + SMTC duration |
| 2 | 0.25 | 4 + SMTC duration |
| 3 | 0.125 | 8 + SMTC duration |

##### 8.2.2.2.3	Interruptions during measurements on SCC for intra-band CA

When one SCell in MCG is deactivated, the UE is allowed on PSCell or any activated SCell due to measurements on the SCC with the deactivated SCell immediately before and immediately after the measured SMTC as indicated to the UE for measurements by the network, an interruption of up to the duration shown in table 8.2.2.2.3-1.

Table 8.2.2.2.3-1: Interruption duration for measurements on deactivated SCC

|  | NR Slot length (ms) | Interruption length (slot) |
| --- | --- | --- |
| 0 | 1 | [1] |
| 1 | 0.5 | [1] |
| 2 | 0.25 | [2] |
| 3 | 0.125 | [4] |

##### 8.2.2.2.4	Interruptions at UL carrier RRC reconfiguration

The requirements in this clause shall apply when a supplementary UL carrier or an UL carrier is configured or de-configured in NR standalone carrier aggregation as defined in [2].

When an UL carrier or supplementary UL carrier is configured or deconfigured, an interruption on PCell and all activated SCells within the same FR as the reconfigured uplink carrier of up to [1] slot, is allowed immediately after the RRC reconfiguration procedure [2]. The interruption is for both uplink and downlink of PCell and all the activated SCells within the same FR as the configured or de-configured UL.