3GPP TSG-RAN5 Meeting #74	R5-170773

# Athens, Greece, 13th Feb 2017 - 17th Feb 2017

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 3832 | rev | - | Current version: | 13.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | D2D - Moving TC 7.3.8.3 to Section 19.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | QUALCOMM Incorporated, Tech Mahindra |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_D2D_Prox-UEConTest |  |  |  |  |  | Date: |  |  | 2017-02-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Moving TC 7.3.8.3 to Section 19.1 as it belongs to Prose Direct Communication and does not test PDCP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Void TC 7.3.8.3 and reassign it to Cl 19.1.8 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | D2D WP will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 36.523-2 CR 0972 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 7.3.8	PDCP ProSe Device to Device

<Start of changed section>

#### 7.3.8.3	Void

<Start of changed section>

<Skipped unchanged section>

## 19.1	ProSe Direct communication

<Start of changed section>

#### 19.1.8	Security Aspects/ ProSe Direct Communication/ release of PDN Connection used to receive MIKEY Messages/ Correct Key Request Message/ MIKEY Verification Message

19.1.8.1	Test Purpose (TP)

(1)

with { UE served by E-UTRAN PLMN supporting ProSe and intending to use One-to-many ProSe direct communication }

ensure that {

when { the UE received MIKEY message on additional PDN and UE is triggered to initiate deactivation of the additional PDN }

then { the UE shall not send PDN DISCONNECT REQUEST }

}

(2)

with { the UE has informed the ProSe Key Management Function that it no longer requires PGK }

ensure that {

when { the UE is triggered to initiate deactivation of the additional PDN }

then { the UE shall send PDN DISCONNECT REQUEST  }

}

19.1.8.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 33.303, clause 6.2.3.3.2.2

[TS 33.303, clause 6.2.3.3.2.2]

…

The UE shall not release the PDN connection used to receive MIKEY messages containing PGKs until the UE has informed the ProSe Key Management Function that it no longer requires PGKs. This is to ensure that the ProSe Key Management Function is aware of the correct UE IP address for the purpose of performing PGK deliveries as specified in clause 6.2.3.3.2.3.

…

19.1.8.3	Test description

19.1.8.3.1	Pre-test conditions

System Simulator:

-	Cell 1

-	System information combination 23 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA Cell 1.

UE:

-	The UE is authorised to perform ProSe Direct Communication.

-	The UE has pre-configured radio parameters (preconfigComm) as defined in TS 36.508 with an associated geographical area and a UICC with all values equal to the default profile given inTS 36.508 [18], section 4.9.3.1 except for those listed in Table 19.1.8.3.1-1.

Table 19.1.8.3.1-1: USIM configuration

| USIM field | Priority | Value |
| --- | --- | --- |
| EFPROSE_RADIO_COM |  | Is present |
| EFUST |  | Service 101 is supported |
| EFAD |  | UE is authorized to use pre-configured parameters for ProSe |
| EFPROSE_PLMN |  | PLMN of Cell 1 |
| EFPST |  | Service n°3 and service n°6 are supported |
| EFPROSE_POLICY |  | Group ID = ‘0000 0000 0000 0000 0000 0000’ [BIN] |
|  |  | ProSe UE ID = ‘0000 0000 0000 0000 0000 0001’ [BIN] |

Preamble:

-	The UE is in state Registered, Idle mode (state 2) according to [18].

19.1.8.3.2	Test procedure sequence

Table 19.1.8.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Trigger the UE to send the KEY_REQUEST. (Note 1) | - | - | - | - |
| 2-12 | TS 36.508 clause 4.5A.22: Step 1-11 of Communication with the ProSe Function procedure is completed. TS 36.508 Table 4.5A.22.3-2: Step 1a1 to 9 is only executed | - | - | - | - |
| 13 | UE sends KEY_REQUEST message to the ProSe Key Management Function | --> | HTTP Request containing KEY_REQUEST | - | - |
| 14 | SS transmits a KEY_RESPONSE message | <-- | HTTP Response containing KEY_RESPONSE | - | - |
| 15 | SS transmits a MIKEY message containing PGK | <-- | MIKEY message containing Key Delivery message | - | - |
| 16 | UE sends MIKEY verification message | --> | MIKEY message containing verification message | - | - |
| 17 | Initiate the deactivation of the additional PDN in UE. (see Note 2) | - | - | - | - |
| 18 | Check: Does the UE transmit PDN DISCONNECT REQUEST message? | --> | PDN DISCONNECT REQUEST | - | F |
| 19 | Trigger the UE to send KEY_REQUEST message to the ProSe Key Management Function to inform that UE no longer requires PGK. (Note 3) | --> | HTTP Request containing KEY_REQUEST | - | - |
| 20 | SS transmits a KEY_RESPONSE message | <-- | HTTP Response containing KEY_RESPONSE | - | - |
| 21 | Initiate the deactivation of the additional PDN in UE. (see Note 2) | - | - | - | - |
| 22 | Check: Does the UE transmit PDN DISCONNECT REQUEST message? | --> | PDN DISCONNECT REQUEST | - | P |
| 23 | Deactivation of the additional PDN is triggered in UE. The additional PDN shall be released as specified in steps 10-13 of TS 36.508 subclause 4.5A.17. | - | - | - | - |
| Note 1: The trigger in step 1 is the same as in the generic procedure in 36.508 clause 4.5A.22 Note 2: Deactivation of the additional PDN is initiated by MMI or AT command Note 3: Trigger is initiated by MMI or AT command |  |  |  |  |  |

19.1.8.3.3	Specific message contents:

Table 19.1.8.3.3-1: KEY_REQUEST (step 19, table 19.1.8.3.2-1)

| Derivation path: 36.508 Table 4.7F.3-1 |  |  |  |
| --- | --- | --- | --- |
| Field | Value/remark | Comment | Condition |
| transaction-ID | Any |  |  |
| GroupKeyReq | Not Present |  |  |
| GroupKeyStop | One entry |  |  |
| GroupId | Same as the GroupId sent in Step 13 |  |  |

Table 19.1.8.3.3-2: KEY_RESPONSE (step 20, table 19.1.8.3.2-1)

| Derivation path: 36.508 Table 4.7F.3-2 |  |  |  |
| --- | --- | --- | --- |
| Field | Value/remark | Comment | Condition |
| transaction-ID | Same as in Step 19 |  |  |
| GroupNotSupported | One entry |  |  |
| GroupId | Same as the GroupId sent in Step 19 |  |  |
| Error-Code | 4 | UE requested to stop receiving PGKs for this group |  |
| GroupResponse | Not Present |  |  |
| Key-info | Not Present |  |  |

<End of changed section>