3GPP TSG-RAN5 Meeting #75	R5-172453

# Hangzhou, China, 15th May 2017 - 19th May 2017

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 4025 | rev | - | Current version: | 14.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on RRC V2V test case 24.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_SL_V2V-UEConTest |  |  |  |  |  | Date: |  |  | 2017-05-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Band 47 is introduced by RAN4 to support V2X PC5. eNB can configured V2X sidelink communication via cross-carrier configuration only. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change the title of the test case. - Delete TP1. Since Uu is not on the V2X sidelink communication carrier. UE can use pre-configuration when there isn’t cross-carrier V2X configuration by eNB. - Change the frequency of cell 1 and cell 3 from the frequency of V2X sidelink communication to the anchor frequency. - Change the test step and specific message contents to align the changes in test purpose. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case can’t work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 24.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

CHANGE START

24.1.1	V2X Sidelink Communication / Pre-configured authorisation / UE in RRC_IDLE on an E-UTRAN cell operating on the anchor carrier frequency provisioned for V2X configuration / Utilisation of the resources of (serving) cells/PLMNs / Transmission

24.1.1.1	Test Purpose (TP)

(1)

Void

(2)

with { UE being authorized for performing V2X sidelink Communication in PLMN1 and pre-configured with Radio parameters for when the UE is "not served by E-UTRAN", and, UE is in RRC_IDLE on Cell1/f1/PLMN1 which is operating on the anchor carrier frequency as the one pre-configured in the UE and is transmitting SystemInformationBlockType21 which does not include v2x-CommTxPoolNormalCommon }

ensure that {

when { UE receives a request from upper layers to transmit V2X sidelink communication }

then { UE initiates an RRC connection, and, successfully completes a Sidelink UE information procedure to indicate the V2X sidelink communication transmission resources required }

}

(3)

with { UE being authorized for performing V2X sidelink Communication in PLMN1) and pre-configured with Radio parameters for when the UE is "not served by E-UTRAN", and, UE is in RRC_IDLE on Cell1/f1/PLMN1 which is operating on the anchor carrier frequency as the one pre-configured in the UE and is transmitting SystemInformationBlockType21 which includes v2x-CommTxPoolNormalCommon }

ensure that {

when { UE is triggered by an upper layer application to transmit V2X sidelink communication }

then { UE is able to transmit V2X sidelink communication using the configured resources in Cell1/f1/PLMN1 via SystemInformationBlockType21 }

}

(4)

with { UE being authorized for performing V2X sidelink Communication in PLMN1 and pre-configured with Radio parameters for when the UE is "not served by E-UTRAN", and, UE is in RRC_IDLE on Cell1/f1/PLMN1 which is not transmitting SystemInformationBlockType21 }

ensure that {

when { Cell3/f2/PLMN1  which is broadcasting SystemInformationBlockType21 providing resources for V2X sidelink communication becomes the highest ranked cell, and, UE reselects to Cell3/f2/PLMN1 }

then { UE is able to transmit V2X sidelink communication using the configured resources in Cell3/f2/PLMN1) }

}

24.1.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 23.285, clauses 4.4.1.1.1, 4.4.1.1.2, 4.4.2, 4.5.1, 5.3, TS 36.331, clauses 5.2.2.4, 5.2.2.28, 5.10.1d, 5.10.13.1.

[TS 23.285, clause 4.4.1.1.1]

The basic principles of service authorization for V2X communications over PC5 reference point:

-	The UE gets authorization to use V2X communications over PC5 reference point on a per PLMN basis in the serving PLMN by the V2X Control Function in the HPLMN.

-	The V2X Control Function in the HPLMN requests authorization information from the V2X Control Function of the serving PLMN.

-	The V2X Control Function in the HPLMN merges authorization information from home and serving PLMNs and informs the UE of the final authorization information.

-	The V2X Control Function in the VPLMN or HPLMN may revoke the authorization at any time. The V2X Control Function in the HPLMN shall be notified when authorization is revoked by the VPLMN.

[TS 23.285, clause 4.4.1.1.2]

The following information is provisioned to the UE for V2X communications over PC5 reference point:

1)	Authorization policy:

-	When the UE is "served by E-UTRAN":

-	PLMNs in which the UE is authorized to perform V2X communications over PC5 reference point.

-	When the UE is "not served by E-UTRAN":

-	Indicates whether the UE is authorized to perform V2X communications over PC5 reference point when "not served by E-UTRAN".

2)	Radio parameters for when the UE is "not served by E-UTRAN":

-	Includes the radio parameters with Geographical Area(s) that need to be configured in the UE in order to be able perform V2X communications over PC5 reference point when "not served by E-UTRAN". These radio parameters (e.g. frequency bands) are defined in TS 36.331 [9]. The UE uses the radio parameters only if the UE can locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

3)	Policy/parameters for V2X communication over PC5 reference point:

-	The mapping of Destination Layer-2 ID(s) and the V2X services, e.g. PSID or ITS-AIDs of the V2X application.

NOTE 1:	PLMN operators coordinate to make sure Destination Layer-2 ID(s) for different V2X services are configured in a consistent manner.

NOTE 2:	To pre-configure a UE with the provisioning parameters, at least the "not served by E-UTRAN" parameters of 1) and 2), and the parameters of 3) need to be included.

-	The mapping of ProSe Per-Packet Priority and packet delay budget for V2X communication (autonomous resources selection mode).

[TS 23.285, clause 4.4.2]

PC5 reference point as defined in TS 23.303 [5] is used for the transmission and reception of V2X messages. The V2X communication over PC5 reference point supports roaming and inter-PLMN operations. V2X communication over PC5 reference point is supported when the UE is "served by E-UTRAN" and when the UE is "not served by E-UTRAN".

A UE is authorized to transmit and receive V2X messages by the V2X Control Function in its home PLMN as described in clause 5.2.

The V2X communication over PC5 reference point is a type of ProSe Direct Communication with the following characteristics:

-	The V2X communication over PC5 reference point is connectionless, and there is no signalling over PC5 control plane for connection establishment.

-	V2X messages are exchanged between UEs over PC5 user plane.

-	Both IP based and non-IP based V2X messages are supported.

-	For IP based V2X messages, only IPv6 is used. IPv4 is not supported in this release.

The identifiers used in the V2X communication over PC5 reference point are described in clause 4.5.1.

[TS 23.285, clause 4.5.1]

Each UE has a Layer-2 ID for the V2X communication over PC5 reference point, which is included in the source Layer-2 ID field of every frame that it sends on the layer-2 link. The UE self-assigns the Layer-2 ID for the V2X communication over PC5 reference point.

When IP based V2X messages are supported, the UE auto-configures a link local IPv6 address to be used as the source IP address, as defined in clause 4.5.3 of TS 23.303 [5].

In order to ensure that a source UE (e.g. vehicle) cannot be tracked or identified by any other UEs (e.g. vehicles) beyond a certain short time-period required by the application, the source Layer-2 ID must be changed over time and randomized. For IP based V2X communication over PC5 reference point, the source IP address must be also changed over time and randomized. The change of the identifiers of a source UE must be synchronized across layers used for PC5, e.g. when application layer identifier changes, the source Layer-2 ID and the source IP address need to be changed.

The UE is configured with the destination Layer-2 ID(s) to be used for V2X services. The Layer-2 ID for a V2X message is selected based on the configuration as described in clause 4.4.1.1.

[TS 23.285, clause 5.3]

To perform V2X communication over PC5 reference point, the UE is configured with the related information as described in clause 4.4.1.1.

The procedure for one-to-many ProSe Direct Communication transmission described in clause 5.4.2 of TS 23.303 [5] is applied to V2X communication over PC5 reference point with following differences:

-	The source Layer-2 ID is set to the Layer-2 ID described in clause 4.5.1.

-	A UE shall be configured with a set of Layer-2 ID corresponding to different type of services.

The procedure for one-to-many ProSe Direct Communication reception described in clause 5.4.3 of TS 23.303 [5] is applied to V2X communication over PC5 reference point.

[TS 36.331, clause 5.2.2.4]

1>	if the UE is capable of V2X sidelink communication and is configured by upper layers to receive or transmit V2X sidelink communication on a frequency:

2> 	if the cell used for V2X sidelink communication meets the S-criteria as defined in TS 36.304 [4]; and

2>	if schedulingInfoList on the concerned frequency indicates that SystemInformationBlockType21 is present and the UE does not have stored a valid version of this system information block:

3>	acquire SystemInformationBlockType21;

[TS 36.331, clause 5.2.2.28]

Upon receiving SystemInformationBlockType21, the UE shall:

1>	if SystemInformationBlockType21 message includes sl-V2X-ConfigCommon:

2>	if configured to receive V2X sidelink communication:

3>	use the resource pool indicated by v2x-CommRxPool in sl-V2X-ConfigCommon for V2X sidelink communication monitoring, as specified in 5.10.12;

2>	if configured to transmit V2X sidelink communication:

3>	use the resource pool indicated by v2x-CommTxPoolNormalCommon or by v2x-CommTxPoolExceptional for V2X sidelink communication transmission, as specified in 5.10.13;

[TS 36.331, clause 5.10.1d]

When it is specified that the UE shall perform V2X sidelink communication operation only if the conditions defined in this section are met, the UE shall perform V2X sidelink communication operation only if:

1>	if the UE’s serving cell is suitable (RRC_IDLE or RRC_CONNECTED); and if either the selected cell on the frequency used for V2X sidelink communication operation belongs to the registered or equivalent PLMN as specified in TS 24.334 [69] or the UE is out of coverage on the frequency used for V2X sidelink communication operation as defined in TS 36.304 [4, 11.4]; or

1>	if the UE has no serving cell (RRC_IDLE);

[TS 36.331, clause 5.10.13.1]

A UE capable of V2X sidelink communication that is configured by upper layers to transmit V2X sidelink communication and has related data to be transmitted shall:

1>	if the conditions for sidelink operation as defined in 5.10.1d are met:

2>	if in coverage on the frequency used for V2X sidelink communication as defined in TS 36.304 [4, 11.4]; or

2>	if the frequency used to transmit V2X sidelink communication is included in v2x-InterFreqInfoList in RRCConnectionReconfiguration or in v2x-InterFreqInfoList within SystemInformationBlockType21:

3>	if the UE is in RRC_CONNECTED:

4>	if the UE is configured, by the current PCell with commTxResources set to scheduled:

5>	if T310 or T311 is running; and if the PCell at which the UE detected physical layer problems or radio link failure broadcasts SystemInformationBlockType21 including v2x-CommTxPoolExceptional in sl-V2X-ConfigCommon; or

5>	if T301 is running and the cell on which the UE initiated connection re-establishment broadcasts SystemInformationBlockType21 including v2x-CommTxPoolExceptional in sl-V2X-ConfigCommon; or

6>	configure lower layers to transmit the sidelink control information and the corresponding data based on sensing using the pool of resources indicated by v2x-CommTxPoolExceptional as defined in TS 36.321 [6] and TS 36.213 [23];

NOTE 1:	The UE is assumed to perform sensing on the resources of the pool (as defined in TS 36.213 [23]) indicated by v2x-CommTxPoolExceptional within SystemInformationBlockType21, before it transmits using the pool of resources.

5>	if T304 is running and the UE is configured with v2x-CommTxPoolExceptional included in mobilityControlInfoV2X in RRCConnectionReconfiguration:

6>	configure lower layers to transmit the sidelink control information and the corresponding data based on random selection using the pool of resources indicated by v2x-CommTxPoolExceptional as defined in TS 36.321 [6];

5>	else:

6>	configure lower layers to request E-UTRAN to assign transmission resources for V2X sidelink communication;

4>	else if the UE is configured with v2x-commTxPoolNormalDedicated in RRCConnectionReconfiguration:

5>	if the time required by the physical layer in TS 36.213 [23] for sensing on the resources configured in v2x-CommTxPoolNormalDedicated is not met;

6>	if v2x-CommTxPoolExceptional is included in mobilityControlInfoV2X in RRCConnectionReconfiguration (i.e., handover case);

7>	configure lower layers to transmit the sidelink control information and the corresponding data based on random selection using the pool of resources indicated by v2x-CommTxPoolExceptional as defined in TS 36.321 [6];

6>	else if the Pcell broadcasts SystemInformationBlockType21 including v2x-CommTxPoolExceptional in sl-V2X-ConfigCommon;

7>	configure lower layers to transmit the sidelink control information and the corresponding data based on sensing using the pool of resources indicated by v2x-CommTxPoolExceptional as defined in TS 36.321 [6] and TS 36.213 [23];

5>	else:

6>	configure lower layers to transmit the sidelink control information and the corresponding data based on sensing (as defined in TS 36.321 [6] and TS 36.213 [23]) using one of the resource pools indicated by v2x-commTxPoolNormalDedicated, which is selected according to 5.10.13.2;

3>	else (i.e. V2X sidelink communication in RRC_IDLE or on cell other than PCell in RRC_CONNECTED):

4>	if the cell chosen for V2X sidelink communication transmission broadcasts SystemInformationBlockType21:

5>	if SystemInformationBlockType21 includes v2x-CommTxPoolNormalCommon in sl-V2X-ConfigCommon:

6>	configure lower layers to transmit the sidelink control information and the corresponding data based on sensing (as defined in TS 36.321 [6] and TS 36.213 [23]) using one of the resource pools indicated by v2x-CommTxPoolNormalCommon, which is selected according to 5.10.13.2;

5>	else if SystemInformationBlockType21 includes v2x-CommTxPoolExceptional in sl-V2X-ConfigCommon:

6>	from the moment the UE initiates connection establishment until receiving an RRCConnectionReconfiguration including sl-V2X-ConfigDedicated or until receiving an RRCConnectionRelease or an RRCConnectionReject;

7>	configure lower layers to transmit the sidelink control information and the corresponding data based on sensing (as defined in TS 36.321 [6] and TS 36.213 [23]) using the pool of resources indicated in v2x-CommTxPoolExceptional;

2>	else (i.e. out of coverage on sidelink carrier):

3>	configure lower layers to transmit the sidelink control information and the corresponding data based on sensing (as defined in TS 36.321 [6] and TS 36.213 [23]) using one of the resource pools indicated by v2x-CommTxPoolList in SL-V2X-Preconfiguration, which is selected according to 5.10.13.2, and in accordance with the timing of the selected reference as defined in 5.10.8;

24.1.1.3	Test description

24.1.1.3.1	Pre-test conditions

System Simulator:

SS-NW

-	Cell 1 and Cell 3

Table 24.1.1.3.1-1: Cell parameters values

| Cell | Frequency | PLMN |
| --- | --- | --- |
| 1 | f1 | HPLMN (PLMN1) |
| 3 | f2 | PLMN1 |
| Note 1: The Frequency f1 and f2 shall be the anchor frequency pre-configured in the UE for when UE is "not served by E-UTRAN". |  |  |

-	System information combination 29 as defined in TS 36.508 [18] clause 4.4.3.1 is used in all active cells.

SS-UE

-	SS-UE1. As defined in TS 36.508 [18], configured for and operating as V2X Sidelink Communication receiving device on the resources which the UE is expected to use for transmission.

UE:

-	V2X sidelink related configuration

-	The UE is authorised to perform V2X Sidelink Communication

Preamble:

-	The UE is in State Switched OFF (state 1) according to TS 36.508 [18].

24.1.1.3.2	Test procedure sequence

Table 24.1.1.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" ... "Tn" are to be applied subsequently. The exact instants on which these values shall be applied are described elsewhere in the present clause.

Table 24.1.1.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 3 |
| --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | "Off" |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 |

Table 24.1.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: SS-NW - Cell 1 transmits SystemInformationBlockType21 neither including v2x-CommTxPoolNormalCommon nor v2x-CommTxPoolNormal in v2x-InterFreqInfoList (i.e. V2X sidelink communication supported by the network, no common resources provided yet).. | - | - | - | - |
| 2 | The UE is switched on. | - | - | - | - |
| - | EXCEPTION: The following events unless otherwise stated are to be observed in Cell 1. | - | - | - | - |
| 3 | Force the UE upper layer application to request continuous transmission of V2X sidelink communication (a maximum of 300 Bytes per communication "message"). NOTE: This can be done e.g. via a MMI command. Note that the max of 300 Bytes is not a 3GPP requirement rather it is requested only for the purpose of facilitating the test case specification. | - | - | - | - |
| 4 | Void | --> | - | - | - |
| 5 | Void | - | - | - | - |
| 6 | Check: Does the UE transmit an RRCConnectionRequest message? | --> | RRCConnectionRequest | 2 | P |
| 7 | SS-NW transmits an RRCConnectionSetup message. | <-- | RRCConnectionSetup | - | - |
| 8 | Check: Does the UE transmit an RRCConnectionSetupComplete message? | --> | RRCConnectionSetupComplete | 2 | P |
| 9 | Check: Does the UE transmit a SidelinkUEInformation message requesting resources for transmission of sidelink communication? | --> | SidelinkUEInformation | 2 | P |
| 10 | SS-NW releases the connection. | <-- | RRCConnectionRelease | - | - |
| 11 | SS-NW changes SystemInformationBlockType21 to include v2x-CommTxPoolNormalCommon. | - | - | - | - |
| 12 | Wait for 2 modification periods to allow for the UE to obtain the new version of the SystemInformationType21. | - | - | - | - |
| 13 | Check: Does the UE transmit STCH PDCP SDU packets of V2X sidelink communication data over the PC5 interface in accordance with the resources indicated in Cell 1 SystemInformationBlockType21? NOTE: The UE may send multiple packets. The reception of one of them is sufficient for achieving the Pass verdict. | --> | STCH PDCP SDU packet | 3 | P |
| 14 | The SS configures: SW-NW Cell 1 and Cell 3 parameters according to the row "T1" in table 23.1.1.3.2-1 in order to simulate needs for cell reselection to Cell3. Cell 3 broadcasts SystemInformationBlockType21 including v2x-CommTxPoolNormalCommon. | - | - | - | - |
| 15 | Wait for 5 sec to allow the UE to adjust to cell changes and start transmission. | - | - | - | - |
| - | EXCEPTION: The following events unless otherwise stated are to be observed in Cell 3. | - | - | - | - |
| 16 | Check: Does the UE transmit STCH PDCP SDU packets of V2X sidelink communication data over the PC5 interface in accordance with the resources indicated in the broadcast on Cell 3 SystemInformationBlockType21? NOTE: The UE may send multiple packets. The reception of one of them is sufficient for achieving the Pass verdict. | --> | STCH PDCP SDU packet | 4 | P |

24.1.1.3.3	Specific message contents

Table 24.1.1.3.3-1: SystemInformationBlockType21 for Cell 1 (step 5, Table 24.1.1.3.2-2)

| Derivation Path: 36.508 [18] , table 4.4.3.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType21-r14 ::= SEQUENCE { |  |  |  |
| sl-V2X-ConfigCommon-r14 SEQUENCE { |  |  |  |
| v2x-CommTxPoolNormalCommon-r14 | Not Present |  |  |
| } |  |  |  |
| v2x-InterFreqInfoList-r14 SEQUENCE { |  |  |  |
| SL-InterFreqInfoV2X-r14[1] SEQUENCE { |  |  |  |
| v2x-UE-SelectionConfigList-r14 SEQUENCE { |  |  |  |
| SL-V2X-InterFreqUE-SelectionConfig-r14[1] SEQUENCE { |  |  |  |
| v2x-CommTxPoolNormal-r14 | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| Note: V2X SideLink communication supported but no resources set for transmission. |  |  |  |

Table 24.1.1.3.3-2: SystemInformationBlockType21 for Cell 1 (when active steps 11 onwards, Table 24.1.1.3.2-2)

Derivation Path: 36.508 [18], table 4.4.3.3-19

Note: V2X SideLink communication supported; resources for transmission in RRC_IDLE provided (v2x-CommTxPoolNormalCommon - TxPool 1) v2x-CommTxPoolNormalCommon-r14.

Table 24.1.1.3.3-3: SystemInformationBlockType21 for Cell 3 (when active steps 11 onwards, Table 24.1.1.3.2-2)

Derivation Path: 36.508 [18], table 4.4.3.3-19

Note: V2X SideLink communication supported; resources for transmission in RRC_IDLE provided (v2x-CommTxPoolNormalCommon - TxPool 2) v2x-CommTxPoolNormalCommon-r14.

Table 24.1.1.3.3-4: SidelinkUEInformation (step 9, Table 24.1.1.3.2-2)

Derivation Path: 36.508 [18], table 4.6.1-21A condition V2V

CHANGE END