3GPP TSG-RAN5 Meeting #84	R5-195828

# Ljubljana, Slovenija,  26th - 30th August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0185 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New Introduction of TC 6.4A.2.2.3 Carrier leakage (4UL CA) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sporton International Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2019-08-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TC6.4A.2.2.4 Carrier leakage for CA (4UL CA) is intrduced in WP & core-spec.[3], so need to add in test specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of Carrier leakage (SA 4UL CA). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TC6.4A.2.2.3 will be still missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4A.2.2.3 (New), F.1.2, F.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## << Unchanged sections omitted >>

6.4A.2.2.3	Carrier leakage for CA (4UL CA)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Test procedure is incompleted due to power window is TBD

- Measurement Uncertainties and Test Tolerances for intra-band contiguous CA is TBD.

6.4A.2.2.3.1	Test purpose

Carrier leakage expresses itself as unmodulated sine wave with the carrier frequency. It is an interference of approximately constant amplitude and independent of the amplitude of the wanted signal. Carrier leakage interferes with the sub carriers at its position (if allocated), especially, when their amplitude is small.

The purpose of this test is to exercise the UE transmitter to verify its modulation quality in terms of carrier leakage.

6.4A.2.2.3.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports FR2 4UL CA.

6.4A.2.2.3.3	Minimum conformance requirements

The normative reference for this requirement is TS 38.101-2 [3] clause 6.4A.2.2.

6.4A.2.2.3.4	Test description

6.4A.2.2.3.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on NR operating bands specified in Table 5.5A.1-1, 5.5A.2-1 and 5.5A.2-2. All of these configurations shall be tested with applicable test parameters for each CA configuration and subcarrier spacing, are shown in table 6.4A.2.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.4A.2.2.3.4.1-1: Intra-band Contiguous CA Test Configuration Table

| Default Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause [4.1] |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause [4.3.1.2.3] for different CA bandwidth classes, and PCC and SCC are mapped onto physical frequencies according to Table 6.1-2 |  |  |  |  | Low and High range |  |  |
| Test CC Combination setting (aggregated BW of the CA configuration) as specified in Table 5.5A.1-1 for the CA Configuration across bandwidth combination sets supported by the UE |  |  |  |  | Mid |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |  |  |
| CA Configuration / Aggregated BW |  |  |  | Downlink Configuration |  | Uplink Configuration |  |
| Test ID | CC | BAND | CBW (MHz) | RB allocation |  | Modulation | RB allocation (NOTE 1) |
| 1 | PCC | nX | 100 | N/A for this test |  | DFT-s-OFDM QPSK | Inner_16RB_Left |
|  | SCC1 | nX | 100 |  |  | DFT-s-OFDM QPSK | Inner_16RB_Left |
|  | SCC2 | nX | 100 |  |  | DFT-s-OFDM QPSK | Inner_16RB_Left |
|  | SCC3 | nX | 100 |  |  | DFT-s-OFDM QPSK | Inner_16RB_Left |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. NOTE 2: CA Configuration Test cumulative aggregated BW settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1-1. NOTE 3: If the UE supports multiple CC Combinations in the CA Configuration with the same cumulative aggregated BW, only the combination with the lowest PCC ChBW is tested. NOTE 4: When the signalled DC carrier position is at Inner_16RB_Left, use Inner_16RB_Right for UL RB allocation. |  |  |  |  |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1 and C.3.0 and TS 38.508-1 [10] subclause 5.2.1.1.1, and uplink signals according to Annex G.0, G.1 and G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.4A.2.2.3.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.4A.2.2.3.4.3

6.4A.2.2.3.4.2	Test procedure

1.	Configure PCC and SCCs according to Annex C.0, C.1, C.2 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCCs as per TS 38.508-1 [10] clause 5.5.1 Procedure to configure SCC(s) for NR RF CA testing. Message contents are defined in clause 6.4A.2.2.3.4.3.

3.	SS activates SCCs by sending the activation MAC CE (Refer TS 38.321, clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[25], clause 9.3).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.4A.2.2.3.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE EIRP is in the range Preq + PW ± PW, where Preq is the power level specified in Tables 6.4A.2.2.3.4.2-1 according to the power class. PW is the power window according to Table [TBD] for the carrier frequency f and the channel bandwidth BW.

6.	Set the UE in the Inband Tx beam peak direction found with a 3D EIRP scan as performed in Annex K.1.1. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

7.	SS activates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.2 using condition TxRx.

8.	Measure carrier leakage on PCC using Global In-Channel Tx-Test (Annex E) for the - and -polarization. For TDD, only slots consisting of only UL symbols are under test.

9.	SS deactivates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.3.

10.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE EIRP is in the range Preq + PW ± PW, where Preq is the power level specified in Tables 6.4A.2.2.3.4.2-1 according to the power class. PW is the power window according to Table [TBD] for the carrier frequency f and the channel bandwidth BW.

11.	SS activates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.2 using condition TxRx.

12.	Measure carrier leakage on PCC using Global In-Channel Tx-Test (Annex E) for the - and -polarization. For TDD, only slots consisting of only UL symbols are under test.

13.	SS deactivates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.3.

NOTE 1:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.1.1.

Table 6.4A.2.2.3.4.2-1: UE EIRP Preq (dBm) for carrier leakage

| Power Class | Preq (dBm) for step 5 | Preq (dBm) for step 10 |
| --- | --- | --- |
| Power Class 1 | 17 | 4 |
| Power Class 2 | 6 | -13 |
| Power Class 3 | 0 | -13 |
| Power Class 4 | 11 | -13 |

Table 6.4A.2.2.3.4.2-2: Power Window (dB) for carrier leakage

TBD

6.4A.2.2.3.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6.

6.4A.2.2.3.5	Test requirement

For each of the n carrier leakage results derived in Annex E.3.1 for θ- and φ-polarization the total value is calculated according to

, where

.

Each of the n total carrier leakage results CarrLeakTotal shall not exceed the values in table 6.4.2.2.5-1 for power class 1, table 6.4.2.2.5-2 for power class 2, table 6.4.2.2.5-3 for power class 3 and table 6.4.2.2.5-4 for power class 4. Allocated RBs are not under test.

## << Unchanged sections omitted >>

F.1.2	Measurement of transmitter

Editor’s note: The measurement uncertainties for 6.5.2.1 Spectrum Emission Mask and 6.5.2.3 Adjacent Channel Leakage Ratio are based on a preliminary MU assessment and require an approval of the uncertainty contributors to be included in the uncertainty assessment as well as the contributors need further technical analysis.

Table F.1.2-1: Maximum Test System Uncertainty (MTSU) for transmitter tests

| Sub clause | Maximum Test System Uncertainty | Derivation of MTSU |
| --- | --- | --- |
| 6.2.1.1 UE maximum output power (EIRP) | PC3 Minimum peak EIRP, Max EIRP DUT ≤ 15 cm ±4.89 dB (FR2a) ±5.09 dB (FR2b) DUT ≤ 30 cm ±4.79 dB (FR2a) ±4.99 dB (FR2b) | MTSU = 1.00 x MU (from Table B.3-2-2, B.3-2-3 in TR 38.903) |
| 6.2.1.1 UE maximum output power (TRP) | PC3 Max TRP DUT ≤ 15 cm ±4.42 dB (FR2a) ±4.62 dB (FR2b) DUT ≤ 30 cm ±4.32 dB (FR2a) ±4.52 dB (FR2b) | MTSU = 1.00 x MU (from Table B.3-2-2, B.3-2-3 in TR 38.903) |
| 6.2.1.2 UE maximum output power (Spherical coverage) | TBD |  |
| 6.2.2 UE maximum output power for modulation / channel bandwidth | TBD |  |
| 6.2.3 UE maximum output power with additional requirements | TBD |  |
| 6.2.4 Configured transmitted power | TBD |  |
| 6.2A.1.1.1 UE maximum output power - EIRP and TRP for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.1.1.2 UE maximum output power - EIRP and TRP for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.1.1.3 UE maximum output power - EIRP and TRP for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3.1 Minimum output power | TBD |  |
| 6.3.2 Transmit OFF power | TBD |  |
| 6.3.3.2 General ON/OFF time mask | TBD |  |
| 6.3.3.4 PRACH time mask | TBD |  |
| 6.3.3.6 SRS time mask | TBD |  |
| 6.3.4.2 Absolute power tolerance | TBD |  |
| 6.3.4.3 Relative power tolerance | TBD |  |
| 6.3.4.4 Aggregate power tolerance | TBD |  |
| 6.3A.2.1 Transmit OFF power for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.2.2 Transmit OFF power for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.2.3 Transmit OFF power for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.4.1 Frequency error | ± 0.01 ppm | MTSU = 1.00 x MU (from B.10.1 and B.10.2 in TR 38.903) |
| 6.4.2.1 Error vector magnitude | TBD |  |
| 6.4.2.2 Carrier leakage | TBD |  |
| 6.4.2.3 In-band emissions | TBD |  |
| 6.4.2.4 EVM equalizer spectrum flatness | TBD |  |
| 6.4.2.5 EVM equalizer spectrum flatness for BPSK modulation | TBD |  |
| 6.4A.1.1 Frequency error for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.4.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.4A.1.2 Frequency error for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.4.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.4A.2.2.3 Carrier leakage for CA (4UL CA) | TBD |  |
| 6.4A.1.3 Frequency error for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.4.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5.1 Occupied bandwidth | TBD |  |

## << Unchanged sections omitted >>

F.3.2	Measurement of transmitter

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- The TT values defined in this section are valid only until RAN5#85 (November 2019).

- RAN5 expects to have the following actions performed by RAN5#85 in efforts to improve the TT values

- For updated TT evaluation only, MU analysis taking into the account the data sheet from commercially available test systems or any additional experimental data to justify the values

- UE vendors’ have supplied further input on TT based on the performance of the existing mmWave UE against the RF test cases.

- At least one statistical analysis has been done on possibilities to improve test confidence level.

- Measurement results & evaluation of network performance / commercially available devices has been provided.

Table F.3.2-1: Derivation of Test Requirements (Transmitter tests)

| Sub clause | Test Tolerance (TT) | Formula for test requirement |
| --- | --- | --- |
| 6.2.1.1 UE maximum output power (EIRP) | PC3 Minimum peak EIRP IFF (DUT ≤ 15 cm) 3.18 dB (FR2a) 3.31 dB (FR2b) IFF (DUT ≤ 30 cm) 3.11 dB (FR2a) 3.24 dB (FR2b) Max EIRP 0 dB | Minimum peak EIRP TT = 0.65 x MTSUIFF |
| 6.2.1.1 UE maximum output power (TRP) | PC3 Max TRP IFF (DUT ≤ 15 cm) 2.87 dB (FR2a) 3.00 dB (FR2b) IFF (DUT ≤ 30 cm) 2.81 dB (FR2a) 2.94 dB (FR2b) | Max TRP TT = 0.65 x MTSUIFF |
| 6.2.1.2 UE maximum output power (Spherical coverage) | PC1 TBD PC2 TBD PC3 TBD PC4 TBD |  |
| 6.2.2 UE maximum output power for modulation / channel bandwidth | TBD |  |
| 6.2.3 UE maximum output power with additional requirements | TBD |  |
| 6.2.4 Configured transmitted power | TBD |  |
| 6.2A.1.1.1 UE maximum output power - EIRP and TRP for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.1.1.2 UE maximum output power - EIRP and TRP for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.1.1.3 UE maximum output power - EIRP and TRP for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3.1 Minimum output power | TBD |  |
| 6.3.2 Transmit OFF power | 0 dB |  |
| 6.3.3.2 General ON/OFF time mask | TBD |  |
| 6.3.3.4 PRACH time mask | TBD |  |
| 6.3.4.2 Absolute power tolerance | TBD |  |
| 6.3.4.3 Relative power tolerance | TBD |  |
| 6.3.4.4 Aggregate power tolerance | TBD |  |
| 6.3A.2.1 Transmit OFF power for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.2.2 Transmit OFF power for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.2.3 Transmit OFF power for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.4.1 Frequency error | 0.005 ppm | TT = 0.5 x MTSU |
| 6.4.2.1 Error vector magnitude | 0%, up to 64QAM | Minimum requirement + TT |
| 6.4.2.2 Carrier leakage | TBD |  |
| 6.4.2.3 In-band emissions | TBD |  |
| 6.4.2.4 EVM equalizer spectrum flatness | TBD |  |
| 6.4.2.5 EVM equalizer spectrum flatness for BPSK modulation | TBD |  |
| 6.4A.1.1 Frequency error for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.4.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.4A.1.2 Frequency error for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.4.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.4A.2.2.3 Carrier leakage for CA (4UL CA) | TBD |  |
| 6.4A.1.3 Frequency error for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.4.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5.1 Occupied bandwidth | 0 kHz | Minimum requirement + TT |

## << End of changes >>