3GPP TSG-RAN5 Meeting #84	R5-196514

Ljubljana, Slovenia, 26th Aug 2019 - 30th Aug 2019

Title:	Proposing new IMS test cases

Source:	NTT DOCOMO, INC.

Agenda item:	6.5.6.5

Document for:	Endorsement

- Introduction

The purpose of this document is to discuss about adding the following 6 TCs for IMS.

TC1: MO MTSI speech call / SRVCC on MT side / Codec Change from AMR-WB to AMR-NB

TC2: MO MTSI speech call / SRVCC on MT side / Codec Change from EVS to AMR-NB

TC3: MO MTSI speech call / MO UE cancels call establishment

TC4: Mobile Originating Concatenated SMS

TC5: Mobile Terminating Concatenated SMS

TC6: MT MTSI speech call / EVS / AMR-WB

- Discussion

- TC1: MO MTSI speech call / SRVCC on MT side / Codec Change from AMR-WB to AMR-NB

- TC2: MO MTSI speech call / SRVCC on MT side / Codec Change from EVS to AMR-NB

In the real NW, when normal SRVCC happens on one side, the voice codec is changed from EVS/AMR-WB to AMR-NB on the other side. Although some test cases in TS 36.523-1, e,g, TC 13.4.3.1, checks the function of normal SRVCC, there is no test case to confirm such codec change behaviour on the other side.

Proposal 1: RAN5 needs to add TC1 and TC2 test cases in TS 34.229-1. TC1 and TC2 are applied to IMS capable UEs from Rel.8 and Rel.12 respectively.

- TC3: MO MTSI speech call / MO UE cancels call establishment

When MO UE cancels the establishment of voice call, it sends SIP_CANCEL to MT UE. This is a basic behaviour, but the current RAN5 test cases doesn’t cover.

Proposal 2: RAN5 needs to add TC3 in TS 34.229-1. TC3 is applied to IMS capable UEs from Rel.8.

- TC4: Mobile Originating Concatenated SMS

- TC5: Mobile Terminating Concatenated SMS

Concatenated SMS specified in TS 23.040 [1] is used when a user sends a long SMS. Although the TC 18.1 and 18.2 in TS 34.229-1 checks SMS over IMS function, there is no test case for Concatenated SMS.

Proposal 3: RAN5 needs to add TC4 and TC5 in TS 34.229-1. TC4 and TC5 are applied to IMS capable UEs from Rel.8.

- TC6: MT MTSI speech call / EVS / AMR-WB

If MO UE supporting EVS initiates a voice call to MT UE not supporting EVS, the voice codec AMR-WB will be negotiated. As you can see in TC 12.25 “MO MTSI speech call / EVS / AMR-WB” in TS 34.229-1, such behaviour on MO side is checked. However, there is no such test case for MT side.

Proposal 4: RAN5 needs to add TC6 in TS 34.229-1. TC6 is applied to IMS capable UEs from Rel.8.

- Proposal

Proposal 1: RAN5 needs to add TC1 and TC2 test cases in TS 34.229-1. TC1 and TC2 are applied to IMS capable UEs from Rel.8 and Rel.12 respectively.

Proposal 2: RAN5 needs to add TC3 in TS 34.229-1. TC3 is applied to IMS capable UEs from Rel.8.

Proposal 3: RAN5 needs to add TC4 and TC5 in TS 34.229-1. TC4 and TC5 are applied to IMS capable UEs from Rel.8.

Proposal 4: RAN5 needs to add TC6 in TS 34.229-1. TC6 is applied to IMS capable UEs from Rel.8.

In RAN5#84, these test cases are proposed in the following CRs.

TC1: R5-196493

TC2: R5-196508

TC3: R5-196509

TC4: R5-196511

TC5: R5-196512

TC6: R5-196510

Test applicability: R5-196516

- References

[1]	3GPP TS 23.040: " Technical realization of the Short Message Service (SMS)".