3GPP TSG-RAN WG5 #85		R5-198262
Reno, NV, USA, 18 – 22 November 2019

Agenda item:	5.3.5.17

Source:	Keysight Technologies

Title:	On Larger Quiet Zone Sizes

Document for:	Discussion and Endorsement

# Introduction

This contribution reviews commercially available tablet and laptop sizes and determines appropriate cylindrical and spherical quiet zone sizes.

# Discussion

Parts of this contribution were initially presented and discussed in RAN4 [1] and are now presented in RAN5 following the advice from the RAN4 chair to present quiet zone size related contributions in RAN5 instead of RAN4.

Various quiet zone/test zone sizes have been introduced for OTA test plans and tabulated in Table 1.

Table 1: Overview of currently defined OTA quiet zones and test zone sizes.

| Quiet Zone/ Test Zone Size | Additional QZ Information | Applicability | Reference |
| --- | --- | --- | --- |
| 15cm | Spherical QZ with diameter of 15cm | UE (RF, DEMOD, RRM) NR FR2 testing for smartphone UEs | [2][3] |
| 20cm | Spherical QZ with diameter of 20cm | NR FR1 and FR2 MIMO OTA for handset devices | [4] |
| 30cm | Cylindrical QZ with diameter and height of 30cm | 2G/3G/4G SISO and LTE MIMO OTA testing | [5][6] |
| 30cm | Spherical QZ with diameter of 30cm | UE (RF, DEMOD, RRM) NR FR2 testing | [2][3] |
| 50cm | Cylindrical QZ with diameter of 50cm and height of 36cm | 2G/3G/4G SISO OTA testing of notebooks and tablets | [5] |

While the quiet zones in the CTIA test plans have been defined as cylindrical quiet zones, the quiet zones defined for NF FR2 testing in 3GPP and CTIA have all been defined as spherical quiet zones.

Observation 1: While the quiet zones in the CTIA test plans have been defined as cylindrical quiet zones, the quiet zones defined for NF FR2 testing in 3GPP and CTIA have all been defined as spherical quiet zones.

This contribution is aimed at defining quiet zones for larger devices than smartphone UEs, e.g., tablets and laptops. The CTIA test plan has defined two cylindrical quiet zones for such wireless enabled devices. The dimensions of the two quiet zones are illustrated in Figure 1 and tabulated in Table 1.

Figure 1: Illustration of the 30 and 50cm quiet zones (Figure 3-8 from [1])

Table 2: Cylindrical Quiet Zone Dimensions of the 30cm and 50cm Quiet Zones

| Quiet Zone Dimensions | 30cm | 50cm |
| --- | --- | --- |
| Radius of Cylinder | 15cm | 25cm |
| Diameter of Cylinder | 30cm | 50cm |
| Height of Cylinder | 30cm | 36cm |

While smartphones can easily fit within the 30cm quiet zone [7], this contribution determines how large spherical or cylindrical quiet zones have to be in order to fit common tablets and laptops.

A brief survey of current commercially available tablets and laptops with WWAN (LTE) connectivity was conducted to study the required quiet zone/test zone sizes for these two types of devices.

The review of the tablet dimensions is summarized in Table 3 for three tablet types: small (screen sizes of up to 8”), regular (screen sizes of up to ~11”), and large (screen sizes of 12” or greater). The minimum cylindrical quiet zone diameter in the last column is the maximum diameter of a circle necessary to enclose the entire device in terms of height and width; the min. spherical quiet zone in the second last column on the other hand is maximum diameter of a sphere necessary to enclose the entire device.

Table 3: Survey of commercially available tablets with WWAN (LTE) connectivity in terms of dimensions

| Type | Models with WWAN | Typical Dimensions [cm] |  |  | Min. Quiet Zone Sphere Diameter [cm] | Min. Cylindrical Quiet Zone Diameter [cm] |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Height | Width | Depth |  |  |
| Mini | Apple Mini | 20.3 | 13.5 | 0.6 | 24.4 | 24.4 |
|  | Kindle Fire HD 8 | 21.4 | 12.8 | 1.0 | 25.0 | 24.9 |
| Regular | Apple iPad | 25.1 | 17.4 | 0.8 | 30.5 | 30.5 |
|  | Apple iPad Air | 25.1 | 17.4 | 0.6 | 30.5 | 30.5 |
|  | Apple iPad Pro (11") | 24.8 | 17.9 | 0.6 | 30.5 | 30.5 |
|  | Samsung Galaxy Tab S6 | 24.5 | 16.0 | 0.6 | 29.2 | 29.2 |
|  | Galaxy Tab A | 26.0 | 16.1 | 0.8 | 30.6 | 30.6 |
| Large | Apple iPad Pro (12.9") | 28.1 | 21.5 | 0.6 | 35.3 | 35.3 |
|  | Microsoft Surface Pro 6 | 29.2 | 20.1 | 0.9 | 35.5 | 35.4 |
|  | Dell Latitude 7200 | 29.2 | 20.9 | 0.9 | 35.9 | 35.9 |
|  | Galaxy Book2 12" | 28.8 | 20.0 | 0.7 | 35.1 | 35.0 |

Due to their very limited height, the min size of the cylindrical diameter closely matches the diameter of the sphere needed to fully enclose the tablets. It should be noted though that a 30cm diameter sphere or cylinder is not sufficient to fit all tablet types; rather a 40cm diameter is necessary to test tablets.

Observation 2: A cylindrical or spherical quiet zone size of 30cm in diameter is not sufficient for the majority of tablet devices with WWAN connectivity

The review of commercially available laptops with WWAN connectivity is summarized in Table 4 for three laptop types: mobile (screen sizes of up to 14”), medium workstation (screen sizes of up to ~15.6”), and large workstation (screen sizes of 17” or greater). The last two columns outline the minimum required cylindrical quiet zone dimensions in terms of diameter and height with the laptop in the open position test condition defined in Appendix L.9.1 of [5], and illustrated in Figure 2. On the other hand, the third last column outlines the minimum required spherical quiet zone diameter to fully enclose the laptop in the open position.

Figure 2: Notebook positioning guideline per CTIA [5]

Table 4: Survey of commercially available laptops with WWAN (LTE) connectivity in terms of dimensions

| Type | Models (with WWAN) | Dimensions (closed) [cm] |  |  | Min. Spherical Quiet Zone [cm] | Cylindrical Quiet Zone Dimensions (when open) [cm] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Width | Depth | Height |  | Diameter | Height |
| Mobile | Dell Latitude 7300 | 30.7 | 20.7 | 1.8 | 46.5 | 41.4 | 21.3 |
|  | Dell Latitude 7400 | 32.0 | 20.0 | 1.5 | 46.4 | 41.7 | 20.3 |
|  | HP EliteBook x360 1030 G4 | 30.6 | 20.4 | 1.6 | 46.0 | 41.1 | 20.8 |
|  | HP EliteBook x360 1040 G6 | 32.1 | 21.5 | 1.7 | 48.4 | 43.2 | 21.9 |
|  | Lenovo X1 Carbon Gen 7 | 32.3 | 21.7 | 1.5 | 48.7 | 43.5 | 21.9 |
| Medium Workstation | HP ZBook Studio G5 | 36.0 | 25.4 | 1.8 | 55.8 | 49.6 | 25.6 |
|  | HP ZBook 15 | 37.6 | 26.4 | 2.5 | 58.5 | 51.7 | 27.4 |
|  | Lenovo P53 | 37.8 | 25.2 | 2.5 | 57.1 | 50.7 | 26.2 |
| Large Workstation | HP Zbook 17 | 41.6 | 28.8 | 3.3 | 64.4 | 56.8 | 30.4 |
|  | Dell Precision 7740 Mobile Workstation | 41.4 | 25.3 | 2.7 | 59.8 | 53.6 | 26.5 |

The results show that only smaller laptops fit into a 50cm cylinder or sphere and that the medium to large laptops do not fit any more.

Observation 3: A cylindrical or spherical quiet zone size of 50cm in diameter is not sufficient for the medium to large laptop devices with WWAN connectivity.

Instead, for larger laptops a cylindrical quiet zone of ~60cm or spherical quiet zone of ~65cm in diameter is necessary.

Observation 4: For larger laptops a cylindrical quiet zone of ~60cm or spherical quiet zone of ~65cm in diameter is necessary.

Feedback from industry is requested on quiet zone sizes beyond 30cm, e.g., 40cm and 65cm, as well as QZ shapes, e.g., spherical or cylindrical, to augment the existing largest spherical quiet zone of 30cm in diameter.

Proposal 1: Industry to provide feedback/recommendations on quiet zone sizes and shapes to augment the existing largest spherical quiet zone of 30cm in diameter.

# Conclusion

The following observations and proposals were made in this contribution

Observation 1: While the quiet zones in the CTIA test plans have been defined as cylindrical quiet zones, the quiet zones defined for NF FR2 testing in 3GPP and CTIA have all been defined as spherical quiet zones.

Observation 2: A cylindrical or spherical quiet zone size of 30cm in diameter is not sufficient for the majority of tablet devices with WWAN connectivity

Observation 3: A cylindrical or spherical quiet zone size of 50cm in diameter is not sufficient for the medium to large laptop devices with WWAN connectivity.

Observation 4: For larger laptops a cylindrical quiet zone of ~60cm or spherical quiet zone of ~65cm in diameter is necessary.

Proposal 1: Industry to provide feedback/recommendations on quiet zone sizes and shapes to augment the existing largest spherical quiet zone of 30cm in diameter.

# References

- R4-1912097, On Quiet Zone Sizes for PC3 Devices, Keysight Technologies, 3GPP TSG-RAN WG4 #92bis, October 2019

- TR38.810, Study on test methods, V16.3.0 (2019-06)

- TR 38.903, Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases, V16.0.0 (2019-06)

- TR 38.827, Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE), V1.0.0 (2019-05)

- Test Plan for Wireless Device Over-the-Air Performance, CTIA, Version 3.8.2, April 2019

- Test Plan for 2x2 Downlink MIMO and Transmit Diversity Over-the-Air Performance, CTIA, Version 1.2, April 2018

- R5-198261, On 15cm Quiet Zone, Keysight Technologies, 3GPP TSG-RAN WG5 #85bis, November 2019