Source:		Rohde & Schwarz

Title:	Discussion on test point selection for In-band Emissions in FR2

# Introduction

At RAN#78, it was decided that for each NR test case a discussion of test point selection regarding frequency, channel BW, SCS, RB allocation and waveform is required and that the analysis is documented in TR 38.905.

In this contribution the selection of frequency, channel BW, SCS, RB allocation and waveform for the In-band Emissions test case is analyzed.

# Discussion

The In-band Emissions are unwanted signals present in non-allocated RBs and are defined in section 6.4.2.3 and Annex F Transmit Modulation of TS 38.101-2 [1].

The minimum requirement is specified as follows:

6.4.2.3	In-band emissions

##### 6.4.2.3.1	General

The in-band emission is defined as the average across 12 sub-carriers and as a function of the RB offset from the edge of the allocated UL transmission bandwidth. The in-band emission is measured as the ratio of the UE output power in a non–allocated RB to the UE output power in an allocated RB.

The basic in-band emissions measurement interval is identical to that of the EVM test.

The requirement is verified with the test metric of In-band emission (Link=TX beam peak direction, Meas=Link angle).

##### 6.4.2.3.2	In-band emissions for power class 1

The relative in-band emission shall not exceed the values specified in Table 6.4.2.3.2-1 for power class 1 UEs.

Table 6.4.2.3.2-1: Requirements for in-band emissions for power class 1

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | max-25 -10.log10NRBLCRB, 20.log10EVM- 5.¦∆RB-1LCRB, -55.1dBm-PRB |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -25 | Output power > 27 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -20 | Output power ≤ 27 dBm |  |
| Carrier leakage | dBc | -25 | Output power > 17 dBm | Carrier frequency (NOTES 4, 5) |
|  |  | -20 | 4 dBm ≤ Output power ≤ 17 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of (PRB - 25 dB) and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in NOTE 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. For pi/2 BPSK with Spectrum Shaping, the limit is expressed as a ratio of measured power in one non-allocated RB to the measured power in the allocated RB with highest PSD NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the carrier frequency, but excluding any allocated RBs. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit are those that are enclosed in the RBs containing the DC frequency if NRB is odd, or in the two RBs immediately adjacent to the DC frequency if NRB is even but excluding any allocated RB. NOTE 6: LCRB is the Transmission Bandwidth (see Figure 5.3.3-1). NOTE 7: NRB is the Transmission Bandwidth Configuration (see Figure 5.3.3-1). NOTE 8: EVM s the limit for the modulation format used in the allocated RBs. NOTE 9: RB is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. RB= 1 or RB= -1 for the first adjacent RB outside of the allocated bandwidth). NOTE 10: PRB is the transmitted power per allocated RB, measured in dBm. NOTE 11: All powers are EIRP in beam peak direction. |  |  |  |  |

##### 6.4.2.3.3	In-band emissions for power class 2

##### 6.4.2.3.4	In-band emissions for power class 3

The relative in-band emission shall not exceed the values specified in Table 6.4.2.3.4-1 for power class 3 UEs.

Table 6.4.2.3.4-1: Requirements for in-band emissions for power class 3

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | max-25 -10.log10NRBLCRB, 20.log10EVM- 5.¦∆RB-1LCRB, -55.1dBm-PRB |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -25 | Output power > 10 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -20 | Output power ≤ 10 dBm |  |
| Carrier leakage | dBc | -25 | Output power > 0 dBm | Carrier frequency (NOTES 4, 5) |
|  |  | -20 | -13 dBm ≤ Output power ≤ 0 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of (PRB - 25 dB) and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in NOTE 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. For pi/2 BPSK with Spectrum Shaping, the limit is expressed as a ratio of measured power in one non-allocated RB to the measured power in the allocated RB with highest PSD NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the carrier frequency, but excluding any allocated RBs. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit are those that are enclosed in the RBs containing the DC frequency if NRB is odd, or in the two RBs immediately adjacent to the DC frequency if NRB is even but excluding any allocated RB. NOTE 6: LCRB is the Transmission Bandwidth (see Figure 5.3.3-1). NOTE 7: NRB is the Transmission Bandwidth Configuration (see Figure 5.3.3-1). NOTE 8: EVM s the limit for the modulation format used in the allocated RBs. NOTE 9: RB is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. RB= 1 or RB= -1 for the first adjacent RB outside of the allocated bandwidth). NOTE 10: PRB is the transmitted power per allocated RB, measured in dBm. NOTE 11: All powers are EIRP in beam peak direction. |  |  |  |  |

##### 6.4.2.3.5	In-band emissions for power class 4

## Test Environment

In LTE, In-band Emissions are tested under extreme conditions. However, since In-band Emissions are not expected to vary significantly with temperature or supply voltage, it is proposed to apply normal condition as test environment in order to save test time.

Proposal 1: Define test environment as Normal for In-band Emissions in FR2.

## Test Subcarrier Spacing

It is not expected that in-band emissions depend on the choice of subcarrier spacing. Smallest SCS is selected since it is the most common one.

Proposal 2: Select smallest SCS for In-band Emissions in FR2.

## Test Frequencies

In LTE, In-band Emissions are tested for Low range, Mid range, and High range. Due to the large width of NR bands in FR2 it is reasonable to follow the LTE approach for NR.

Proposal 3: Test In-band Emissions for Low range, Mid range, and High range frequencies in FR2.

## Test Channel Bandwidth

In LTE, In-band Emissions are tested for Lowest, 5 MHz and Highest channel bandwidths. As in-band emissions of an UE depend on the unknown details of UE implementation it is reasonable to follow the LTE approach for NR.

Proposal 4: Test In-band Emissions for Lowest, Mid and Highest channel bandwidth in FR2.

## Test Modulations and waveforms for PUSCH

In LTE In-band Emissions are tested for test points without MPR. However for all modulations an MPR is defined in FR2. DFT-s-OFDM PI/2 BPSK should be selected as modulation and waveform as it has the lowest MPR.

Moreover, CP OFDM has a higher CREST factor than DFT-s-OFDM and may result in higher in-band emissions although a higher MPR is applied. Therefore, both modulations should be tested.

Proposal 5: Test In-band Emissions with DFT-s-OFDM PI/2 BPSK and CP-OFDM QPSK in FR2.

## RB allocation for PUSCH

In LTE, In-band Emissions are tested with highest allocation without MPR where the partial allocation is located on the lower and upper channel edge. For power class 1 the high MPR of 10 dB for an allocated bandwidth less or equal to 10.8 MHz shall be avoided which requires an allocation of at least 16 RBs for DFT-s-OFDM. When this is fulfilled the MPRWT applies which is smaller for Inner RB allocations than for outer RB allocations. For a CHBW of 50MHz an Inner_16RB_Left_Region1 allocation does not fulfill the test purpose when assuming that the carrier is at the center of the channel as the RBs at the carrier and part of the image RBs would be allocated. Therefore, with an Inner-Region1 allocation MPRnarrow cannot be avoided. As a remedy, an Inner-Region2 allocation is applied since this results in an MPR of only 3dB for 50 MHz channel bandwidth instead of 10dB for MPRnarrow.

For PC3, to avoid MPR is more crucial than for PC1 in order to minimize the measurement uncertainty due to impact of noise. For PC3, there is no MPRnarrow for an Inner allocation. Therefore, an allocation of Inner_4RB is selected to fulfil the test purpose and optimize the MU.

Proposal 6: For PC1, test Carrier Leakage in FR2 with Inner_16RB_Left_Region2 and Inner_16RB_Right_Region2. 
For PC2, PC3, and PC4, test Carrier Leakage in FR2 with Inner_4RB_Left and Inner_4RB_Right.

## Test Modulations, waveforms and formats for In-band Emissions PUCCH

Depending on the PUCCH format QPSK or PI/2- BPSK are applied as modulation. In LTE, for TDD format 1a / 1b are tested. The same approach should be followed in 5G.

Due to the differences in the modulation scheme (with and without precoding) and due to the different CREST factors for DFT-s-OFDM and CP-OFDM both waveforms should be tested.

Proposal 7: Test In-band Emissions PUCCH for DFT-s-OFDM and CP-OFDM with Format 1a / 1b.

## Downlink configuration for In-band Emissions PUCCH

In LTE, QPSK is applied as modulation for the downlink and RB allocation is partial. However, the In-band Emissions of PUCCH should not depend on the downlink allocation and therefore a specification of the downlink allocation is not required.

Proposal 8: Do not specify a downlink allocation for In-band Emissions PUCCH.

# Conclusion

To sum up, the following selection of test points is proposed for In-band Emissions in FR2:

Proposal 1: Define test environment as Normal for In-band Emissions in FR2.

Proposal 2: Select smallest SCS for In-band Emissions in FR2.

Proposal 3: Test In-band Emissions for Low range, Mid range, and High range frequencies in FR2.

Proposal 4: Test In-band Emissions for Lowest, Mid and Highest channel bandwidth in FR2.

Proposal 5: Test In-band Emissions with DFT-s-OFDM PI/2 BSPK and CP-OFDM QPSK in FR2.

Proposal 6: For PC1, test Carrier Leakage in FR2 with Inner_16RB_Left_Region2. In case the signaled carrier position is at Inner_16RB_Left_Region2, use Inner_16RB_Right_Region2. 
For PC2, PC3, and PC4, test Carrier Leakage in FR2 with Inner_4RB_Left. In case the signaled carrier position is at Inner_4RB_Left, use Inner_4RB_Right.

Proposal 7: Test In-band Emissions PUCCH for DFT-s-OFDM and CP-OFDM with Format 1a / 1b.

Proposal 8: Do not specify a downlink allocation for In-band Emissions PUCCH.

# Number of test points

In this document several proposals have been stated for the configuration parameters. In all of cases the analysis has been focused on reducing testing time without losing coverage in UE testing for NR FR2.

PUSCH:

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- |
| 1 | 3 | 3 | 1 | 4 | 36 |

PUCCH:

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (format and waveform) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- |
| 1 | 3 | 3 | 1 | 2 | 18 |

# References

- TS 38.101-2 V15.2.0 , 3GPP TSG RAN WG4, June 2018